3GPP TSG-SA5 Meeting #125 	S5-193431

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0720 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Rate-Control information and triggers to CDRs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT-CH, TEI15 |  |  |  |  |  | Date: |  |  | 2019-04-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | APN rate control and serving PLMN rate control information are not introduced for List of Traffic Volume container for IP-CAN session level charging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new fields and triggers to List of Traffic Volume container. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for this function is not complete in 3GPP and it is not possible to do proper implementation in case IP-CAN session level charging is used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.2.25, 5.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.2.2.25	List of Traffic Data Volumes

This list applicable in S-CDR, SGW-CDR, IPE-CDR, ePDG-CDR and TWAG-CDR, includes one or more containers.

This list applicable in PGW-CDR when charging per IP-CAN session is active and IP-CAN bearer charging is being performed for the session.

In SGW-CDR, PGW-CDR, IPE-CDR, ePDG-CDR and TWAG-CDR, containers are per QCI/ARP pair. This means that if QoS control within one IP-CAN bearer is applicable in S-GW, P-GW, ePDG and TWAG, there can be several containers open at same time one per each applied QCI/ARP pair.

Each container includes the following fields:

-	Data Volume Uplink, Data Volume Downlink, Change Condition and Change Time.

-	Data Volume Uplink includes the number of octets transmitted during the use of the packet data services in the uplink direction. In MBMS charging, this field is normally to be set to zero, because MBMS charging is based on the volume of the downlink data. The counting of uplink data volumes is optional. In S-CDR this field is not present when the SGSN has successfully established Direct Tunnel between the RNC and the GGSN.

-	Data Volume Downlink includes the number of octets transmitted during the use of the packet data services in the downlink direction. In S-CDR this field is not present when the SGSN has successfully established Direct Tunnel between the RNC and the GGSN.

-	Change Condition defines the reason for closing the container (see TS 32.251 [11]), such as tariff time change, QoS change or closing of the CDR. This field is derived from Change-Condition AVP Traffic-Data-Volumes AVP level defined in TS 32.299 [40] when received on Rf. Each value is mapped to the corresponding value in "ChangeCondition" field. When no Change-Condition AVP is provided, the "recordClosure" value is set for the container. For User Location Change, when one of the "CGI/SAI, ECGI or TAI or RAI Change" are reported as user location change, the dedicated value in service Condition Change is set instead of the generic "user location change" value.

-	Change Time is a time stamp, which defines the moment when the volume container is closed or the CDR is closed. All the active IP-CAN bearers do not need to have exactly the same time stamp e.g. due to same tariff time change (variance of the time stamps is implementation and traffic load dependent, and is out of the scope of standardisation).

-	User Location Information contains the location (e.g. CGI/SAI, ECGI/TAI or RAI) where the UE is located and used during the transfer of the data volume captured by the container (applicable only to the SGW-CDR). This is included in the Traffic data container only if previous container's change condition is "user location change". Note the user location information in SGW-CDR main level contains the location where the UE was when SGW-CDR was opened, and the "Last user location information" in SGW-CDR main level contains the location where the UE was when SGW-CDR is closed.

-	UWAN User Location Information contains the Untrusted Wireless Access Network (UWAN) user location during the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container's change condition is "user location change". Note the user location information in ePDG-CDR main level contains the location where the UE was when ePDG-CDR was opened.

-	TWAN User Location Information contains the user location in a Trusted WLAN Access Network during the the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container’s change condition is "user location change". Note the user location information in TWAG-CDR main level contains the location where the UE was when TWAG-CDR was opened.

-	Presence Reporting Area Status contains the status of the UE presence (i.e. indication on whether the UE is inside or outside) in the Presence Reporting Area(s) identified by "Presence Reporting Area identifier(s)" contained in SGW-CDR main level. This is included in the Traffic data container when the initial status is reported after opening of SGW-CDR, or if previous container’s change condition is "Presence in Presence Reporting Area Change". This field is not applicable in multiple PRA(s).

-	List of Presence Reporting Area Information contains the list of Presence Reporting Area Information. Each Presence Reporting Area Information includes the "Presence Reporting Area identifier", the status of the UE presence (i.e. indication on whether the UE is inside or outside the Presence Reporting Area or Presence Reporting Area(s) inactive). In the PGW it contains the Node (PCRF, OCS) which subscribed-to the PRA.

-	User CSG information contains the status of the user accessing a CSG cell (CSG ID within the PLMN, Access mode and indication on CSG membership) during the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container’s change condition is "user CSG information change". Note the "user CSG information" in S-CDR/SGW-CDR main level contains the "user CSG information" when S-CDR/SGW-CDR was opened.

-	EPC QoS Information In case of IP-CAN bearer specific container this contains authorized QoS for the IP-CAN bearer. First container for each QCI/ARP pair includes this field. In following containers this field is present if previous change condition is "QoS change". This field is applicable only in SGW-CDR, PGW-CDR, IPE-CDR, ePDG-CDR, and TWAG-CDR.

In S-CDR first container includes following optional fields: QoS Requested and QoS Negotiated. In following containers QoS Negotiated is present if previous change condition is "QoS change". In addition to the QoS Negotiated parameter the QoS Requested parameter is present in following containers if the change condition is "QoS change" and the QoS change was initiated by the MS via a IP-CAN bearer modification procedure.

Table 5.1.2.2.25.1 illustrates an examplefor S-CDR but same principles are applicable also for SGW-CDR, PGW-CDR, IPE-CDR, ePDG-CDR and TWAG-CDR. There are five containers (sets of volume counts) caused by one QoS change, one location change, one tariff time change and one Direct Tunnel establishment (direct tunnel change applicable in S-CDR only). When CDR is opened the subscriber is in CGI1.

Table 5.1.2.2.25.1: Example list of traffic data volumes

| QoS Requested = QoS1 QoS Negotiated = QoS1 Data Volume Uplink = 1 Data Volume Downlink = 2 Change Condition = QoS change Time Stamp = TIME1 | QoS Requested = QoS2 (if requested by the MS) QoS Negotiated = QoS2 Data Volume Uplink = 5 Data Volume Downlink = 6 Change Condition = Tariff change Time Stamp = TIME2 | Data Volume Uplink = 10 Data Volume Downlink = 3 Change Condition = CGI/SAI Change Time Stamp = TIME3 |
| --- | --- | --- |

| Data Volume Uplink = 3 Data Volume Downlink = 4 User Location Info = CGI2 Change Condition = Direct Tunnel establishment Occurrence Time Stamp = TIME4 | Change Condition = Record closed Time Stamp = TIME5 |
| --- | --- |

First container includes initial QoS values and corresponding volume counts. Second container includes new QoS values and corresponding volume counts before tariff time change. Third container includes the indication of location change and corresponding volume counts before the location change and after the tariff time change. Fourth container includes volume counts after the location change and contains the indication of Direct Tunnel establishment. Last container includes no volume count as it refers to Direct Tunnel establishment. The total volume counts can be itemised as shown in Table 5.1.2.2.25.2 (tariff1 is used before and tariff2 after the tariff time change):

Table 5.1.2.2.25.2: Itemised list of total volume count corresponding to table 5.1.2.2.25.1

|  |  | Container |
| --- | --- | --- |
| QoS1+Tariff1 | uplink = 1, downlink = 2 | 1 |
| QoS2+Tariff1 | uplink = 5, downlink = 6 | 2 |
| QoS2+Tariff2 | uplink = 13, downlink = 7 | 3+4 |
| QoS1 | uplink = 1, downlink = 2 | 1 |
| QoS2 | uplink = 18, downlink = 13 | 2+3+4 |
| Tariff1 | uplink = 6, downlink = 8 | 1+2 |
| Tariff2 | uplink = 13, downlink = 7 | 3+4 |
| CGI1 | uplink = 16, downlink = 11 | 1+2+3 |
| CGI2 | uplink = 3, downlink = 4 | 4 |
| No Direct Tunnel | uplink = 19, downlink = 15 | 1+2+3+4 |
| Direct Tunnel | -, - | 5 |

The amount of data counted in the S-GW shall be the payload of the user plane at the S1-U/S4/S2interface. Therefore the data counted already includes the IP PDP bearer protocols i.e. IP or PPP.

The data volume counted in the SGSN is dependent on the system. For GSM SGSN the data volume is the payload of the SNDCP PDUs at the Gb interface. For UMTS-SGSN it is the GTP-U PDUs at the Iu-PS interface. Therefore, in both systems, the data counted already includes the overheads of any PDP bearer protocols.

In GSM, in order to avoid that downstream packets transmitted from the old SGSN to the new SGSN at inter SGSN RA update induce the increase of the PDP CDR downstream volume counters in both SGSN the following rules must be followed:

-	For PDP contexts using LLC in unacknowledged mode: an SGSN shall update the PDP CDR when the packet has been sent by the SGSN towards the MS;

For PDP contexts using LLC in acknowledged mode, a GSM-SGSN shall only update the PDP CDR at the reception of the acknowledgement by the MS of the correct reception of a downstream packet. In other worlds, for inter SGSN RA update, the new SGSN shall update the PDP CDR record when a downstream packet sent by the old SGSN is received by the MS and acknowledged by the MS towards the new SGSN through the RA update complete message.

In UMTS, the not transferred downlink data can be accounted for in the S-CDR with "RNC Unsent Downlink Volume" field, which is the data that the RNC has either discarded or forwarded during handover. Data volumes retransmitted (by RLC or LLC) due to poor radio link conditions shall not be counted.

The following additional fields are applicable in the P-GW CDR when charging per IP-CAN session is active and IP-CAN bearer charging is being performed for the session:

-	Charging Id In case of IP-CAN bearer specific container this contains the charging Id assigned to the bearer.

-	RAT Type This field contains the RAT type for the current IP-CAN bearer.

-	Access Availability Change Reason indicates the reason why the avalibility of an access is changed by the PCEF, i.e. RAN rule indication or Access usable/unusable as defined in TS 29.212 [220].

-	Related Change of Charging Condition indicates the reason a related container was closed when the current container is indirectly closed and the supplemental information for the event. This information is applicable when charging per IP-CAN session is active for a multi-access PDN connection.

-	Diagnostics In case of IP-CAN bearer specific container this contains the Diagnostics as per clause 5.1.2.2.11 associated to the bearer.

-	Enhanced Diagnostics In case of IP-CAN bearer specific container this contains the Enhanced Diagnostics as per clause 5.1.2.2.13Aa associated to the bearer.

-	CP CIoT EPS optimisation indicator This field contains the indication on whether Control Plane CIoT EPS optimisation is used for the transfer of the data volume captured by the container. This is included in the Traffic data container only if previous container's change condition is "change in user plane to UE". Note the CP CIoT EPS Optimisation indicator field in SGW-CDR main level contains the CP CIoT EPS optimisation indicator value when SGW-CDR was opened.

-	Serving PLMN Rate Control This field contains the Serving PLMN Rate Control applied by MME during the transfer of the data volume captured by the container (applicable only to the SGW-CDR). This is included in the Traffic data container only if previous container's change condition is " Serving PLMN Rate Control change ". Note the Serving PLMN Rate Control field in SGW-CDR main level contains the Serving PLMN Rate Control when SGW-CDR was opened.

-	APN Rate Control This field contains the APN Rate Control applied by PGW during the transfer of the data volume captured by the container (applicable only to the PGW-CDR). This is included in the Traffic data container only if previous container's change condition is "APN Rate Control change ". Note the APN Rate Control field in PGW-CDR main level contains the APN Rate Control when PGW-CDR was opened.

Second change

#### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS and EPC CDR types defined in TS 32.251 [11].

.$GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AddressString,

CallDuration,

CallingNumber,

CauseForRecClosing,

CellId,

ChargingID,

CivicAddressInformation,

Diagnostics,

DiameterIdentity,

DynamicAddressFlag,

EnhancedDiagnostics,

GSNAddress,

IPAddress,

LCSCause,

LCSClientIdentity,

LCSQoSInfo,

LevelOfCAMELService,

LocalSequenceNumber,

LocationAreaAndCell,

LocationAreaCode,

ManagementExtensions,

MBMSInformation,

MessageReference,

MSISDN,

MSTimeZone,

NodeID,

PDPAddress,

PLMN-Id,

PositioningData,

RATType,

RecordingEntity,

RecordType,

RoutingAreaCode,

SCSASAddress,

ServiceSpecificInfo,

SMSResult,

SmsTpDestinationNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

DefaultGPRS-Handling,

DefaultSMS-Handling,

NotificationToMSUser,

ServiceKey

FROM MAP-MS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version18 (18)}

-- from TS 29.002 [214]

IMEI,

IMSI,

ISDN-AddressString,

RAIdentity

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CommonDataTypes (18) version18 (18)}

-- from TS 29.002 [214]

CallReferenceNumber

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CH-DataTypes (13) version18 (18)}

-- from TS 29.002 [214]

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25) version18 (18) }

-- from TS 29.002 [214]

LocationMethod

FROM SS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version12 (12)}

-- from TS 24.080 [209]

;

--

--  GPRS RECORDS

--

GPRSRecord	::= CHOICE

--

-- Record values 20, 22..27 are specific

-- Record values 76, 77, 86 are MBMS specific

-- Record values 78,79 and 92, 95, 96 are EPC specific

--

{

sgsnPDPRecord			[20] SGSNPDPRecord,

sgsnMMRecord			[22] SGSNMMRecord,

sgsnSMORecord			[23] SGSNSMORecord,

sgsnSMTRecord			[24] SGSNSMTRecord,

sgsnMTLCSRecord			[25] SGSNMTLCSRecord,

sgsnMOLCSRecord			[26] SGSNMOLCSRecord,

sgsnNILCSRecord			[27] SGSNNILCSRecord,

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord,

sGWRecord				[78] SGWRecord,

pGWRecord				[79] PGWRecord,

gwMBMSRecord			[86] GWMBMSRecord,

tDFRecord				[92] TDFRecord,

iPERecord				[95] IPERecord,

ePDGRecord				[96] EPDGRecord,

tWAGRecord				[97] TWAGRecord

}

SGWRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

s-GWAddress					[4] GSNAddress,

chargingID					[5] ChargingID,

servingNodeAddress			[6] SEQUENCE OF GSNAddress,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servingNodePLMNIdentifier	[27] PLMN-Id OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

mSTimeZone 					[31] MSTimeZone OPTIONAL,

userLocationInformation		[32] OCTET STRING OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

servingNodeType				[35] SEQUENCE OF ServingNodeType,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 	[41] NULL OPTIONAL,

userCSGInformation			[42] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

lowPriorityIndicator		[44] NULL OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

s-GWiPv6Address				[48] GSNAddress OPTIONAL,

servingNodeiPv6Address		[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

userLocationInfoTime		[52] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[53] CNOperatorSelectionEntity OPTIONAL,

presenceReportingAreaInfo	[54] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation	[55] OCTET STRING OPTIONAL,

lastMSTimeZone				[56] MSTimeZone OPTIONAL,

enhancedDiagnostics			[57] EnhancedDiagnostics OPTIONAL,

cPCIoTEPSOptimisationIndicator [59] CPCIoTEPSOptimisationIndicator OPTIONAL,

uNIPDUCPOnlyFlag			[60] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl		[61] ServingPLMNRateControl OPTIONAL,

pDPPDNTypeExtension			[62] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter		[63] MOExceptionDataCounter OPTIONAL,

listOfRANSecondaryRATUsageReports [64] SEQUENCE OF RANSecondaryRATUsageReport OPTIONAL

}

PGWRecord 	::= SET

--

--	List of traffic volumes is only applicable when Charging per IP-CAN session is active and

--	IP-CAN bearer charging is being performed for the session.

--

--	EPC QoS Information is only applicable when Charging per IP-CAN session is active.

--

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

chargingID						[5] ChargingID,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext				[25] NULL OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

cAMELChargingInformation		[33] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 		[42] NULL OPTIONAL,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

lowPriorityIndicator				[46] NULL OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

userLocationInfoTime				[53] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		    	[54] CNOperatorSelectionEntity OPTIONAL,

ePCQoSInformation					[55] EPCQoSInformation OPTIONAL,

presenceReportingAreaInfo			[56] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation			[57] OCTET STRING OPTIONAL,

lastMSTimeZone						[58] MSTimeZone OPTIONAL,

enhancedDiagnostics					[59] EnhancedDiagnostics OPTIONAL,

nBIFOMMode							[60] NBIFOMMode OPTIONAL,

nBIFOMSupport						[61] NBIFOMSupport OPTIONAL,

uWANUserLocationInformation			[62] UWANUserLocationInfo OPTIONAL,

sGiPtPTunnellingMethod				[64] SGiPtPTunnellingMethod OPTIONAL,

uNIPDUCPOnlyFlag					[65] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl				[66] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[67] APNRateControl OPTIONAL,

pDPPDNTypeExtension					[68] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter				[69] MOExceptionDataCounter OPTIONAL,

chargingPerIPCANSessionIndicator    [70] ChargingPerIPCANSessionIndicator OPTIONAL,

threeGPPPSDataOffStatus				[71] ThreeGPPPSDataOffStatus OPTIONAL,

sCSASAddress						[72] SCSASAddress OPTIONAL,

listOfRANSecondaryRATUsageReports	[73] SEQUENCE OF RANSecondaryRATUsageReport OPTIONAL

}

TDFRecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

tDFAddress							[53] GSNAddress,

tDFiPv6AddressUsed					[54] GSNAddress OPTIONAL,

tDFPLMNIdentifier					[55] PLMN-Id OPTIONAL,

servedFixedSubsID					[56] FixedSubsID OPTIONAL,

accessLineIdentifier				[57] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation		[59] FixedUserLocationInformation OPTIONAL

}

IPERecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

iPEdgeAddress					[4] GSNAddress,

chargingID						[5] ChargingID,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

iPCANsessionType				[8] PDPType OPTIONAL,

servedIPCANsessionAddress		[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI		   				[29] IMEI OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servedMNNAI						[36] SubscriptionID OPTIONAL,

iPEdgeOperatorIdentifier		[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

servedIPCANsessionAddressExt 	[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt			[47] DynamicAddressFlag OPTIONAL,

iPEdgeiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission   				[52] NULL OPTIONAL,

servedFixedSubsID				[55] FixedSubsID OPTIONAL,

accessLineIdentifier			[56] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation	[57] FixedUserLocationInformation OPTIONAL

}

EPDGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

ePDGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

ePDGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

uWANUserLocationInformation	[53] UWANUserLocationInfo OPTIONAL,

userLocationInfoTime		[54] TimeStamp OPTIONAL,

iMSIunauthenticatedFlag 	[55] NULL OPTIONAL

}

TWAGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

tWAGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

tWAGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

tWANUserLocationInformation	[53] TWANUserLocationInfo OPTIONAL,

iMSIunauthenticatedFlag 	[54] NULL OPTIONAL

}

SGSNMMRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

sgsnAddress					[3] GSNAddress OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

routingArea					[5] RoutingAreaCode OPTIONAL,

locationAreaCode			[6] LocationAreaCode OPTIONAL,

cellIdentifier				[7] CellId OPTIONAL,

changeLocation				[8] SEQUENCE OF ChangeLocation OPTIONAL,

recordOpeningTime			[9] TimeStamp,

duration					[10] CallDuration OPTIONAL,

sgsnChange					[11] SGSNChange OPTIONAL,

causeForRecClosing			[12] CauseForRecClosing,

diagnostics					[13] Diagnostics OPTIONAL,

recordSequenceNumber		[14] INTEGER OPTIONAL,

nodeID						[15] NodeID OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

localSequenceNumber			[17] LocalSequenceNumber OPTIONAL,

servedMSISDN				[18] MSISDN OPTIONAL,

chargingCharacteristics		[19] ChargingCharacteristics,

cAMELInformationMM 			[20] CAMELInformationMM OPTIONAL,

rATType						[21] RATType OPTIONAL,

chChSelectionMode			[22] ChChSelectionMode OPTIONAL,

cellPLMNId					[23] PLMN-Id OPTIONAL,

servingNodePLMNIdentifier	[24] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[25] CNOperatorSelectionEntity OPTIONAL

}

SGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

sgsnAddress					[5] GSNAddress OPTIONAL,

msNetworkCapability			[6] MSNetworkCapability OPTIONAL,

routingArea					[7] RoutingAreaCode OPTIONAL,

locationAreaCode			[8] LocationAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

chargingID					[10] ChargingID,

ggsnAddressUsed				[11] GSNAddress,

accessPointNameNI			[12] AccessPointNameNI OPTIONAL,

pdpType						[13] PDPType OPTIONAL,

servedPDPAddress			[14] PDPAddress OPTIONAL,

listOfTrafficVolumes		[15] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[16] TimeStamp,

duration					[17] CallDuration,

sgsnChange					[18] SGSNChange OPTIONAL,

causeForRecClosing			[19] CauseForRecClosing,

diagnostics					[20] Diagnostics OPTIONAL,

recordSequenceNumber		[21] INTEGER OPTIONAL,

nodeID						[22] NodeID OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[25] APNSelectionMode OPTIONAL,

accessPointNameOI			[26] AccessPointNameOI OPTIONAL,

servedMSISDN				[27] MSISDN OPTIONAL,

chargingCharacteristics		[28] ChargingCharacteristics,

rATType						[29] RATType OPTIONAL,

cAMELInformationPDP  		[30] CAMELInformationPDP OPTIONAL,

rNCUnsentDownlinkVolume		[31] DataVolumeGPRS OPTIONAL,

chChSelectionMode			[32] ChChSelectionMode OPTIONAL,

dynamicAddressFlag			[33] DynamicAddressFlag OPTIONAL,

iMSIunauthenticatedFlag 	[34] NULL OPTIONAL,

userCSGInformation			[35] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[36] PDPAddress OPTIONAL,

lowPriorityIndicator		[37] NULL OPTIONAL,

servingNodePLMNIdentifier	[38] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[39] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMORecord	::= SET

--

--   also for MME UE originated SMS record

--

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

serviceCentre				[5] AddressString OPTIONAL,

recordingEntity				[6] RecordingEntity OPTIONAL,

locationArea				[7] LocationAreaCode OPTIONAL,

routingArea					[8] RoutingAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

messageReference			[10] MessageReference,

eventTimeStamp				[11] TimeStamp,

smsResult					[12] SMSResult OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

nodeID						[14] NodeID OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		[16] ChargingCharacteristics,

rATType						[17] RATType OPTIONAL,

destinationNumber			[18] SmsTpDestinationNumber OPTIONAL,

cAMELInformationSMS			[19] CAMELInformationSMS OPTIONAL,

chChSelectionMode			[20] ChChSelectionMode OPTIONAL,

servingNodeType				[21] ServingNodeType,

servingNodeAddress			[22] GSNAddress OPTIONAL,

servingNodeiPv6Address		[23] GSNAddress OPTIONAL,

mMEName						[24] DiameterIdentity OPTIONAL,

mMERealm					[25] DiameterIdentity OPTIONAL,

userLocationInformation		[26] OCTET STRING OPTIONAL,

retransmission				[27] NULL OPTIONAL,

servingNodePLMNIdentifier	[28] PLMN-Id OPTIONAL,

userLocationInfoTime		[29] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[30] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMTRecord	::= SET

--

--   also for MME UE terminated SMS record

--

{

recordType					 [0] RecordType,

servedIMSI					 [1] IMSI,

servedIMEI					 [2] IMEI OPTIONAL,

servedMSISDN				 [3] MSISDN OPTIONAL,

msNetworkCapability			 [4] MSNetworkCapability OPTIONAL,

serviceCentre				 [5] AddressString OPTIONAL,

recordingEntity				 [6] RecordingEntity OPTIONAL,

locationArea				 [7] LocationAreaCode OPTIONAL,

routingArea					 [8] RoutingAreaCode OPTIONAL,

cellIdentifier				 [9] CellId OPTIONAL,

eventTimeStamp				 [10] TimeStamp,

smsResult					 [11] SMSResult OPTIONAL,

recordExtensions			 [12] ManagementExtensions OPTIONAL,

nodeID						 [13] NodeID OPTIONAL,

localSequenceNumber			 [14] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [15] ChargingCharacteristics,

rATType						 [16] RATType OPTIONAL,

chChSelectionMode			 [17] ChChSelectionMode OPTIONAL,

cAMELInformationSMS			 [18] CAMELInformationSMS OPTIONAL,

originatingAddress			 [19] AddressString OPTIONAL,

servingNodeType				 [20] ServingNodeType,

servingNodeAddress			 [21] GSNAddress OPTIONAL,

servingNodeiPv6Address		 [22] GSNAddress OPTIONAL,

mMEName						 [23] DiameterIdentity OPTIONAL,

mMERealm					 [24] DiameterIdentity OPTIONAL,

userLocationInformation		 [25] OCTET STRING OPTIONAL,

retransmission				 [26] NULL OPTIONAL,

servingNodePLMNIdentifier	 [27] PLMN-Id OPTIONAL,

userLocationInfoTime		 [28] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMTLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType,

lcsClientIdentity			 [3] LCSClientIdentity,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationType				 [7] LocationType,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

notificationToMSUser		 [13] NotificationToMSUser OPTIONAL,

privacyOverride				 [14] NULL OPTIONAL,

location					 [15] LocationAreaAndCell OPTIONAL,

routingArea					 [16] RoutingAreaCode OPTIONAL,

locationEstimate			 [17] Ext-GeographicalInformation OPTIONAL,

positioningData				 [18] PositioningData OPTIONAL,

lcsCause					 [19] LCSCause OPTIONAL,

diagnostics					 [20] Diagnostics OPTIONAL,

nodeID						 [21] NodeID OPTIONAL,

localSequenceNumber			 [22] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [23] ChargingCharacteristics,

chChSelectionMode			 [24] ChChSelectionMode OPTIONAL,

rATType						 [25] RATType OPTIONAL,

recordExtensions			 [26] ManagementExtensions OPTIONAL,

causeForRecClosing			 [27] CauseForRecClosing,

servingNodePLMNIdentifier	 [28] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMOLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationMethod				 [7] LocationMethod,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNNILCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI OPTIONAL,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

servedIMEI					 [7] IMEI OPTIONAL,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofRAs					[3] SEQUENCE OF RAIdentity OPTIONAL,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

sgsnPLMNIdentifier			[15] PLMN-Id OPTIONAL,

numberofReceivingUE			[16] INTEGER OPTIONAL,

mbmsInformation				[17] MBMSInformation OPTIONAL

}

GGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

mbmsInformation				[15] MBMSInformation OPTIONAL

}

GWMBMSRecord	::= SET

{

recordType					[0] RecordType,

mbmsGWAddress				[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

pdpPDNType					[5] PDPType OPTIONAL,

servedPDPPDNAddress			[6] PDPAddress OPTIONAL,

listOfTrafficVolumes		[7] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[8] TimeStamp,

duration					[9] CallDuration,

causeForRecClosing			[10] CauseForRecClosing,

diagnostics					[11] Diagnostics OPTIONAL,

recordSequenceNumber		[12] INTEGER OPTIONAL,

nodeID						[13] NodeID OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

commonTeid   				[17] CTEID OPTIONAL,

iPMulticastSourceAddress	[18] PDPAddress OPTIONAL

}

--

--  PS DATA TYPES

--

AccessAvailabilityChangeReason		::= INTEGER (0..4294967295)

--

-- 0 (RAN rule indication)   : This value shall be used to indicate that the availability

--                             of an access is changed due to the RAN rule indication.

-- 1 (Access usable/unusable): This value shall be used to indicate that the availability

--                             of an access is changed due to the access is unusable or usable

--                             again.

--

AccessLineIdentifier	::= SEQUENCE

--

-- "Physical Access Id" includes a port identifier and the identity of the access node where the

--  port resides. "logical Access Id" contains a Circuit-ID. Both are defined ETSI TS 283 034 [314]

--

{

physicalAccessID	[0] UTF8String OPTIONAL,

logicalAccessID		[1] OCTET STRING OPTIONAL

}

AccessPointNameNI	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of APN in  dot representation.

-- For example, if the complete APN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- NI is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

AccessPointNameOI	::= IA5String (SIZE(1..37))

--

-- Operator Identifier part of APN in dot representation.

-- In the 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs' example, the OI portion is 'mnc022.mcc111.gprs'

-- and is presented in this form in the CDR.

--

ADCRuleBaseName		::= IA5String

--

-- identifier for the group of charging rules

-- see ADC-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

AdditionalExceptionReports		::= ENUMERATED

{

notAllowed		(0),

allowed			(1)

}

AFChargingIdentifier	::= OCTET STRING

--

-- see AF-Charging-Identifier AVP as defined in TS 29.214[221]

--

AFRecordInformation	::= SEQUENCE

{

aFChargingIdentifier		[1] AFChargingIdentifier,

flows						[2] Flows OPTIONAL

}

APNRateControl		::= SEQUENCE

--

-- See TS 24.008 [208] for more information

--

{

aPNRateControlUplink	[0] APNRateControlParameters OPTIONAL,

aPNRateControlDownlink	[1] APNRateControlParameters OPTIONAL

}

APNRateControlParameters		::= SEQUENCE

{

additionalExceptionReports	[0] AdditionalExceptionReports OPTIONAL,

rateControlTimeUnit			[1] RateControlTimeUnit OPTIONAL,

rateControlMaxRate			[2] INTEGER OPTIONAL,

rateControlMaxMessageSize	[3] DataVolumeGPRS OPTIONAL -- aPNRateControlDownlink only

}

APNSelectionMode	::= ENUMERATED

--

-- See Information Elements TS 29.060 [215], TS 29.274 [223] or TS 29.275 [224]

--

{

mSorNetworkProvidedSubscriptionVerified				(0),

mSProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

CAMELAccessPointNameNI	::= AccessPointNameNI

CAMELAccessPointNameOI	::= AccessPointNameOI

CAMELInformationMM		::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

numberOfDPEncountered  			[4] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[5] LevelOfCAMELService OPTIONAL,

freeFormatData					[6] FreeFormatData OPTIONAL,

fFDAppendIndicator				[7] FFDAppendIndicator OPTIONAL

}

CAMELInformationPDP	::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

cAMELAccessPointNameNI			[4] CAMELAccessPointNameNI OPTIONAL,

cAMELAccessPointNameOI			[5] CAMELAccessPointNameOI OPTIONAL,

numberOfDPEncountered			[6] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[7] LevelOfCAMELService OPTIONAL,

freeFormatData					[8] FreeFormatData OPTIONAL,

fFDAppendIndicator				[9] FFDAppendIndicator OPTIONAL

}

CAMELInformationSMS		::= SET

{

sCFAddress								[1] SCFAddress OPTIONAL,

serviceKey								[2] ServiceKey OPTIONAL,

defaultSMSHandling						[3] DefaultSMS-Handling OPTIONAL,

cAMELCallingPartyNumber					[4] CallingNumber OPTIONAL,

cAMELDestinationSubscriberNumber		[5] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[6] AddressString OPTIONAL,

freeFormatData							[7] FreeFormatData OPTIONAL,

smsReferenceNumber						[8] CallReferenceNumber	OPTIONAL

}

ChangeCondition	::= ENUMERATED

{

qoSChange						(0),

tariffTime						(1),

recordClosure					(2),

cGI-SAICHange					(6),	-- bearer modification. "CGI-SAI Change"

rAIChange						(7),	-- bearer modification. "RAI Change"

dT-Establishment				(8),

dT-Removal						(9),

eCGIChange						(10),	-- bearer modification. "ECGI Change"

tAIChange						(11),	-- bearer modification. "TAI Change"

userLocationChange				(12),	-- bearer modification. "User Location Change"

userCSGInformationChange		(13), 	-- bearer modification. "User CSG info Change"

presenceInPRAChange				(14),	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

removalOfAccess					(15),	-- NBIFOM "Removal of Access"

unusabilityOfAccess				(16),	-- NBIFOM "Unusability of Access"

indirectChangeCondition			(17),	-- NBIFOM "Indirect Change Condition"

userPlaneToUEChange				(18),	-- bearer modification. "Change of user plane to UE"

servingPLMNRateControlChange	(19),

-- bearer modification “Serving PLMN Rate Control Change"

threeGPPPSDataOffStatusChange	(20),   -- "Change of 3GPP PS DataO ff Status"

aPNRateControlChange			(21)	-- bearer modification. "APN Rate ControlChange"

}

ChangeOfCharCondition	::= SEQUENCE

--

-- qosRequested and qosNegotiated are used in S-CDR only

-- ePCQoSInformation used in SGW-CDR,PGW-CDR, IPE-CDR, TWAG-CDR and ePDG-CDR only

-- userLocationInformation is used only in S-CDR, SGW-CDR and PGW-CDR

-- chargingID used in PGW-CDR only when Charging per IP-CAN session is active

-- accessAvailabilityChangeReason and relatedChangeOfCharCondition applicable only in PGW-CDR

-- cPCIoTOptimisationIndicator is used in SGW-CDR only

-- aPNRateControl is valid for PGW-CDR only

--

{

qosRequested						[1] QoSInformation OPTIONAL,

qosNegotiated							[2] QoSInformation OPTIONAL,

dataVolumeGPRSUplink					[3] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink					[4] DataVolumeGPRS OPTIONAL,

changeCondition							[5] ChangeCondition,

changeTime								[6] TimeStamp,

userLocationInformation					[8] OCTET STRING OPTIONAL,

ePCQoSInformation						[9] EPCQoSInformation OPTIONAL,

chargingID								[10] ChargingID OPTIONAL,

presenceReportingAreaStatus 			[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation						[12] UserCSGInformation OPTIONAL,

diagnostics								[13] Diagnostics OPTIONAL,

enhancedDiagnostics						[14] EnhancedDiagnostics OPTIONAL,

rATType									[15] RATType OPTIONAL,

accessAvailabilityChangeReason			[16] AccessAvailabilityChangeReason OPTIONAL,

uWANUserLocationInformation				[17] UWANUserLocationInfo OPTIONAL,

relatedChangeOfCharCondition			[18] RelatedChangeOfCharCondition OPTIONAL,

cPCIoTEPSOptimisationIndicator			[19] CPCIoTEPSOptimisationIndicator OPTIONAL,

servingPLMNRateControl					[20] ServingPLMNRateControl OPTIONAL,

threeGPPPSDataOffStatus					[21] ThreeGPPPSDataOffStatus OPTIONAL,

listOfPresenceReportingAreaInformation	[22] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL,

aPNRateControl							[23] APNRateControl OPTIONAL

}

ChangeOfMBMSCondition	::= SEQUENCE

--

-- Used in MBMS record

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeMBMSUplink		[3] DataVolumeMBMS OPTIONAL,

dataVolumeMBMSDownlink		[4] DataVolumeMBMS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL

}

ChangeOfServiceCondition	::= SEQUENCE

--

-- Used for Flow based Charging and Application based Charging service data container

-- presenceReportingAreaStatus is used in PGW-CDR Only

--

{

ratingGroup 						[1] RatingGroupId,

chargingRuleBaseName				[2] ChargingRuleBaseName OPTIONAL,

resultCode							[3] ResultCode OPTIONAL,

localSequenceNumber					[4] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage					[5] TimeStamp OPTIONAL,

timeOfLastUsage						[6] TimeStamp OPTIONAL,

timeUsage 							[7] CallDuration OPTIONAL,

serviceConditionChange				[8] ServiceConditionChange,

qoSInformationNeg					[9] EPCQoSInformation OPTIONAL,

servingNodeAddress 					[10] GSNAddress OPTIONAL,

datavolumeFBCUplink					[12] DataVolumeGPRS OPTIONAL,

datavolumeFBCDownlink				[13] DataVolumeGPRS OPTIONAL,

timeOfReport						[14] TimeStamp,

failureHandlingContinue				[16] FailureHandlingContinue OPTIONAL,

serviceIdentifier					[17] ServiceIdentifier OPTIONAL,

pSFurnishChargingInformation		[18] PSFurnishChargingInformation OPTIONAL,

aFRecordInformation					[19] SEQUENCE OF AFRecordInformation OPTIONAL,

userLocationInformation				[20] OCTET STRING OPTIONAL,

eventBasedChargingInformation		[21] EventBasedChargingInformation OPTIONAL,

timeQuotaMechanism					[22] TimeQuotaMechanism OPTIONAL,

serviceSpecificInfo					[23] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

sponsorIdentity						[25] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[26] OCTET STRING OPTIONAL,

aDCRuleBaseName						[27] ADCRuleBaseName OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedChangeOfServiceCondition		[33] RelatedChangeOfServiceCondition OPTIONAL,

servingPLMNRateControl				[35] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[36] APNRateControl OPTIONAL,

threeGPPPSDataOffStatus             [37] ThreeGPPPSDataOffStatus OPTIONAL,

trafficSteeringPolicyIDDownlink     [38] TrafficSteeringPolicyIDDownlink OPTIONAL,

trafficSteeringPolicyIDUplink       [39] TrafficSteeringPolicyIDUplink OPTIONAL,

tWANUserLocationInformation			[40] TWANUserLocationInfo OPTIONAL,

listOfPresenceReportingAreaInformation	[41] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL

}

ChangeLocation	::= SEQUENCE

--

-- used in SGSNMMRecord only

--

{

locationAreaCode		[0] LocationAreaCode,

routingAreaCode			[1] RoutingAreaCode,

cellId					[2] CellId OPTIONAL,

changeTime				[3] TimeStamp,

mCC-MNC					[4] PLMN-Id OPTIONAL

}

ChargingCharacteristics	::= OCTET STRING (SIZE(2))

ChargingPerIPCANSessionIndicator	::= ENUMERATED

{

inactive		(0),

active               (1)

}

ChargingRuleBaseName	::= IA5String

--

-- identifier for the group of charging rules

-- see Charging-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

ChChSelectionMode		::= ENUMERATED

{

servingNodeSupplied			(0),	-- For S-GW/P-GW

subscriptionSpecific		(1),	-- For SGSN only

aPNSpecific					(2),	-- For SGSN only

homeDefault					(3),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

roamingDefault				(4),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

visitingDefault				(5),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

fixedDefault				(6)		-- For TDF and IP-Edge

}

CNOperatorSelectionEntity	::= ENUMERATED

{

servCNSelectedbyUE		(0),

servCNSelectedbyNtw		(1)

}

CPCIoTEPSOptimisationIndicator ::= BOOLEAN

CSGAccessMode		::= ENUMERATED

{

closedMode  (0),

hybridMode  (1)

}

CSGId		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 23.003 [200]. Coded according to TS 29.060 [215] for GTP, and

-- in TS 29.274 [223] for eGTP.

--

CTEID		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 32.251[11] for MBMS-GW-CDR. Common Tunnel Endpoint Identifier

-- of MBMS GW for user plane, defined in TS 23.246 [207].

--

DataVolumeGPRS		::= INTEGER

--

-- The volume of data transferred in octets.

--

DataVolumeMBMS ::= INTEGER

--

-- The volume of data transferred in octets.

--

EPCQoSInformation	::= SEQUENCE

--

-- See TS 29.212 [220] for more information

--

{

qCI							[1] INTEGER,

maxRequestedBandwithUL		[2] INTEGER OPTIONAL,

maxRequestedBandwithDL		[3] INTEGER OPTIONAL,

guaranteedBitrateUL			[4] INTEGER OPTIONAL,

guaranteedBitrateDL			[5] INTEGER OPTIONAL,

aRP							[6] INTEGER OPTIONAL,

aPNAggregateMaxBitrateUL	[7] INTEGER OPTIONAL,

aPNAggregateMaxBitrateDL	[8] INTEGER OPTIONAL,

extendedMaxRequestedBWUL	[9] INTEGER OPTIONAL,

extendedMaxRequestedBWDL	[10] INTEGER OPTIONAL,

extendedGBRUL				[11] INTEGER OPTIONAL,

extendedGBRDL				[12] INTEGER OPTIONAL,

extendedAPNAMBRUL			[13] INTEGER OPTIONAL,

extendedAPNAMBRDL			[14] INTEGER OPTIONAL

}

EventBasedChargingInformation		::= SEQUENCE

{

numberOfEvents		[1] INTEGER,

eventTimeStamps  	[2] SEQUENCE OF TimeStamp OPTIONAL

}

FailureHandlingContinue		::= BOOLEAN

--

-- This parameter is included when the failure handling procedure has been executed and new

-- containers are opened. This parameter shall be included in the first and subsequent

-- containers opened after the failure handling execution.

--

FFDAppendIndicator	::= BOOLEAN

FixedSubsID	::= OCTET STRING

--

-- The fixed subscriber Id identifier is defined in Broadband Forum TR 134 [601].

--

FixedUserLocationInformation	::= SEQUENCE

--

-- See format in IEEE Std 802.11-2012 [408] for "SSID" and "BSSID".

--

{

sSID					[0] OCTET STRING OPTIONAL ,

bSSID					[1] OCTET STRING OPTIONAL,

accessLineIdentifier	[2] AccessLineIdentifier OPTIONAL

}

Flows		::= 	SEQUENCE

--

-- See Flows AVP as defined in TS 29.214 [221]

--

{

mediaComponentNumber	[1] INTEGER,

flowNumber  			[2] SEQUENCE OF INTEGER OPTIONAL

}

FreeFormatData		::=	OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FurnishChargingInformationGPRS

-- see TS 29.078 [217]

--

GSNAddress		::= IPAddress

MOExceptionDataCounter		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

counterValue		[0] INTEGER,

counterTimestamp	[1] TimeStamp

}

MSNetworkCapability	::= OCTET STRING (SIZE(1..8))

--

-- see TS 24.008 [208]

--

NBIFOMMode		::= ENUMERATED

{

uEINITIATED				(0),

nETWORKINITIATED		(1)

}

NBIFOMSupport 	::= ENUMERATED

{

nBIFOMNotSupported			(0),

nBIFOMSupported				(1)

}

NetworkInitiatedPDPContext		::= BOOLEAN

--

-- Set to true if PDP context was initiated from network side

--

NumberOfDPEncountered  ::= INTEGER

PDPType		::= OCTET STRING (SIZE(2))

--

-- OCTET 1: PDP Type Organization

-- OCTET 2: PDP/PDN Type Number

-- See TS 29.060 [215] for encoding details.

--

PDPPDNTypeExtension	::= INTEGER

--

-- This integer is 1:1 copy of the PDP type value as defined in TS 29.061 [215].

--

PresenceReportingAreaElementsList ::= OCTET STRING

--

-- See Presence-Reporting-Area-Elements-List AVP defined in TS 29.212 [220]

--

PresenceReportingAreaInfo	::= SEQUENCE

{

presenceReportingAreaIdentifier	 [0] OCTET STRING,

presenceReportingAreaStatus		 [1] PresenceReportingAreaStatus OPTIONAL,

presenceReportingAreaElementsList[2] PresenceReportingAreaElementsList OPTIONAL,

presenceReportingAreaNode		 [3] PresenceReportingAreaNode OPTIONAL

}

PresenceReportingAreaNode ::= BIT STRING

{

oCS   		 (0),

pCRF  		 (1)

}

--

--	This bit mask has the same format as Presence-Reporting-Area-Node AVP in TS 29.212 [220]

--

PresenceReportingAreaStatus ::= ENUMERATED

{

insideArea   (0),

outsideArea  (1),

inactive 	 (2)

}

PSFurnishChargingInformation		::= SEQUENCE

{

pSFreeFormatData		[1] FreeFormatData,

pSFFDAppendIndicator	[2] FFDAppendIndicator OPTIONAL

}

QoSInformation	::= OCTET STRING (SIZE (4..255))

--

-- This  octet string

-- is a 1:1 copy of the contents (i.e. starting with octet 5) of the "Bearer Quality of

-- Service" information element specified in TS 29.274 [223].

--

RANSecondaryRATUsageReport	::= SEQUENCE

--

{

dataVolumeUplink				[1] DataVolumeGPRS,

dataVolumeDownlink				[2] DataVolumeGPRS,

rANStartTime					[3] TimeStamp,

rANEndTime						[4] TimeStamp,

secondaryRATType				[5] SecondaryRATType OPTIONAL,

chargingID						[6] ChargingID OPTIONAL

}

RateControlTimeUnit ::= INTEGER

{	unrestricted	(0),

minute			(1),

hour			(2),

day				(3),

week			(4)

}

RatingGroupId		::= INTEGER

--

-- IP service flow identity (DCCA), range of 4 byte (0... 4294967295)

-- see Rating-Group AVP as used in TS 32.299 [50]

--

RelatedChangeOfCharCondition	::= SEQUENCE

{

changeCondition					[5] ChangeCondition,

changeTime						[6] TimeStamp,

userLocationInformation			[8] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 	[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation				[12] UserCSGInformation OPTIONAL,

rATType							[15] RATType OPTIONAL,

uWANUserLocationInformation		[17] UWANUserLocationInfo OPTIONAL

}

RelatedChangeOfServiceCondition	::= SEQUENCE

{

userLocationInformation				[20] OCTET STRING OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedServiceConditionChange		[33] ServiceConditionChange OPTIONAL

}

ResultCode		::= INTEGER

--

-- charging protocol return value, range of 4 byte (0... 4294967295)

-- see Result-Code AVP as used in 32.299 [40]

--

SecondaryRATType	::= INTEGER

{

nR				(0)		-- New Radio 5G

}

ServiceConditionChange	::= BIT STRING

{

qoSChange 								 (0),	-- bearer modification

sGSNChange 								 (1),	-- bearer modification:

-- apply to Gn-SGSN /SGW Change

sGSNPLMNIDChange 						 (2),	-- bearer modification

tariffTimeSwitch 						 (3),	-- tariff time change

pDPContextRelease 						 (4),	-- bearer release

rATChange 								 (5),	-- bearer modification

serviceIdledOut 						 (6),	-- IP flow idle out, DCCA QHT expiry

reserved 								 (7),	-- old: QCTexpiry is no report event

configurationChange 					 (8),	-- configuration change

serviceStop 							 (9),	-- IP flow termination.From "Service Stop" in

-- Change-Condition AVP

dCCATimeThresholdReached 				(10),	-- DCCA quota reauthorization

dCCAVolumeThresholdReached 				(11),	-- DCCA quota reauthorization

dCCAServiceSpecificUnitThresholdReached	(12),	-- DCCA quota reauthorization

dCCATimeExhausted 						(13),	-- DCCA quota reauthorization

dCCAVolumeExhausted 					(14),	-- DCCA quota reauthorization

dCCAValidityTimeout 					(15),	-- DCCA quota validity time (QVT expiry)

reserved1								(16),	-- reserved due to no use case,

-- old: return Requested is covered by (17),(18)

dCCAReauthorisationRequest 				(17),	-- DCCA quota reauthorization request by OCS

dCCAContinueOngoingSession 				(18),	-- DCCA failure handling (CCFH),

-- continue IP flow

dCCARetryAndTerminateOngoingSession		(19),	-- DCCA failure handling (CCFH),

-- terminate IP flow after DCCA retry

dCCATerminateOngoingSession 			(20),	-- DCCA failure handling,

-- terminate IP flow

cGI-SAIChange							(21),	-- bearer modification. "CGI-SAI Change"

rAIChange								(22),	-- bearer modification. "RAI Change"

dCCAServiceSpecificUnitExhausted		(23),	-- DCCA quota reauthorization

recordClosure							(24),	-- PGW-CDR closure

timeLimit								(25),	-- intermediate recording. From "Service Data

--	Time Limit" Change-Condition AVP value

volumeLimit								(26),	-- intermediate recording.From "Service Data

--	Volume Limit" Change-Condition AVP value

serviceSpecificUnitLimit				(27),	-- intermediate recording

envelopeClosure 						(28),

eCGIChange								(29),	-- bearer modification. "ECGI Change"

tAIChange								(30),	-- bearer modification. "TAI Change"

userLocationChange						(31),	-- bearer modification. "User Location Change"

userCSGInformationChange				(32),	-- bearer modification. "User CSG info Change"

presenceInPRAChange						(33), 	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

accessChangeOfSDF						(34),	-- "access change of service data flow"

indirectServiceConditionChange			(35),	-- NBIFOM: "indirect service condition change"

servingPLMNRateControlChange			(36), 	-- bearer modification. "Serving PLMNRate

-- Control Change"

aPNRateControlChange					(37)	-- bearer modification. "APN Rate ControlChange

}

--

-- Trigger and cause values for IP flow level recording are defined for support of independent

-- online and offline charging and also for tight interworking between online and offline charging.

-- Unused bits will always be zero.

-- Some of the values are non-exclusive (e.g. bearer modification reasons).

--

SCFAddress	::= AddressString

--

-- See TS 29.002 [214]

--

ServiceIdentifier	::= INTEGER (0..4294967295)

--

-- The service identifier is used to identify the service or the service component

-- the service data flow relates to. See Service-Identifier AVP as defined in TS 29.212 [220]

--

ServingNodeType	::= ENUMERATED

{

sGSN		(0),

pMIPSGW		(1),

gTPSGW		(2),

ePDG		(3),

hSGW		(4),

mME			(5),

tWAN		(6)

}

ServingPLMNRateControl		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

sPLMNDLRateControlValue	[0] INTEGER,

sPLMNULRateControlValue	[1] INTEGER

}

SGiPtPTunnellingMethod		::= ENUMERATED

{

uDPIPbased		(0),

others			(1)

}

SGSNChange	::= BOOLEAN

--

-- present if first record after inter SGSN routing area update in new SGSN

--

SGWChange		::= BOOLEAN

--

-- present if first record after inter serving node change (SGW, ePDG, TWAG, HSGW)

--

TimeQuotaMechanism		::= SEQUENCE

{

timeQuotaType					[1] TimeQuotaType,

baseTimeInterval				[2] INTEGER

}

TimeQuotaType		::= ENUMERATED

{

dISCRETETIMEPERIOD			(0),

cONTINUOUSTIMEPERIOD		(1)

}

TrafficSteeringPolicyIDDownlink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-DL AVP as defined in TS 29.212[220]

TrafficSteeringPolicyIDUplink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-UL AVP as defined in TS 29.212[220]

TWANUserLocationInfo		::= SEQUENCE

{

sSID					 [0] OCTET STRING,			-- see format in IEEE Std 802.11-2012 [408]

bSSID					 [1] OCTET STRING OPTIONAL, 	-- see format in IEEE Std 802.11-2012 [408]

civicAddressInformation	[2] CivicAddressInformation OPTIONAL,

wLANOperatorId			[3] WLANOperatorId OPTIONAL,

logicalAccessID			[4] OCTET STRING OPTIONAL

}

UNIPDUCPOnlyFlag	::= BOOLEAN

UserCSGInformation		::= SEQUENCE

{

cSGId						[0] CSGId,

cSGAccessMode				[1] CSGAccessMode,

cSGMembershipIndication		[2] NULL OPTIONAL

}

UWANUserLocationInfo		::= SEQUENCE

{

uELocalIPAddress		 [0] IPAddress,

uDPSourcePort			 [1] OCTET STRING (SIZE(2)) OPTIONAL,

sSID					 [2] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

bSSID					 [3] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

tCPSourcePort			[4] OCTET STRING (SIZE(2)) OPTIONAL,

civicAddressInformation	[5] CivicAddressInformation OPTIONAL,

wLANOperatorId			[6] WLANOperatorId OPTIONAL,

logicalAccessID			[7] OCTET STRING OPTIONAL

}

WLANOperatorId ::= SEQUENCE

{

wLANOperatorName	[0] OCTET STRING,

wLANPLMNId			[1] PLMN-Id

}

.#END

End of change