3GPP TSG-SA5 Meeting #125 	S5-193326

# Newport Beach, US, 8-12 April 2019												Revision of S5-193282

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0008 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NEF API trigger information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_NEFCH |  |  |  |  |  | Date: |  |  | 2019-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NEF API trigger information is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding trigger information for the NEF API charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NEF cannot trigger charging using Converged Charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.4	NEF API converged charging scenarios

### 5.4.1	Basic principles

#### 5.4.1.1	General

Converged charging may be performed by the NEF interacting with CHF using Nchf specified in TS 32.290 [19] and TS 32.291 [20]. In order to provide the data required for the management activities outlined in TS 32.240 [2] (Credit-Control, accounting, billing, statistics etc.), the NEF shall be able to perform converged charging for each of the API requests and notifications.

The NEF shall be able to perform convergent charging by interacting with CHF, for charging data related to API invocation or notification. The Charging Data Request and Charging Data Response are exchanged between the NEF and the CHF, based on either IEC or ECUR scenarios specified in TS 32.290 [19]. The Charging Data Request is issued by the NEF towards the CHF when certain conditions (chargeable events) are met.

Converged charging uses centralized or decentralized unit determination and centralized rating scenarios for event based convergent charging specified in TS 32.290 [19].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [20].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [3].

The selection of the CHF can be configured in the NEF but may also rely on NRF.

The chargeable events or messages exchanged between the NEF and the other nodes are described with generic names 
(i.e API Invocation, API notification), to reflect API usage to/from the NEF, independently from the actual API used.

#### 5.4.1.2	Applicable Triggers in the NEF

##### 5.4.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier.

Each trigger condition (i.e. chargeable event) defined for the NEF API charging functionality, is specified with the associated behaviour when they are met.

When an API request is sent to the NEF, and the converged charging is activated, the API request invokes a Charging Data Request [Initial] towards the CHF to get authorization to start in ECUR mode. In IEC mode, the Charging Data Request [Event] is sent towards the CHF.

Table 5.4.1.2.1 summarizes the set of default trigger conditions and their category which shall be supported by the NEF. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Termination] message sent from NEF towards the CHF.

Table 5.4.1.2.1: Default Trigger conditions in NEF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| API Invocation Request | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Initial] |
| API Invocation Response | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |
| API Notification to AF | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Notification Acknowledge | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |

For converged charging, the following details of chargeable events and corresponding actions in the NEF are defined in Table 5.4.1.2.2:

Table 5.4.1.2.2: Chargeable events and their related actions in NEF

| Chargeable event | Conditions | NEF action |
| --- | --- | --- |
| API Invocation Request |  | ECUR: Charging Data Request [Initial] with a possible request quota for later use |
| API Invocation Response |  | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination], indicating that charging session is terminated |
| API Notification to AF |  | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] with a possible request quota for later use |
| Notification Acknowledge |  | ECUR: Charging Data Request [Termination], indicating that charging session is terminated |

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event, Initial, Termination] issued by the NEF for these chargeable events, is specified in clause 5.4.3.

End of change