3GPP TSG-SA5 Meeting #125 	S5-193317

# Newport Beach, US, 8-12 April 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0071 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Offline only charging service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The offline only charging service needs to be added in TS 32.291. The proper operations and procedures when applies offline only charging service should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support offline only charging service definition, a number of changes are added in TS 32.291: The support of offline only charging service through Nchf interface; The operations of offline only charging service are clarified; The procedures of operators from protocol perspective are given. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The offline charging service through Nchf interface is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 5.1, 5.3 (new), 5.3.1 (new), 5.3.2 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change to TS 32.291

4.1	Service architecture

The Converged Charging Service or Offline Only Charging Service is provided by the CHF to the consumer and shown in the SBI representation model in figure 4. 1.1. The 5G Data connectivity domain charging is depicted in 3GPP TS 32.255 [30].

The ConvergedCharging Service (Nchf_ ConvergedCharging) or OfflineOnlyCharging Service (Nchf_OfflineOnlyCharging) is part of the Nchf service-based interface exhibited by the Charging Function (CHF), with SMF and SMSF as the NF Service Consumer.

Figure 4. 1.1: Reference Architecture for the Nchf_ConvergedCharging Service; SBI representation

2nd change to TS 32.291

5.1	Introduction

The following services are provided by the CHF.

Table 5.1-1: NF Services provided by CHF

| Service Name | Description | Consumer |
| --- | --- | --- |
| Nchf_ConvergedCharging service | This service provides a converged charging for session and event based NF services, with and without quota management, as well as charging information record generation | SMF, SMSF |
| Nchf_OfflineOnlyCharging service | This service provides an offline only charging for session based NF service. | SMF |
| Nchf_SpendingLimitControl | This service enables the PCF to retrieve policy counter status information per UE from the CHF by subscribing to spending limit reporting (i.e. notifications of policy counter status changes). | PCF |

The "Nchf_SpendingLimitControl" service is defined in 29.594 [301].

3rd change to TS 32.291

5.3	Nchf_OfflineOnlyCharging service

5.3.1	Service Description

This service provides charging in offline only charging scenario by the CHF to the NF service consumer (i.e. SMF) as defined in subclause 6.5 in 3GPP TS 32.290 [58].

It includes the following functionalities:

-	Create resource at service establishment based on the request from NF consumer;

-	During the service consumption lifecycle, update resource based on the request from NF consumer;

-	Release upon service termination;

-	Charging information record generation.

5.3.2	Service Operations

5.3.2.1	Introduction

The service operations defined for Nchf_OfflineOnlyCharging are shown in table 5.3.2.1-1.

# Table 5.3.2.1-1: Nchf_OfflineOnlyCharging Operations

| Service Operation Name | Description | Initiated by | Corresponding Offline only charging messages in 3GPP TS 32.290[58] |
| --- | --- | --- | --- |
| Nchf_OfflineOnlyCharging_Create | Initial report of service usage. | NF consumer | Charging Data Request/Response [Initial] |
| Nchf_OfflineOnlyCharging_Update | Intermediate report of service usage. | NF consumer | Charging Data Request/Response [Update] |
| Nchf_OfflineOnlyCharging_Release | Last report of service usage. | NF consumer | Charging Data Request/Response [Termination] |

5.3.2.2	Nchf_OfflineOnlyCharging_Create Operation

The Nchf_OfflineOnlyCharging_Create operation provides means for NF (CTF) to request initial report of service usage.

The following procedures using the Nchf_OfflineOnlyCharging_Create service operation are supported:

-	No existing charging data resource.

Figure 5.3.2.2-1: Nchf_OfflineOnlyCharging_Create Service Operation

1.  NF (CTF) sends a Nchf_OfflineOnlyCharging_Create request to the CHF to create resource for starting charging.

2a.  At successful operation, "201 Created" response is returned. In the "201 Created" response, the CHF includes a Location header field in the body. The Location header field shall contain the URI of the created resource. The NF (CTF) shall use the URI received in the Location header in subsequent requests to the CHF for the same PDU session.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.x.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.x.7.3-1.

5.3.2.3	Nchf_OfflineOnlyCharging_Update Operation

The Nchf_OfflineOnlyCharging_Update operation provides means for NF (CTF) to update the charging data.

The following procedures using the Nchf_OfflineOnlyCharging_Update service operation are supported:

-	charging events occur.

Figure 5.3.2.3-1: Nchf_OfflineOnlyCharging_Update Service Operation

1. NF (CTF) sends a Nchf_OfflineOnlyCharging_Update request to the CHF. The {OfflineChargingDataRef} in the URI identifies the "Offline Only Charging Data" to be updated. The used service unit is included in the request body.

2a. At successful operation, "200 OK" response is returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.x.3.3.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.x.7.3-1.

5.3.2.4	Nchf_OfflineOnlyCharging_Release Operation

The Nchf_OfflineOnlyCharging_Release service operation provides means for NF (CTF) to terminate charging Session.

The following procedures using the Nchf_OfflineOnlyCharging_Release service operation are supported.

Figure 5.3.2.4-1: Nchf_OfflineOnlyCharging_Release Service Operation

1. NF(CTF) sends a Nchf_OfflineOnlyCharging_Release request to the CHF. The {OfflineChargingDataRef} in the URI identifies the "Offline Only Charging Data" to be updated and then released. The final used service unit is included in the request body.

2a. At successful operation, "204 No Content" response is returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.x.3.3.4.3.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.x.7.3-1.

End of change