3GPP TSG-WG SA2 Meeting #136-AH	S2-2000343

# Incheon, Korea, 13 – 17 January, 2020	(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1989 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 5GS to EPS Idle mode mobility using N26 interface with I-SMF removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2020-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At last SA2 meeting most procedures for EPC IWK with ETSUN were updated to align with the general EPS IWK procedures for home-routed roaming. The 5GS to EPS Idle mode mobility using N26 interface with I-SMF removal was however not updated since the corresponding home-routed procedure in 4.11.1.3.2 required further work. That procedure has now been updated (23.502 CR 1875). The ETSUN procedure in 4.23.12.2 is now not aligned with 4.11.1.3.2 anymore, as e.g. release of V-SMF resources have been included in 4.11.1.3.2 at last SA2 meeting, which results in duplication since the corresponding steps are also shown separately in 4.23.12.2 Furthermore, it is better just refer to the HR roaming case, as is done with the other ETSUN procedures for EPC IWK. This can reuse the existing procedure and avoid duplication and potential inconsistencies. |  |  |  |  |  |  |  |  |
| xx |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 4.23.12.2 to refer to home-routed case 5GS to EPS Idle mode mobility |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent procedures and incorrect ETSUN call flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

#### 4.23.12.2	5GS to EPS Idle mode mobility using N26 interface with I-SMF removal

For 5GS to EPS Idle mode mobility using N26 with I-SMF removal, the procedure "5GS to EPS Idle mode mobility using N26 interface" defined in clause 4.11.1.3.2 for the home routed-roaming case are re-used, with the following change:

- The V-SMF is replaced by I-SMF, H-SMF is replaced by SMF, and V-UPF is replaced by I-UPF.

- The V-CN Tunnel Info is replaced by Tunnel Info at I-UPF, H-CN Tunnel Info is replaced by Tunnel Info at UPF (PSA).

***** End of Changes *****