3GPP TSG-SA2 Meeting # 148E (e-meeting)	S2-2108392

# Elbonia, November 15 - 22, 2021											 (revision of )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0172 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UE triggered Policy provisioning Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 6.2.4 of TS 23.287 states the following: 1. The UE sends the UE Policy provisioning request including the UE Policy Container (V2X Policy Provisioning Request) to the AMF. However, per stage 3 (TS 24.501), “UE Policy Provisioning Request” does not include UE Policy Container. Instead, “UE Policy Provisioning Request” is one of the UE policy delivery service types specified in “UE Policy Container” encapsulated in Payload Container IE carried in e.g. UL NAS TRANSPORT message. The content of “UE Policy Container” is transparent to the AMF, that is, the AMF is not aware of the UE policy delivery service type “UE Policy Provisioning Request”. The above means that step 2 of Figure 6.2.4-1 of TS 23.287 will not happen, i.e. the AMF would not send Npcf_UEPolicyControl_Update sent to PCF. Figure 6.2.4-1: UE triggered V2X Policy provisioning procedure To address issue, it’s proposed to follow TS 23.502 clause 4.2.4.3 for AMF interaction with PCF, see step 5 below: === TS 23.502==== 4.2.4.3 UE Configuration Update procedure for transparent UE Policy delivery Figure 4.2.4.3-1: UE Configuration Update procedure for transparent UE Policy deliver That is, when AMF receives UL NAS TRANSPORT message carrying UE Policy container, the AMF sends Namf_Communication_N1MessageNotify instead of Npcf_UEPolicyControl_Update to PCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the description related to UE Policy provisioning request. When AMF receives UL NAS TRANSPORT message carrying UE Policy container, the AMF sends Namf_Communication_N1MessageNotify instead of Npcf_UEPolicyControl_Update to PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not implementable requirement, misalignment with stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

### 6.2.4	Procedure for UE triggered V2X Policy provisioning

The UE triggered Policy Provisioning procedure is initiated by the UE to request V2X Policy/Parameter from the PCF when UE determines the V2X Policy/Parameter is invalid in the following cases:

-	if the validity timer indicated in the V2X Policy/Parameter expires;

-	if there are no valid parameters, e.g. for the V2X service type a UE wants to use, for current area, or due to abnormal situation.

Figure 6.2.4-1: UE triggered V2X Policy provisioning procedure

1.	The UE sends UL NAS TRANSPORT message carrying the UE Policy Container (UE V2X Policy Provisioning Request to request V2X policies) to the AMF.

2.	The AMF sends the Namf_Communication_N1MessageNotify request to the PCF including the UE Policy Container received from UE.

3.	The PCF receives UE Policy Container which indicates UE Policy Provisioning Request to request V2X policies. If the V2X policies are authorized based on AMF input as specified in clause 6.2.2, the PCF performs the UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [7].

****************** END CHANGES ***************