3GPP TSG-SA WG6 Meeting #45-bis-e	S6-212257

e-meeting, 11th – 19th October 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0300 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCGWUE 3GPP access network related location information management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell, FirstNet, BDBOS, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2021-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces information flows and procedures for handling the location information related to 3GPP access network by the MC service clients residing on the non-3GPP devices and accessing the MC service system through MC gateway UE. These information flows and procedures are based on the solution described in 3GPP TR 23.700-79. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 11.5.2 is filled with the required details capuring the information flows and procedures for management of 3GPP access network related location information by the MC service clients residing on non-3GPP devices. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented to the spec after implementing S6-212256_Introduction of subclauses to capture MC gateway UE function details |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 11.5.2	3GPP access network related location information management

#### 11.5.2.1	General

The MC clients residing on the non-3GPP devices may receive the location reporting configuration from the location management server containing the trigger criteria related to 3GPP access network related location parameters and the requested location information may contain the 3GPP access network related location information. The procedures defined in this subclause enables the MC clients residing on the non-3GPP devices to request the MC gateway UE to handle these triggers on its behalf and to request the required location information (3GPP access network related) from MC gateway UE.

#### 11.5.2.2	Information flows

##### 11.5.2.2.1	MC GW location reporting configuration

Table 11.5.2.2.1-1 describes the information flow from the MC client, which resides on a non-3GPP device, to the MC gateway UE for the location reporting configuration.

Table 11.5.2.2.1-1: MC GW location reporting configuration

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report (see NOTE 2) |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent |
| NOTE 1: If none of the information elements is present, this represents a cancellation for location reporting, if configured. NOTE 2: The triggering criteria contains only the events related to the 3GPP access network. |  |  |

##### 11.5.2.2.2	MC GW location information report

Table 11.5.2.2.2-1 describes the information flow from the MC gateway UE to the MC client residing on a non-3GPP device for the location information reporting.

Table 11.5.2.2.2-1: MC GW location information report

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user |
| Triggering event | O | Identity of the event that triggered the sending of the report |
| Location information (see NOTE) | O | Location information of the MC gateway UE |
| NOTE: The following location information elements which are related to 3GPP access network shall be present (configurable): Serving and neighbouring ECGI, MBMS SAIs, MBMSfnArea, PLMN ID. |  |  |

##### 11.5.2.2.3	MC GW location information request

Table 11.5.2.2.3-1 describes the information flow from the MC client residing on a non-3GPP device to the MC gateway UE for requesting an immediate location information report.

Table 11.5.2.2.3-1: MC GW location information request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user |

#### 11.5.2.3	Procedures

##### 11.5.2.3.1	Event triggered location reporting procedure

The procedure for how the MC clients residing on non-3GPP devices handling the location reporting configuration containing the trigger criteria related to the 3GPP access network related location information is shown in the figure 11.5.2.3.1-1.

Pre-conditions

-	The MC service user wishes to have access to MC services by using a non-3GPP device.

-	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.2.3.1-1: Event-triggered location reporting procedure

1.	MC client receives the location reporting configuration request from LMS which contains the triggering criteria of 3GPP access network related location information changes.

2.	MC client sends the MC GW location reporting configuration to the MC gateway UE containing the 3GPP access network related location information triggers and the requested location information. MC Gateway UE stores the location reporting configuration and starts monitoring for the triggers as received in the MC GW location reporting configuration.

3.	A location reporting event occurs, triggering step 4.

4.	MC gateway UE sends the MC GW Location information report containing the location information requested by the MC client.

5.	MC client updates the locally available location information with the location information received from the MC gateway UE.

6.	The MC client sends a location information report to the location management server, containing location information identified by the location management server and available to the MC client.

##### 11.5.2.3.2	On-demand location reporting procedure

The MC client may need to immediately send the location report to the location management sometimes and the requested location information may be related to the 3GPP access network. Under these circumstances the MC client can request the MC gateway UE to report its location information as described in the figure 11.5.2.3.2-1.

Pre-conditions

-	The MC service user wishes to have access to MC services by using a non-3GPP device.

-	The MC client successfully completed service authorization via MC Gateway UE.

Figure 11.5.2.3.2-1: On-demand location reporting procedure

1.	MC client receives the location information request from LMS to send the location information immediately or any other events where it has to send the location report to the location management server immediately like initial login, group call etc. Requested location information includes the location information related to 3GPP access network.

2.	MC service user is notified and asked for permission to share location information. MC service user can accept or deny the request.

3.	MC client sends the MC GW Location information request to the MC gateway requesting for the location information related to the 3GPP access network of the MC gateway UE.

4.	MC gateway UE sends the MC GW Location information report containing the location information requested by the MC client.

5.	MC client updates the locally available location information with the location information received from the MC gateway UE.

6.	The MC client sends a location information report to the location management server, containing location information identified by the location management server and available to the MC client.

##### 11.5.2.3.3	Location reporting cancel procedure

The location reporting cancel procedure reuses the information flow of location reporting configuration as defined in the subclause 11.5.2.3.1 as described in the figure 11.5.2.3.3-1

Pre-conditions

-	The MC service user wishes to have access to MC services by using a non-3GPP device.

-	The MC client successfully completed service authorization via MC gateway UE.

-	The MC client no longer needs the location information report from MC gateway UE.

Figure 11.5.2.3.3-1: On-demand location reporting procedure

1.	The location management client sends MC GW location reporting configuration without any information element to the MC gateway UE to stop location reporting from the MC gateway UE.

2.	The MC gateway UE stops sending location information reports to the MC client.