3GPP TSG-SA5 Meeting #145-e 	S5-225227

# E-meeting, 15-24 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.100 | CR | 0001 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct solution for fault management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ANL |  |  |  |  |  | Date: |  |  | 2022-7-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The solutions for generic MnS requirements is documented in clause 7.3.5 in TS 28.100, and the solution for level 3 is based on generic Fault supervision MnS as defined in TS 28.532. However such solution is not correct according to the current generic Fault supervision MnS. It is suggested to update the solution for fault management |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update REQ-ANL-FM-Level_3-MnS-6 by using MDA service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

7.3.5	Solutions for generic MnS requirements

Table 7.3.5-1: Solutions for generic MnS requirements of autonomous network level for fault management

| Autonomous Network Level | Requirements (Defined in Clause 7.1.4) | Corresponding solutions |
| --- | --- | --- |
| Level 1 | REQ-ANL-FM-Level_1-MnS-1 | This is implemented by fault supervision MnS defined in TS 28.532 [4], and provisioning MnS defined in TS 28.532 [4] with 5G Network Resource model defined in TS 28.541 [5]. |
|  | REQ-ANL-FM-Level_1-MnS-2 | This is implemented by fault supervision MnS defined in TS 28.532 [4], provisioning MnS defined in TS 28.532 [4] with 5G Network Resource model defined in TS 28.541 [5], PM control NRM fragment and Trace control NRM fragment define in TS 28.622 [6]. |
|  | REQ-ANL-FM-Level_1-MnS-3 | This can be implemented by utilizing fault supervision MnS (e.g. getAlarmList) defined in TS 28.532 [4] to specify the alarm filtering control information. |
|  | REQ-ANL-FM-Level_1-MnS-4 | This is implemented by fault supervision MnS defined in TS 28.532 [4], provisioning MnS defined in TS 28.532 [4] with 5G Network Resource model defined in TS 28.541 [5], streaming data reporting MnS and File data reporting MnS defined in TS 28.532 [4]. |
|  | REQ-ANL-FM-Level_1-MnS-5 | This can be implemented by utilizing fault supervision MnS (e.g. getAlarmList, notifyNewAlarm) defined in TS 28.532 [4] to obtain the filtered alarm data. |
| Level2 | REQ-ANL-FM-Level_2-MnS-1 | This can be implemented by utilizing generic provisioning MnS defined in TS 28.532 [4] to specify the fault recognition control information. |
|  | REQ-ANL-FM-Level_2-MnS-2 | This can be implemented by utilizing fault supervision MnS (e.g. getAlarmList, notifyNewAlarm) defined in TS 28.532 [4] to obtain the recognized fault information. |
|  | REQ-ANL-FM-Level_2-MnS-3 | This can be implemented by utilizing generic provisioning MnS defined in TS 28.532 [4] to specify the fault demarcation control information. |
|  | REQ-ANL-FM-Level_2-MnS-4 | This can be implemented by Fault supervision MnS defined in TS 28.532 [4] to obtain the fault demarcation information (including fault type and corresponding affected managed object). |
| Level3 | REQ-ANL-FM-Level_3-MnS-1 | This can be implemented by utilizing generic provisioning MnS defined in TS 28.532 [4] to specify the fault root cause analysis control information. |
|  | REQ-ANL-FM-Level_3-MnS-2 | This can be implemented by utilizing generic provisioning MnS defined in 28.532 [4] to specify the fault recovery mechanism analysis control information. |
|  | REQ-ANL-FM-Level_3-MnS-3 | This can be implemented by utilizing generic provisioning MnS defined in 28.532 [4] to specify the fault recovery mechanism decision control information. |
|  | REQ-ANL-FM-Level_3-MnS-4 | This can be implemented by Fault supervision MnS (e.g. getAlarmList, notifyNewAlarm) defined in TS 28.532 [4] to obtain the root cause of the network fault. |
|  | REQ-ANL-FM-Level_3-MnS-5 | This can be implemented by Fault supervision MnS (e.g. getAlarmList, notifyNewAlarm) defined in TS 28.532 [4] to obtain the recommended fault recovery mechanism. |
|  | REQ-ANL-FM-Level_3-MnS-6 | This can be implemented by MDA MnS (i.e. MDAAssistedFaultManagement.FailurePrediction) defined in TS 28.104 [6] to obtain the potential fault prediction information. |

End of Change