3GPP TSG-SA4 Meeting #121	S4- 221518

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.260 | CR | 0004 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TS 26.260 Clause 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATIAS |  |  |  |  |  | Date: |  |  | 2022-16-08 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrects a typo, adds reference to real spherical harmonics and fixes incorrect reference to an Annex within the spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the CR as expected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Text will remain incorrect, reference will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<BEGINNING OF MODIFIED CLAUSE>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	J. Fliege und U. Maier: "A two-stage approach for computing cubature formulae for the sphere," Dortmund University, 1999.

[3]	ISO 3745 - Annex A: "Acoustics - Determination of sound power levels and sound energy levels of noise sources using sound pressure -- Precision methods for anechoic rooms and hemi-anechoic rooms - Annex A: General procedures for qualification of anechoic and hemi-anechoic rooms".

[4]	ISO 1996 Acoustics: "Description, measurement and assessment of environmental noise".

[5]	ANSI S1.4: "Specifications for Sound Level Meters".

[6]	ISO 3: "Preferred numbers – Series of preferred numbers".

[7]	B. Rafaely, “Analysis and design of spherical microphone arrays,” IEEE Transactions on Speech and Audio Processing, no. 13, 2005, pp. 135 – 143

[8]	M. Poletti, “Unified Description of Ambisonics Using Real and Complex Spherical Harmonics,” Ambisonics Symposium 2009, June 25-27, 2009, Graz, Austria.

<<END OF MODIFIED CLAUSE>>

<<BEGINNING OF MODIFIED CLAUSE>>

# 4	Objective Test Methodologies for Immersive Audio Systems

## 4.1	Objective Test Methodologies for Assessment of Immersive Audio Systems in the Sending Direction

### 4.1.1	Diffuse-field Send Frequency Response for Scene-based Audio

#### 4.1.1.1	Introduction

This test is applicable to UEs capturing scene-based audio (e.g. First and Higher Order Ambisonics).

NOTE: Currently, the test method uses a periphonic loudspeaker array for generation of a diffuse-field. Additional loudspeaker setups for the derivation of the diffuse sound field are under consideration.

General test conditions

Free-field propagation conditions

-	The test environment shall contain a free-field volume, wherein free-field sound propagation conditions shall be observed.

-	The free-field sound propagation conditions shall be observed down to a frequency of 200 Hz or less.

-	Qualification of the free-field volume shall be performed using the method and limits for deviation from ideal free-field conditions described in [3].

Test environment noise floor

Within the free-field volume, the equivalent continuous sound level of the test environment in each 1/3rd octave band, Leq(f), shall be less than the limits of the NR10 curve, following the noise rating determination procedures in [4].

#### 4.1.1.2	Definition

The Diffuse-field Send Frequency Response for Scene-based Audio is defined as the transfer function, Gf, between:

Pf, the estimated sound pressure magnitude spectrum obtained from a diffuse-field scene-based audio capture and reference synthesis at the geometric center of a free-field volume; and

a)Pf, the sound pressure magnitude spectrum obtained from a diffuse-field microphone recording the same diffuse field at the origin of a spherical coordinate system.

Figure 1 describes a typical block diagram for the scene-based audio sending direction with measurement points when using a periphonic loudspeaker array.

Figure 1: Scene-based audio capture block diagram for sending direction measurements

Definition of Equivalent Spatial Domain

The equivalent spatial domain representation, w(t), of a Nth order Ambisonics soundfield representation c(t) is obtained by rendering c(t) to K virtual loudspeaker signals wj (t), 1  j  K1≤j≤K, with K = (N+1)2. The respective virtual loudspeaker positions are expressed by means of a spherical coordinate system, where each position lies on the unit sphere, i.e., a radius of 1. Hence, the positions can be equivalently expressed by order-dependent directions j(N)=(j(N),  j(N)), 1  j  K, where j(N) and  j(N) θj(N)ϕj(N)denote the inclinations and azimuths, respectively. These directions are defined according to [2] and reproduced in Annex A for convenience.

The rendering of ct into the equivalent spatial domain can be formulated as a matrix multiplication:

w(t) = ((N,N))-1 c(t),

where ⋅-1()-1 denotes the inversion.

The matrix (N,N) of order N with respect to the order-dependent directions j(N) is defined by:

(N,N) := [S1(N)    S2(N)    …    SK(N)],

with:

Sj(N) := [S00(j(N))    S-1-1(j(N))    S-10(j(N))    S-11(j(N))    S-11(j(N))    …    SNN(j(N))]T ,

where Snm() represents the real valued spherical harmonics of the order n and degree m as defined in [8].

The matrix (N,N) is invertible so that the HOA representation ctc(t) can be converted back from the equivalent spatial domain by:

c(t) = (N,N)·w(t)

<<END OF MODIFIED CLAUSE>>