3GPP TSG-SA4 Meeting #121   	S4-221522

# Toulouse, France, 14 Nov 2022 - 18 Nov 2022	Revision of S4-221270

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.131 | CR | 0086 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Performance requirements of vehicle-mounted hands-free UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HEAD acoustics GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUET |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was noticed that the UE type "vehicle-mounted hands-free" (i.e., network access/SIM card integrated into onboard unit) is rather outdated and performance requirements are only sparsely available. It should be removed from the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description of vehicle-mounted hands-free UE and requirements for this UE type are removed from the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for the outdated and uncommon vehicle-mounted hands-free UE remains part of the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4, 5.2.3, 5.4.3, 5.4.4, 5.7.2, 5.13.4, 6.2.3, 6.4.3, 6.4.4, 6.7.2, 6.13.4, 7.2.3, 7.4.3, 7.4.4, 7.7.2, 7.13.4, 8.2.3, 8.4.3, 8.4.4, 8.7.2, 8.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 26.132 CR 0107 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 26.132: "Speech and video telephony terminal acoustic test specification".

[…]

[20]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".

[21]	ITU-T Recommendation P.1100: "Narrowband hands-free communication in motor vehicles".

[22]	ITU-T Recommendation P.1110: "Wideband hands-free communication in motor vehicles".

[23]	ITU-T Recommendation P.1120: "Super-wideband and fullband stereo hands-free communication in motor vehicles".

# 4	Interface definition

## 4.1	General

The interfaces required to define terminal electro-acoustic characteristics are shown in TS 26.132. These are the air interface and the point of interconnect (POI). The interfaces are shown for one-channel (mono) operation, interfaces for two-channel (stereo) operation are for further study.

Measurements can be made using the system simulator (SS) described in TS 26.132. MTSI speech aspects are specified by TS 26.114 [17].

## 4.2	Air interfaces

The Air Interfaces for GSM, 3G, LTE and NR are specified by GSM 05, 3GPP 45, 3GPP 25, 3GPP 36 and 3GPP 38 series specifications, and the Air Interface for WLAN access to EPC is specified by WLAN access to EPC as defined in TS 23.402 [19] and TS 24.302 [20].

The POI with the public switched telephone network (PSTN) is considered to have a relative level of 0 dBr.

## 4.3	Acoustical interfaces

The following classes of acoustical interface are considered in this specification:

-	Handset UE including softphone UE used as a handset;

-	Headset UE including softphone UE used with headset;

-	Desktop-mounted hands-free UE including softphone UE with external loudspeaker(s) used in hands-free mode;

-	Hand-held hands-free UE including softphone UE with internal loudspeaker(s) used in hands-free mode.

(See definition of softphone in Clause 3.1)

Vehicle Mounted Hands-free UE is out of scope. In case performance evaluations are planned for Vehicle Mounted Hands-free UE, test setup, methods and requirements specified in Recommendation ITU-T P.1100 [21] (for NB), P.1110 [22] (for WB) or P.1120 [23] (for SWB and FB) can be used.

The requirements and performance objectives for a softphone UE shall be derived according to the following rules:

-	When using a softphone UE as a handset: requirements and performance objectives shall correspond to handset mode.

-	When using a softphone UE with headset: requirements and performance objectives shall correspond to headset mode.

-	When using a softphone UE in hands-free mode:

-	When using internal loudspeaker(s), requirements and performance objectives shall correspond to hand-held hands-free.

-	When using external loudspeaker(s), requirements and performance objectives shall correspond to desktop-mounted hands-free.

### 5.2.3	Connections with desktop hands-free UE

The nominal values of SLR/RLR to/from the POI shall be:

SLR = 13 ± 4 dB;

RLR = 5 ± 4 dB.

Where a user-controlled volume control is provided, the RLR shall meet the nominal value for at least one setting of the control. It is recommended that a volume control giving at least 15 dB increase from the nominal RLR (louder) is provided for hands-free units. This is to allow for the increased acoustic noise level in the usage environment.

RLR at the maximum volume control setting should be ≤ (equal or louder than) 1 dB.

Compliance shall be checked by the relevant tests described in TS 26.132.

NOTE:	The target value for nominal RLR, as recommended in ITU-T G.111 Annex B – Table B.1 [4], lies between 1 and 3 dB. The higher RLR requirement of 5 dB for desktop hands-free is appreciative of the limitations in transducer output with current typical form factors.

### 5.4.3	Desktop hands-free UE sending

The sending sensitivity frequency response from the MRP to the SS audio output (digital output of the reference speech decoder of the SS) shall be as follows:

The sending sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 3 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 3: Desktop hands-free sending sensitivity/frequency mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | -12 |  |
| 200 | 0 |  |
| 300 | 0 | -12 |
| 1 000 | 0 | -6 |
| 2 000 | 4 | -6 |
| 3 000 | 4 | -6 |
| 3 400 | 4 | -9 |
| 4 000 | 0 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Figure 3: Desktop hands-free sending sensitivity/frequency mask

Compliance shall be checked by the relevant test described in TS 26.132.

### 5.4.4	Desktop hands-free UE receiving

The receiving sensitivity frequency response from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the free-field shall be as follows:

The receiving sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 4 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 4: Desktop hands-free receiving sensitivity/frequency mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 200 | 6 |  |
| 315 | 6 | -9 |
| 400 | 6 | -6 |
| 3 100 | 6 | -6 |
| 4 000 | 6 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Figure 4: Desktop receiving sensitivity/frequency mask

Compliance shall be checked by the relevant test described in TS 26.132.

### 5.7.2	Acoustic echo control in desktop hands-free UE

The TCLw for the desktop hands-free UE shall be ≥ 40 dB for any setting of the volume control.

The TCLw for the desktop hands-free UE shall be ≥ 46 dB when measured under free-field conditions at the nominal setting of the volume control.

NOTE:	A TCLw for the desktop hands-free UE of ≥ 55 dB is recommended as a performance objective when measured under free-field conditions at the nominal setting of the volume control. Depending on the UE idle channel noise in the sending direction, it may not always be possible to measure an echo loss ≥ 55 dB.

The echo canceller should be designed to cope with the expected reverberation and dispersion. In the case of the hands-free UE, this reverberation and dispersion may be time variant. Compliance with this requirement shall be checked by the relevant test described in TS 26.132.

### 5.13.4	Desktop hands-free

Requirements are for further study.

### 6.2.3	Connections with desktop hands-free UE

The nominal values of SLR/RLR to/from the POI shall be:

SLR = 13 ± 4 dB;

RLR = 5 ± 4 dB (for desktop hands-free UE).

Where a user-controlled volume control is provided, the RLR shall meet the nominal value for at least one setting of the control. It is recommended that a volume control giving at least 15 dB increase from the nominal RLR (louder) is provided for hands-free units. This is to allow for increased acoustic noise level in the usage environment.

RLR at the maximum volume control setting should ≤ (equal or louder than) 1 dB.

Compliance shall be checked by the relevant tests described in TS 26.132.

NOTE:	The target value for nominal RLR, as recommended in ITU-T G.111 Annex B – Table B.1 [4], lies between 1 and 3 dB. The higher RLR requirement of 5 dB for desktop hands-free is appreciative of the limitations in transducer output with current typical form factors.

### 6.4.3	Desktop hands-free UE sending

The sending sensitivity frequency response from the MRP to the SS audio output (digital output of the reference speech decoder of the SS) shall be as follows:

The sending sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 11 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 11: Desktop hands-free sending sensitivity/frequency mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 0 |  |
| 200 | 5 | -5 |
| 5 000 | 5 | -5 |
| 6 300 | 5 | -10 |
| 8 000 | 5 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Figure 11: Desktop hands-free sending sensitivity/frequency mask

Compliance shall be checked by the relevant test described in TS 26.132.

### 6.4.4	Desktop hands-free UE receiving

The receiving sensitivity frequency response from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the free-field shall be as follows:

The receiving sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 12 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 12: Desktop hands-free receiving sensitivity/frequency mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 125 | 8 |  |
| 200 | 8 | -12 |
| 250 | 8 | -9 |
| 315 | 7 | -6 |
| 400 | 6 | -6 |
| 5 000 | 6 | -6 |
| 6 300 Hz | 6 | -9 |
| 8 000 Hz | 6 | - |
| NOTE: The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) - logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

Figure 12: Desktop hands-free receiving sensitivity/frequency mask

It is recommended as a performance objective that the receiving sensitivity frequency response be within the mask which can be drawn with straight lines between the breaking points in table 12.a on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 12a: Performance objective for desktop hands-free receiving sensitivity/frequency response

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | 0 |  |
| 200 | 0 | -18 |
| 250 | 0 | -15 |
| 315 | 0 | -12 |
| 6 300 | 0 | -12 |
| 8 000 | 0 |  |

Figure 12a: Performance objective for desktop hands-free receiving sensitivity/frequency response

Compliance shall be checked by the relevant test described in TS 26.132.

### 6.7.2	Acoustic echo control in desktop hands-free UE

The TCLw for the desktop hands-free UE shall be ≥ 40 dB for any setting of the volume control.

The TCLw for the desktop hands-free UE shall be ≥ 46 dB when measured under free-field conditions at the nominal setting of the volume control.

NOTE:	A TCLw for desktop hands-free UE of ≥ 55 dB is recommended as a performance objective when measured under free-field conditions at the nominal setting of the volume control. Depending on the UE idle channel noise in the sending direction, it may not always be possible to measure an echo loss ≥ 55 dB.

The echo canceller should be designed to cope with the expected reverberation and dispersion. In the case of the hands-free UE, this reverberation and dispersion may be time variant. Compliance with this requirement shall be checked by the relevant test described in TS 26.132.

### 6.12.4	Desktop hands-free

Requirements are for further study.

### 7.2.3	Connections with desktop hands-free UE

The nominal values of SLR/RLR to/from the POI shall be:

SLR = 13 ± 4 dB;

RLR = 5 ± 4 dB

Where a user-controlled volume control is provided, the RLR shall meet the nominal value for at least one setting of the control. It is recommended that a volume control giving at least 15 dB increase from the nominal RLR (louder) is provided for hands-free units. This is to allow for increased acoustic noise level in the usage environment.

RLR at the maximum volume control setting should ≤ (equal or louder than) 1 dB.

Compliance shall be checked by the relevant tests described in TS 26.132.

NOTE:	The target value for nominal RLR, as recommended in ITU-T G.111 Annex B – Table B.1 [4], lies between 1 and 3 dB. The higher RLR requirement of 5 dB for desktop hands-free is appreciative of the limitations in transducer output with current typical form factors.

### 7.4.3	Desktop hands-free UE sending

The sending sensitivity frequency response from the MRP to the SS audio output (digital output of the reference speech decoder of the SS) shall be as follows:

The sending sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 23 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 23: Desktop hands-free sending sensitivity/frequency mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | [3...5] |  |
| 200 | [3...5] | [-3...-5] |
| 5000 | [3...5] | [-3...-5] |
| 12500 | [3...5] | [-5...-7] |
| 16000 | [3...5] |  |
| NOTE 1: All sensitivity values are expressed in dB on an arbitrary scale. NOTE 2: Values within [] are provisional and expected to be defined as single values based on future studies. |  |  |

It is recommended as a performance objective that the sending sensitivity/frequency response be within the mask which can be drawn with straight lines between the breaking points in table 24.

Table 24: Desktop hands-free  sending sensitivity/frequency performance objective mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | [3] |  |
| 200 | [3] | [-3] |
| 5000 | [3] | [-3] |
| 12500 | [3] | [-5] |
| 16000 | [3] |  |
| NOTE 1: All sensitivity values are expressed in dB on an arbitrary scale. NOTE 2: Values within [] are provisional and expected to be confirmed, revised or removed based on future studies. |  |  |

TBD

Figure 18: Desktop hands-free sending sensitivity/frequency masks

Compliance shall be checked by the relevant test described in TS 26.132.

### 7.4.4	Desktop hands-free UE receiving

The receiving sensitivity frequency response from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the free-field shall be as follows:

The receiving sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 25 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 25: Desktop and vehicle-mounted hands-free receiving sensitivity/frequency mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| TBD | TBD | TBD |
| NOTE: The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) - logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

It is recommended as a performance objective that the receiving sensitivity frequency response be within the mask which can be drawn with straight lines between the breaking points in table 26 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 26: Performance objective for desktop hands-free receiving sensitivity/frequency response

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| TBD | TBD | TBD |

TBD

Figure 19: Desktop hands-free receiving sensitivity/frequency masks

Compliance shall be checked by the relevant test described in TS 26.132.

### 7.7.2	Acoustic echo control in desktop hands-free UE

The TCLw for the desktop and vehicle-mounted hands-free UE shall be ≥ 40 dB for any setting of the volume control.

The TCLw for the desktop hands-free and vehicle-mounted hands-free UE shall be ≥ 46 dB when measured under free-field conditions at the nominal setting of the volume control.

NOTE:	A TCLw for desktop hands-free and vehicle-mounted hands-free UE of ≥ 55 dB is recommended as a performance objective when measured under free-field conditions at the nominal setting of the volume control. Depending on the UE idle channel noise in the sending direction, it may not always be possible to measure an echo loss ≥ 55 dB.

The echo canceller should be designed to cope with the expected reverberation and dispersion. In the case of the hands-free UE, this reverberation and dispersion may be time variant. Compliance with this requirement shall be checked by the relevant test described in TS 26.132.

### 7.12.4	Desktop hands-free

Requirements are for further study.

### 8.2.3	Connections with desktop hands-free UE

See requirements for super-wideband.

Compliance shall be checked by the relevant tests described in TS 26.132.

### 8.4.3	Desktop hands-free UE sending

See requirements for super-wideband.

Compliance shall be checked by the relevant test described in TS 26.132.

### 8.4.4	Desktop hands-free UE receiving

See requirements for super-wideband.

Compliance shall be checked by the relevant test described in TS 26.132.

### 8.7.2	Acoustic echo control in desktop hands-free UE

See requirements for super-wideband.

Compliance with this requirement shall be checked by the relevant test described in TS 26.132.

### 8.12.4	Desktop hands-free

Requirements are for further study.

- End of changes