3GPP TSG-SA4 Meeting #121	S4-221514

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022	revision of S4-221271

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.131 | CR | 0087 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for idle noise and sidetone requirements (electrical interface UE) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HEAD acoustics GmbH, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HInT |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clauses 5.3.4, 6.3.4 and 7.3.4 define performance requirements for idle noise in receiving for electrical interface UE and for different volume settings. But for digital electrical interface UE, different volume control settings are not applicable according to the test setup specified in clause 5.1.6.2 of TS 26.132. In the heading of clause 7.5 (sidetone for SWB), analog electrical interface UE is missing in the list of applicable UE types. Headings of clauses 5.5, 6.5 and 8.5 (NB, WB and FB) include analog electrical interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Removal of performance requirements for digital electrical interface UE. 2) Adding analog electrical interface UE to the heading of clause 7.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The idle noise performance requirements for digital electrical interface UE contradict the specified test setup and remain confusing to the reader. The heading of clause 7.5 might be interpreted in a way that sidetone does not have a performance requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.4, 5.5, 6.3.4, 6.5, 7.3.4, 7.5, 8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.4	Receiving (electrical interface UE)

The maximum A-weighted noise level when no signal is applied to the input of the SS shall be as follows:

If no user-controlled receiving volume control is provided, or, if it is provided, at the setting of the user-controlled receiving volume control at which the RJLR is equal to the nominal value, the noise measured at the receive output of the electrical reference interface contributed by the receiving equipment alone shall be ≤ -87 dBV(A) for analogue and -64 dBm0(A) for digital connections.

Where a volume control is provided, the measured noise shall be ≤ -84 dBV(A) for analogue -connection at the maximum setting of the volume control.

For the nominal volume control setting, the level of single frequency disturbances should be ≤ -90 dBV(A) for analogue and -67 dBm0(A) for digital connections in the frequency range from 100 Hz to 10 kHz. As a performance objective it is recommended that the level should be ≤ -94 dBV(A) for analogue and -71 dBm0(A) for digital connections.

NOTE:	In a connection with the PSTN, noise conditions as described in ITU-T Recommendation G.103 [3] can be expected at the input (POI) of the 3G, LTE, NR or WLAN network. The characteristics of this noise may be influenced by the speech transcoding process (for further study).

Compliance shall be checked by the relevant test described in 3GPP TS 26.132.

## 5.5	Sidetone characteristics (handset, headset, and analogue electrical interface UE)

[…]

### 6.3.4	Receiving (electrical interface UE)

The maximum A-weighted noise level when no signal is applied to the input of the SS shall be as follows:

If no user-controlled receiving volume control is provided, or, if it is provided, at the setting of the user-controlled receiving volume control at which the RJLR is equal to the nominal value, the noise measured at the receive output of the electrical reference interface contributed by the receiving equipment alone shall be ≤ -87 dBV(A) for analogue and -64 dBm0(A) for digital connections.

Where a volume control is provided, the measured noise shall be ≤ -84 dBV(A) for analogue -connection at the maximum setting of the volume control.

For the nominal volume control setting, the level of single frequency disturbances should be ≤ -90 dBV(A) for analogue and -67 dBm0(A) for digital connections in the frequency range from 100 Hz to 10 kHz. As a performance objective it is recommended that the level should be ≤ -94 dBV(A) for analogue and -71 dBm0(A) for digital connections.

NOTE:	In a connection with the PSTN, noise conditions as described in ITU-T Recommendation G.103 [3] can be expected at the input (POI) of the 3G, LTE, NR or WLAN network. The characteristics of this noise may be influenced by the speech transcoding process (for further study).

Compliance shall be checked by the relevant test described in 3GPP TS 26.132.

## 6.5	Sidetone characteristics (handset, headset, and analogue electrical interface UE)

[…]

### 7.3.4	Receiving (electrical interface UE)

The maximum A-weighted noise level when no signal is applied to the input of the SS shall be as follows:

-	If no user-controlled receiving volume control is provided, or, if it is provided, at the setting of the user-controlled receiving volume control at which the RJLR is equal to the nominal value, the noise measured at the receive output of the electrical reference interface contributed by the receiving equipment alone shall not exceed -87 dBV(A) for analogue and -64 dBm0(A) for digital connections.

-	Where a volume control is provided, the measured noise shall be ≤ -84 dBV(A) for analogue -connection at the maximum setting of the volume control.

For the nominal volume control setting, the level of single frequency disturbances shall be ≤ -90 dBV(A) for analogue and -67 dBm0(A) for digital connections in the frequency range from 100 Hz to 16 kHz. As a performance objective it is recommended that the level should be ≤ -94 dBV(A) for analogue and -71 dBm0(A) for digital connections.

NOTE:	In a connection with the PSTN, noise conditions as described in ITU-T Recommendation G.103 [3] can be expected at the input (POI) of the 3G, LTE, NR or WLAN network. The characteristics of this noise may be influenced by the speech transcoding process (for further study).

Compliance shall be checked by the relevant test described in TS 26.132.

## 7.5	Sidetone characteristics (handset, headset, and analogue electrical interface UE)

[…]

## 8.5	Sidetone characteristics (handset, headset, and analogue electrical interface UE)

[…]