TSG SA Meeting #SP-98-e	SP-221058

13 - 19 December 2022, Electronic meeting

3GPP TSG-SA4 Meeting #121	S4- 221518

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.260 | CR | 0004 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TS 26.260 Clause 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATIAS |  |  |  |  |  | Date: |  |  | 2022-16-08 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrects a typo, adds reference to real spherical harmonics and fixes incorrect reference to an Annex within the spec. The CR rev1 was agreed by SA4, but it is revised to rev2 to correct the Release in 3GU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the CR as expected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Text will remain incorrect, reference will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<BEGINNING OF MODIFIED CLAUSE>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	J. Fliege und U. Maier: "A two-stage approach for computing cubature formulae for the sphere," Dortmund University, 1999.

[3]	ISO 3745 - Annex A: "Acoustics - Determination of sound power levels and sound energy levels of noise sources using sound pressure -- Precision methods for anechoic rooms and hemi-anechoic rooms - Annex A: General procedures for qualification of anechoic and hemi-anechoic rooms".

[4]	ISO 1996 Acoustics: "Description, measurement and assessment of environmental noise".

[5]	ANSI S1.4: "Specifications for Sound Level Meters".

[6]	ISO 3: "Preferred numbers – Series of preferred numbers".

[7]	B. Rafaely, “Analysis and design of spherical microphone arrays,” IEEE Transactions on Speech and Audio Processing, no. 13, 2005, pp. 135 – 143

[8]	M. Poletti, “Unified Description of Ambisonics Using Real and Complex Spherical Harmonics,” Ambisonics Symposium 2009, June 25-27, 2009, Graz, Austria.

<<END OF MODIFIED CLAUSE>>

<<BEGINNING OF MODIFIED CLAUSE>>

# 4	Objective Test Methodologies for Immersive Audio Systems

## 4.1	Objective Test Methodologies for Assessment of Immersive Audio Systems in the Sending Direction

### 4.1.1	Diffuse-field Send Frequency Response for Scene-based Audio

#### 4.1.1.1	Introduction

This test is applicable to UEs capturing scene-based audio (e.g. First and Higher Order Ambisonics).

NOTE: Currently, the test method uses a periphonic loudspeaker array for generation of a diffuse-field. Additional loudspeaker setups for the derivation of the diffuse sound field are under consideration.

General test conditions

Free-field propagation conditions

-	The test environment shall contain a free-field volume, wherein free-field sound propagation conditions shall be observed.

-	The free-field sound propagation conditions shall be observed down to a frequency of 200 Hz or less.

-	Qualification of the free-field volume shall be performed using the method and limits for deviation from ideal free-field conditions described in [3].

Test environment noise floor

Within the free-field volume, the equivalent continuous sound level of the test environment in each 1/3rd octave band, Leq(f), shall be less than the limits of the NR10 curve, following the noise rating determination procedures in [4].

#### 4.1.1.2	Definition

The Diffuse-field Send Frequency Response for Scene-based Audio is defined as the transfer function, Gf, between:

Pf, the estimated sound pressure magnitude spectrum obtained from a diffuse-field scene-based audio capture and reference synthesis at the geometric center of a free-field volume; and

a)Pf, the sound pressure magnitude spectrum obtained from a diffuse-field microphone recording the same diffuse field at the origin of a spherical coordinate system.

Figure 1 describes a typical block diagram for the scene-based audio sending direction with measurement points when using a periphonic loudspeaker array.

Figure 1: Scene-based audio capture block diagram for sending direction measurements

Definition of Equivalent Spatial Domain

The equivalent spatial domain representation, w(t), of a Nth order Ambisonics soundfield representation c(t) is obtained by rendering c(t) to K virtual loudspeaker signals wj (t), 1  j  K1≤j≤K, with K = (N+1)2. The respective virtual loudspeaker positions are expressed by means of a spherical coordinate system, where each position lies on the unit sphere, i.e., a radius of 1. Hence, the positions can be equivalently expressed by order-dependent directions j(N)=(j(N),  j(N)), 1  j  K, where j(N) and  j(N) θj(N)ϕj(N)denote the inclinations and azimuths, respectively. These directions are defined according to [2] and reproduced in Annex A for convenience.

The rendering of ct into the equivalent spatial domain can be formulated as a matrix multiplication:

w(t) = ((N,N))-1 c(t),

where ⋅-1()-1 denotes the inversion.

The matrix (N,N) of order N with respect to the order-dependent directions j(N) is defined by:

(N,N) := [S1(N)    S2(N)    …    SK(N)],

with:

Sj(N) := [S00(j(N))    S-1-1(j(N))    S-10(j(N))    S-11(j(N))    S-11(j(N))    …    SNN(j(N))]T ,

where Snm() represents the real valued spherical harmonics of the order n and degree m as defined in [8].

The matrix (N,N) is invertible so that the HOA representation ctc(t) can be converted back from the equivalent spatial domain by:

c(t) = (N,N)·w(t)

<<END OF MODIFIED CLAUSE>>