3GPP TSG-SA WG2 Meeting #154	S2-2211182

# Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0554 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 23.288 Enhancement to Support AI/ML Data Transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In study phase, it is concluded for KI#5 and more specifically for Solution #10 that the PCF uses the extended Analytics of NWDAF to derive the candidate time window(s) that may fulfil the requirements requested from AF. This CR proposes a set of changes in TS 23.288 to provide the extended analytics, concluded in the study phase, as follows: - Overall Average and Maximum Packet Delay for UL/DL per UE per time window; - Overall Average and Maximum Packet Loss Rate for UL/DL per UE per time window; - Overall Average, Minimum and Maximum Traffic rate for UL/DL per UE per time window. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Clause 6.14 DN Performance Analystics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PCF is not aware of DN performance in terms of the Average/Maximum packet delay/packet loss/traffic rate, and consequently cannot derive candidate times windows that may fulfil AF request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.14.2, 6.14.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Expected WID is AIMLsys |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

### 6.14.2	Input Data

The data collected from the AF are defined in table 6.14.2-1

Table 6.14.2-1: Performance Data from AF

| Information | Source | Description |
| --- | --- | --- |
| UE identifier | AF | IP address of the UE at the time the measurements was made. |
| UE location | AF | The location of the UE when the performance measurement was made. |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service). |
| IP filter information | AF | Identify a service flow of the UE for the application. |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Application Server Instance address | AF/NEF | The IP address/FQDN of the Application Server that the UE had a communication session when the measurement was made. |
| UL/DL Performance Data | AF | The performance associated with the communication session of the UE with an Application Server that includes: Average/Maximum Packet Delay, Average/Maximum Loss Rate and Average/Minimum/Maximum Throughput. |
| Timestamp | AF | A time stamp associated to the Performance Data provided by the AF. |

The data collected by the SMF are described in table 6.4.2-2.

The NWDAF subscribes to network data as defined in clause 6.4.2.

Data may be collected from OAM as described in table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

The Event Filters for the service data collection from SMF, AMF and AF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

***** Next Change *****

### 6.14.3	Output Analytics

The DN performance analytics is shown in table 6.14.3-1 and table 6.14.3-2.

Table 6.14.3-1: DN service performance statistics

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See note 1. |
| DNN | Identifies the data network name (e.g. "internet") for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performances for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF info | The UPF ID/address/FQDN information for the involved anchor UPF. See NOTE 1. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance | Performance indicators. |
| >> Average Traffic rate (NOTE 2) | Average traffic rate observed for UEs communicating with the application. |
| >> Maximum Traffic rate (NOTE 2) | Maximum traffic rate observed for UEs communicating with the application. |
| >> Minimum Traffic rate (NOTE 3) | Minimum traffic rate observed for UEs communicating with the application. |
| >> Average Packet Delay (NOTE 2) | Average packet delay observed for UEs communicating with the application. |
| >> Maximum Packet Delay (NOTE 2) | Maximum packet delay for observed for UEs communicating with the application. |
| >> Average Packet Loss Rate (NOTE 2) | Average packet loss observed for UEs communicating with the application. |
| >> Maximum Packet Loss Rate (NOTE 2) | Maximum packet loss observed for UEs communicating with the application. |
| > Spatial Validity Condition | Area where the DN performance analytics applies. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. |
| NOTE 1: The item "DNN", "S-NSSAI" and "Serving anchor UPF info" shall not be included if the consumer NF is an untrusted AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 3: Minimum traffic rate measurements are only derived from active traffic. |  |

Table 6.14.3-2: DN service performance predictions

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See NOTE 1. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performance for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF info | The UPF ID/address/FQDN information for the involved anchor UPF. See NOTE 1. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance | Performance indicators |
| >> Average Traffic rate (NOTE 2) | Average traffic rate predicted for UEs communicating with the application. |
| >> Maximum Traffic rate (NOTE 2) | Maximum traffic rate predicted for UEs communicating with the application. |
| >> Minimum Traffic rate (NOTE 3) | Minimum traffic rate predicted for UEs communicating with the application. |
| >> Average Packet Delay (NOTE 2) | Average packet delay predicted for UEs communicating with the application. |
| >> Maximum Packet Delay (NOTE 2) | Maximum packet delay for predicted for UEs communicating with the application. |
| >> Average Packet Loss Rate (NOTE 2) | Average packet loss predicted for UEs communicating with the application. |
| >> Maximum Packet Loss Rate (NOTE 2) | Maximum packet loss predicted for UEs communicating with the application. |
| > Spatial Validity Condition | Area where the DN performance analytics applies. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: The item "DNN", "S-NSSAI" and "Serving anchor UPF info" shall not be included if the consumer is an untrusted AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 3: Minimum traffic rate measurements are only derived from active traffic. |  |

***** End of Change *****