3GPP TSG-SA3 Meeting #109	S3-223891

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0197 | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MCSec] 33180 R14 Incorrect example (Mirror) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCSec |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect examples are used for the scope of an access token. The scope of an access token should be a space-delimited list, but in the examples it uses an array. In section B.2.2.2, the scope is defined as a “JSON string containing a space-separated list of the MCX authorization scopes associated with this token.”, while in the examples given in sections B.4.2.5, B.5.3 and B.7.5, an array is used for the scope of the access token. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of access token examples given in B.4.2.5, B.5.3 and B.7.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect example used for the access token. Incoherence between text and examples. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.4.2.5, B.5.3, B.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************** START of 1st CHANGE *****************************

### B.4.2.5	Access token response

If the access token request is valid and authorized, the IdM server returns an ID token, access token and refresh token to the MCX client in an access token response message; otherwise it will return an error.

The access token response standard parameters are shown in table B.4.2.5-1.

Table B.4.2.5-1: Access token response standard parameters

| Parameter | Values |
| --- | --- |
| access_token | REQUIRED. This is the issued access token. |
| token_type | REQUIRED. This field shall be “bearer” |
| expires_in | REQUIRED. The lifetime in seconds of the access token. |
| Id_token | OPTIONAL. This is the issued id token. |
| Refresh_token | OPTIONAL. This is the issued refresh token. |

An example of a successful response might look like:

EXAMPLE:

HTTP/1.1 200 OK

Content-Type: application/json;charset=UTF-8

Cache-Control: no-store

Pragma: no-cache

{
   "access_token":"eyJhbGciOiJSUzI1NiJ9.eyJtY3B0dF9pZCI6ImFsaWNlQG9yZy5jb20iLCJleHAiOjE0NTM1MDYxMjEsInNjb3BlIjoib3BlbmlkIDNncHA6bWNwdHQ6cHR0X3NlcnZlciIsImNsaWVudF9pZCI6Im1jcHR0X2NsaWVudCJ9.yXGl0gu3SsRQ_VUrDTWuEp-X8wrj-xUzXzWhTnqntXn0fI6xLR0i6VHtY3L25w81u260bsOFUMiYLLBC0LOKi-SOcwMqrzH6_BZwjkRhExiAciHZNtwgIlHWYwpCuxQII0dcg7_bw_YAFEQQC9IDzsifPXl9JmazI8YMtGcAWohoiSi0KY1pnbORUDBSPLLFlekQH5aOWbe_6EF4pDbm5pH8GXLd_ZtxS7jC6tAAMsCvsRy7Pb_GeDr_jT8-IevGKWO82of7gaUQkF8qnKVagr4-qc2FJeSDuhj4ZvfL510cgcRGy4NJ_7xxS10bzFNLavfqOlgOelJv0KA9IAd-1Q",

"refresh_token":"Y7NSzUJuS0Jp7G4SKpBKSOJVHIZxFbxqsqCIZhOEk9",

"id_token":"eyJhbGciOiJSUzI1NiJ9.eyJzdWIiOiIxMjM0NTY3ODkwIiwiYXVkIjoibWNwdHRfY2xpZW50IiwiaXNzIjoiSWRNUy5zZXJ2ZXIuY29tOjkwMzEiLCJpYXQiOjE0NTM0OTgxNTgsImV4cCI6MTQ1MzQ5ODQ1OCwibWNwdHRfaWQiOiJhbGljZUBvcmcuY29tIn0.Dpn7AhIMaqMEgg12NYUUfJGSFJMPG8M2li9FLtPotDlHvwU2emBws8z5JLw81SXQnoLqZ8ZF8tIhZ1W7uuMbufF4Wsr7PAadZixz3CnV2wxFV9qR_VA1-0ccDTPukUsRHsic0SgZ3aIbcYKd6VsehFe_GDwfqysYzD7yPwCfPZo",
"token_type": "Bearer",
"expires_in": 7199
}

The MCX client may now validate the user with the ID token and configure itself for the user (e.g. by extracting the MC service ID from the ID Token). The MCX client then uses the access token to make authorized requests to the MCX resource servers (MCPTT server, MCVideo server, MCData server, KMS, etc.) on behalf of the end user.

************************** END of 1st CHANGE *****************************

************************** START of 2nd CHANGE *****************************

## B.5.3	Access token response

In response to the access token request (above) the token endpoint on the IdM server will return an access token to the MCX client, and optionally another refresh token in an access token response message.

The access token response standard parameters are shown in table B.5.3-1.

Table B.5.3-1: Access token response standard parameters

| Parameter | Values |
| --- | --- |
| access_token | REQUIRED. This is the issued access token. |
| token_type | REQUIRED. This field shall be “bearer” |
| expires_in | REQUIRED. The lifetime in seconds of the access token. |
| Id_token | OPTIONAL. This is the issued id token. |
| Refresh_token | OPTIONAL. This is the issued refresh token. |

An example of a successful response for MCX Connect might look like:

EXAMPLE:

HTTP/1.1 200 OK

Content-Type: application/json;charset=UTF-8

Cache-Control: no-store

Pragma: no-cache

{

"access_token":"eyJhbGciOiJSUzI1NiJ9.eyJtY3B0dF9pZCI6ImFsaWNlQG9yZy5jb20iLCJleHAiOjE0NTM1MDYxMjEsInNjb3BlIjoib3BlbmlkIDNncHA6bWNwdHQ6cHR0X3NlcnZlciIsImNsaWVudF9pZCI6Im1jcHR0X2NsaWVudCJ9.yXGl0gu3SsRQ_VUrDTWuEp-X8wrj-xUzXzWhTnqntXn0fI6xLR0i6VHtY3L25w81u260bsOFUMiYLLBC0LOKi-SOcwMqrzH6_BZwjkRhExiAciHZNtwgIlHWYwpCuxQII0dcg7_bw_YAFEQQC9IDzsifPXl9JmazI8YMtGcAWohoiSi0KY1pnbORUDBSPLLFlekQH5aOWbe_6EF4pDbm5pH8GXLd_ZtxS7jC6tAAMsCvsRy7Pb_GeDr_jT8-IevGKWO82of7gaUQkF8qnKVagr4-qc2FJeSDuhj4ZvfL510cgcRGy4NJ_7xxS10bzFNLavfqOlgOelJv0KA9IAd-1Q",
   "refresh_token": "iTxQYALqlc7uLyFGpnl8tR8Y9gkw91mFy2qC9Yywkz",
   "token_type": "Bearer",
   "expires_in": 7199
}

It is possible to configure the IdM server to confirm that the user account is still valid each time the refresh token is presented, and to revoke the refresh token if not. This security practice is RECOMMENDED.

************************** END of 2nd CHANGE *****************************

************************** START of 3rd CHANGE *****************************

## B.7.5	Token Response

If the token request is valid and authorized, the partner IdM server returns an access token to the MCX client specific to the user for the partner services and optionally a refresh token in a token response message; otherwise, it will return an error.

The token response standard parameters are shown in table B.7.5-1.

Table B.7.5-1: Token response standard parameters

| Parameter | Values |
| --- | --- |
| access_token | REQUIRED. This is the issued access token. |
| token_type | REQUIRED. This field shall be “bearer” |
| expires_in | REQUIRED. The lifetime in seconds of the access token. |
| Id_token | OPTIONAL. This is the issued id token. |
| Refresh_token | OPTIONAL. This is the issued refresh token. |

An example of a successful response might look like:

EXAMPLE:

HTTP/1.1 200 OK

Content-Type: application/json;charset=UTF-8

Cache-Control: no-store

Pragma: no-cache

{

"access_token":"eyJhbGciOiJSUzI1NiJ9.eyJtY3B0dF9pZCI6ImFsaWNlQG9yZy5jb20iLCJleHAiOjE0NTM1MDYxMjEsInNjb3BlIjoib3BlbmlkIDNncHA6bWNwdHQ6cHR0X3NlcnZlciIsImNsaWVudF9pZCI6Im1jcHR0X2NsaWVudCJ9.yXGl0gu3SsRQ_VUrDTWuEp-X8wrj-xUzXzWhTnqntXn0fI6xLR0i6VHtY3L25w81u260bsOFUMiYLLBC0LOKi-SOcwMqrzH6_BZwjkRhExiAciHZNtwgIlHWYwpCuxQII0dcg7_bw_YAFEQQC9IDzsifPXl9JmazI8YMtGcAWohoiSi0KY1pnbORUDBSPLLFlekQH5aOWbe_6EF4pDbm5pH8GXLd_ZtxS7jC6tAAMsCvsRy7Pb_GeDr_jT8-IevGKWO82of7gaUQkF8qnKVagr4-qc2FJeSDuhj4ZvfL510cgcRGy4NJ_7xxS10bzFNLavfqOlgOelJv0KA9IAd-1Q",
 "refresh_token": "iTxQYALqlc7uLyFGpnl8tR8Y9gkw91mFy2qC9Yywkz",
 "token_type": "Bearer",
 "expires_in": 7199
}

The MCX client then uses the access token to make authorized requests to the partner MCX resource servers (MCPTT group management service, MCVideo group management service, MCData group management service, etc) on behalf of the end user.

************************** END of 3rd CHANGE *****************************