TSG SA Meeting #SP-98-e	SP-221160

13 - 19 December 2022, Electronic meeting

# Source:	SA WG5

# Title:	New Study on CHF Segmentation

# Document for:	Approval

Agenda Item:	6.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on CHF Segmentation

Acronym: FS_CHFSeg

Unique identifier:	980026

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

# 3	Justification

This study item aims to study how to enhance CHF discovery and selection.

Currently SUPI is used for NRF based discovery of CHF. The CHF discovery mechanism does not support discovery of a CHF by the NF consumers like the SMF, the AMF, the SMSF and the PCF based on information about network segments that the CHF belongs to. For example:

- In a network deployment it may be possible that group(s) of CHF(s) is deployed for supporting charging of enterprise customers.

- It may be also possible to deploy group(s) of CHF(s) to support charging for customers belonging to specific branches of the operator.

The SUPI based CHF discovery through NRF is not efficient in such network deployments and a Group ID based CHF discovery is needed. Although in 3GPP Rel-17 the CHF can register with the NRF using CHF-Group-ID (along other parameters such as CHF-Hostname, IP Address etc.) and an NF consumers can use CHF-Group-ID during CHF discovery process through the NRF, the current specification is not sufficient to support the above use cases because:

- The NF consumers like the SMF, the AMF, the SMSF and the PCF does not have CHF-Group-ID information associated with UE.

- Group ID Mapping stored in UDR is not accessible to all NF consumers (e.g. AMF, SMF etc.). This also requires the UDR to store/maintain the mapping of all the subscribers in the network to the CHF-Group-ID

The AM and SM subscription data in the UDM includes a "Charging Characteristics" information that may also contain CHF address(es) selection information; however, the encoding is not standardized and is freely defined by Operators.

Additional CHF selection data providing information on CHF segmentation or grouping in the deployment will benefit the NF consumers by allowing them to perform CHF discovery based on segmentation. One enhancement that is required is to extend the subscription data (e.g. policy control subscription information in UDR or UE's subscription data in the UDM) to include the CHF-Group-ID information in order to enable the NF consumers like the SMF, the AMF, the SMSF and the PCF to perform CHF discovery and selection using this CHF-Group-ID. The subscription data extension in UDM/UDR should support to further extend the CHF selection information with additional data that may be required in future.

TS 32.290 and TS 29.510 contains the definition of CHF Group ID for NF consumers which is not sufficient for CHF segmentation, and it needs to be further evaluated.

# 4	Objective

The objective of the study is to:

- Identify new charging scenarios and requirements which makes use of CHF Segmentation.

- study the potential charging solutions which make use of CHF Segmentation.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | TR 28.xxx | Study on CHF Segmentation | TSG SA#99 (Mar 2023) | TSG SA#101 (Sep 2023) | João Rodrigues, Nokia, joao.a.rodrigues@nokia.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

João Rodrigues, Nokia, joao.a.rodrigues@nokia.com

# 7	Work item leadership

SA5

# 8	Aspects that involve other WGs

The following aspects are potentially analysed by SA2:

- SDM resource structure enhancement

- Implications on PCF, SMF, AMF, SMSF and NEF

# 9	Supporting Individual Members

Supporting IM name

Amdocs

China Mobile

Ericsson LM

Huawei

MATRIXX Software

Nokia

Nokia Shanghai Bell

Verizon UK Ltd