3GPP TSG-SA5 Meeting #146	S5-226664

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0153 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.531 deleting allocate network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute India |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The use case defined in clause 5.1.19 require a mechanism to create a 3GPP sub-network. The use case does not mention slicing at all. Considering the UC and the requirements, the allocate network functionality in clasue 6.4 and 6.5 has the following defects. - The AllocateNetwork operation defines attributeListIn as “This parameter specifies the network related requirements defined in ServiceProfile in Clause 6.3.3 in TS 28.541 [6].”. The ServiceProfile represents the requirement for a Network Slice not for a network/sub-network. - The AllocateNetwork operations Ouput includes serviceProfileId. This is incorrect. We cannot return service profile in response to network allocation request. - No stage 3 content defined for AllocateNetwork operation. Considering the UC and the requirements, the procedure defined in clause “7.16 Procedure of allocating network with or without slicing for communication services” has the following defects - The usecase and requirements does not talks about slicing at all. In this case procedures talking about with slice or without slice is incorrect. The procedure includes - The procedure use terms like NS_Consumer, NS_Provider, NMS_Provider which are not defined. - The step 3b seems to be the only relevant step and that does not say anything. It does not refer to anything defined. Hence, the procedure is incorrect and incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete all stage 2 content for allocate network functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and un-implementable specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4, 6.5.5, 7.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 6.4	Void

Next Modified Section

### 6.5.5	Void

Next Modified Section

## 7.16	Void

End of Modified Section