3GPP TSG-SA5 Meeting #146	S5-226806

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.156 | CR | 0045 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deprecating model elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_MetDep |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When a model element needs to be removed or changed in a non-backwards compatible manner, there is a need to notify users in advance. If there is a replacement model element for the changed element, it is good to allow the usage of both the old and the new element side-by-side, while marking the old element as deprecated: not recommended for future use. The deprecated property will indicate that for a model element there is a better alternative in the NRM or that it will be removed in the future. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a lifecycleStatus property to the attribute, data type and IOC model elements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Users will have no notification when a model element is replaced or deprecated elements will be kept forever. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1, 5.2.A, 5.2.A.1, 5.2.A.1.1, 5.3.2.2, 5.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.1.1	Description

It is a typed element representing a property of a class. See 10.2.5 Property of [1].

An element that is typed implies that the element can only refer to a constrained set of values.

See 10.1.4 Type of [1] for more information on type.

See 5.3.4 and 5.4.3 for predefined data types and user-defined data types that can apply type information to an element.

The following table captures the properties of this modelled element.

Table 5.2.1.1-1: Attribute properties

| Property name | Description | Legal values |
| --- | --- | --- |
| documentation | Contains a textual description of the attribute. Should refer (to enable traceability) to the specific requirement. | Any |
| isOrdered | For a multi-valued multiplicity; this specifies if the values of this attribute instance are sequentially ordered. See subclause 7.3.44 and its Table 7.1 of [2]. If the property is present for attributes with a multiplicity of greater than “1”, it shall be set to either “True” or “False”. It shall not be set to “N/A”. | True, False (default) |
| isUnique | For a multi-valued multiplicity, this specifies if the values of this attribute instance are unique (i.e., no duplicate attribute values). See subclause 7.3.44 and its Table 7.1 of [2]. If the property is present for attributes with a multiplicity of greater than “1”, it shall be set to either “True” or “False”. It shall not be set to “N/A”. | True (default), False |
| isReadable | Specifies that this attribute can be read by the manager. | True , False (default), (see NOTE 3) |
| isWritable | Specifies that this attribute can be written by the manager under the conditions specified in Annex B. | True, False (default) (see NOTE 4) |
| type | Refers to a predefined (see subclause 5.4.3) or user defined data type (see section 5.3.4). See also subclause 7.3.44 of [2], inherited from StructuralFeature. | NA |
| isInvariant | Attribute value is set at object creation time and cannot be changed under the conditions specified in Annex B. | True, False (default) |
| allowedValues | Identifies the values the attribute can have. When specific values or restrictions are listed, this means that other values are not allowed, except if the attribute description explicitly states that other values are allowed. If other values shall be allowed the description should include the text: "isExtensible=True" | Dependent on type |
| isNotifyable | Identifies if a notification shall be sent in case of a value change (see Note 1, Note 2). | True (default), False |
| defaultValue | Identifies a value at specification time that is used at object creation time under conditions defined in Annex B. If there is no defined default value, the property shall be omitted from the attribute description or specified as ‘defaultValue: None.’. | No value (default) or a value that is dependent on allowedValues |
| multiplicity | Defines the number of values the attribute can simultaneously have. See subclause 7.3.44 of [2]; inherited from StructuralFeature. | See 5.2.8 Default is 1 |
| isNullable | Identifies if an attribute can carry no information. The implied meaning of carrying “no information” is context sensitive and is not defined in this Model Repertoire. | True, False (default) |
| supportQualifier | Identifies the required support of the attribute. See also subclause 6. | M, O (default), CM, CO, C |
| passedById | See Table 5.2.9.1-1: passedById property | True, False (default) |
| lifecycleStatus | See Table 5.2.A.1-1 | Current (default), Deprecated |

NOTE 1: Whether a client/manager can receive the notification depends on a) if the client/manager has subscribed or registered for reception of such notification and b) if a notification mechanism is supported.

NOTE 2: If the attribute is a role-attribute and its property passedById is ‘False’, then changes in the navigable association target end instance do not trigger a notification.

NOTE 3: The value True is conditional on the presence of a mechanism for the MnS Producer to authenticate the MnS Consumer and verify whether the authenticated MnS Consumer is authorized to read.

NOTE 4: The value True is conditional on the presence of a mechanism for the MnS Producer to authenticate the MnS Consumer and verify whether the authenticated MnS Consumer is authorized to write.

Next change

### 5.2.A	LifecycleStatus

#### 5.2.A.1	Description

Model elements may have a life-cycle. They are created, updated, become obsolete and may be removed. The lifecycleStatus property indicates this. LifecycleStatus is applicable to attributes, data types, IOCs operations and notifications.

Table 5.2.A.1-1: lifecycleStatus property

| Property name | Description | Legal values |
| --- | --- | --- |
| lifecycleStatus | "Current" means that the definition of the element is current and valid, it is freely available for use. "Deprecated" means the element has a valid definition, it is available for use, but its use is discouraged. Deprecated elements may already have a replacement element defined. Deprecated elements may be removed in the next release. | Current(default), Deprecated |

##### 5.2.A.1.1 Removing/Deprecating model elements

When removal or a non backwards compatible change is needed for a model element, it shall be kept in the specification as-is but be marked as deprecated for one release. The deprecated element may be removed in the next release.

A new replacing model element may be defined beside the original. In this case the replacing element shall be indicated in the specification of the old element.

Implementations of the previous release that now implement the current release shall continue to support usage of the deprecated attributes/classes as well as any new replacing attributes/classes, but not at the same time. As soon as the newer (replacing) attributes/classes are used, it may no longer be possible to also support usage of the deprecated elements or show correct values for the deprecated attributes. (E.g. when the type of an attribute is changed from integer to string). Once the replacing attribute/IOC is used, the old attribute/IOC may lose functionality and should not be used anymore.

In case the deprecated or the replacing element was or is intended to have a multiplicity strictly greater than zero (mandatory to configure/report), the model elements should be declared with a multiplicity including zero, as only one of the deprecated and the replacement elements will be used at any one time.

The deprecating procedure shall be used between releases. There is no need to follow it during the development of a single release, as long as the release is not yet frozen.

Next change

#### 5.3.2.2	Example

This example shows an AbcFunction <<InformationObjectClass>>.

Figure 5.3.2.2-1: <<InformationObjectClass>> notation

The following table captures the properties of this modelled element.

Table 5.3.2.2-1: <<InformationObjectClass>> properties

| Property name | Description | Legal values |
| --- | --- | --- |
| documentation | Contains a textual description of this modelled element. Should refer (to enable traceability) to a specific requirement. | Any |
| isAbstract | Indicates if the class can be instantiated or is just used for inheritance. | True, False (default) |
| isNotifyable | Identifies the list of the supported notifications. | List of names of notification |
| supportQualifier | Identifies the required support of the class. See also subclause 6. | M, O (default), CM, CO, C |
| lifecycleStatus | See Table 5.2.A.1-1 | Current (default), Deprecated |

Next change

#### 5.3.4.1	Description

It represents an attribute property type (see Table 5.2.1.1-1: Attribute properties).

This repertoire uses two kinds of data types: predefined data types and user-defined data types. The former is defined in subclause 5.4.3. The latter is defined by the specifications authors using this <<dataType>> model element.

The names of predefined data types and user-defined data types must be chosen such that they do not clash.

The user-defined data types support the modelling of structured data types (see <<dataType>> PLMNId in 5.3.4.2).

User defined data types are qualified by the lifecycleStatus property. (See Table 5.2.A.1-1).

When a user-defined or predefined data type is used to apply type (see property named type in Table 5.2.1.1-1: Attribute properties) information to a class attribute, the data type name is shown along with the class attribute. See Example below.

End of  changes