3GPP TSG-SA5 Meeting #146	S5-226192

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.160 | CR | 0031 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deprecating model elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_MetDep |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When a model element needs to be removed or changed in a non-backwards compatible manner, there is a need to notify users in advance. If there is a replacement model element for the changed element, it is good to allow the usage of both the old and the new element side-by-side, while marking the old element as deprecated: not recommended for future use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a lifecycleStatus property to the operation, notification, attribute, data type and IOC model elements. lifecycleStatus for attributes is included in the attribte constrains table because: - an attribute can be deprecated for a specific IOC while it can still be current for other IOCs. Because the lifecycleStatus can be different for different IOCs it cannot be included in the common attributes table. - it is rare that the property is specified to anything beside its default value, so putting it in the attribute table as a separate column (below each IOC) would be a waste of space. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Users will have no notification when a model element is replaced or deprecated elements will be kept forever. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.1.11.a(new),6.2.a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... 32.156 CR-0045 |  |  |
| Other comments: |  | This CR is dependent on the CR-0045 for TS 32.156 Note to MCC: While this is NOT the topic of this CR, the heading numbers in the current TS 32.160 under heading 5.2 should be improved, together with setting the appropriate header levels for the paragraphs. Currently these headers don't appear in the table of contents. Some heading are in incorrect order e.g. clause number Yb.1.a follows clause number Yb.1.a.4,2. That is confusing and if MCC could correct it, it would be nice. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### W4.3.a.1	Definition

This clause is written in natural language. The <definition> clause refers to the class itself.

Classes(and datatypes) have a  lifecycleStatus property as defined by [3] clause 5.2.A. If the lifecycleStatus is not current (its default value), that shall be indicated in this subclause.

Optionally, information on traceability back to one or more requirements supported by this class may be defined here, in the following form:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| TS 28.xyz [xy] | REQ-SM-CON-23 | Optional clarification |
| TS 28.xyz [xy] | REQ-SM-FUN-11 | Optional clarification |

Next change

#### W4.3.a.3	Attribute constraints

This clause presents constraints for the attributes, and one use is to present the predicates for conditional qualifiers (CM/CO).

This information is provided in a table. An example of such a table is given here below:

| Name | Definition |
| --- | --- |
| configuredMaxTxPower CM support qualifier | Condition: The sector-carrier has a downlink [4]. |
| sNSSAIList CM support qualifier | Condition: Network slicing feature is supported [4]. |

Attributes have a  lifecycleStatus property as defined by [3] clause 5.2.A. If the lifecycleStatus is not current (its default value), that shall be indicated in this table.

This clause shall state "None." when there is no attribute constraint to define.

Next change

##### Yb.1.a.1.1	Description

This subclause shall be written in natural language.

Operations have a  lifecycleStatus property as defined by [3] clause 5.2.A. If the lifecycleStatus is not current (its default value), that shall be indicated in this subclause.

Information on traceability back to one or more requirements supported by this operation should also be defined here, in the following form:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.xyz [xy] | REQ-SM-CON-23 | Optional clarification |
| 3GPP TS 32.xyz [xy] | REQ-SM-FUN-11 | Optional clarification |

Next change

#### Yb.1.a.1	Definition

This subclause shall be written in natural language.

Notifications have a  lifecycleStatus property as defined by [3] clause 5.2.A. If the lifecycleStatus is not current (its default value), that shall be indicated in this subclause.

Information on traceability back to one or more requirements supported by this notification should also be defined here, in the following form:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.xyz [xy] | REQ-SM-CON-23 | Optional clarification |
| 3GPP TS 32.xyz [xy] | REQ-SM-FUN-11 | Optional clarification |

Next change

#### 6.1.11.a	Attribute property "lifecycleStatus"

LifecycleStatus=current is the default case so it is not mapped to any JSON schema keyword.

LifecycleStatus=deprecated shall be mapped the "deprecated" keyword with a value of true.

Next change

### 6.2.a	LifecycleStatus

#### 6.2.a.1	Introduction

Reference [3] clause 5.2.A - LifecycleStatus

#### 6.2.a.2	YANG mapping

LifecycleStatus=current is the default case in YANG so it needs no mapping.

LifecycleStatus=deprecated shall be mapped to the YANG statement

status deprecated;

under the relevant leaf, leaf-list, list, container or grouping.

End of  changes