3GPP TSG-SA5 Meeting #146	S5-226199

# 14 - 18 November 2022, Toulouse, FR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.240 | CR | 0449 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the missing reference point for EES charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, MATRIXX Doftware |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_CH |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The reference point for EES charging is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the reference point for EES charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EES charging cannot be supported without the reference point. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 4.4.2, 4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 32.257 CR #0004 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3G	3rd Generation

3GPP	3rd Generation Partnership Project

5GC	5G Core Network

5GS	5G System

5G DDNMF	5G Direct Discovery Name Management Function

ABMF	Account Balance Management Function

AF	Application Function

AMF	Access and Mobility Management Function

AoC	Advice of Charge

API	Application Program Interfaces

APN	Access Point Name

AS	Application Server

BD	Billing Domain

BGCF	Breakout Gateway Control Function

BS	Bearer Services

BSC	Base Station Controller

BSS	Base Station Subsystem

BTS	Base Transceiver Station

CAMEL	Customized Applications for Mobile network Enhanced Logic

CAP	CAMEL Application Part

CCS	Converged Charging System

CDF	Charging Data Function

CDR	Charging Data Record

CG	Charging Gateway

CGF	Charging Gateway Function

CHF	Charging Function

CN	Core Network

CP	Control Plane

CS	Circuit Switched

CSCF	Call Session Control Function (I-Interrogating; E-Emergency; P-Proxy; and S-Serving)

CTF	Charging Trigger Function

EATF	Emergency Access Transfer Function

EBCF	Event Based Charging Function

ECUR	Event Charging with Unit Reservation

EES	Edge Enabler Server

EIR	Equipment Identity Register

EPC	Evolved Packet Core

ePDG	Evolved Packet Data Gateway

EPS	Evolved Packet System

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FQPC	Fully Qualified Partial CDR

GGSN	Gateway GPRS Support Node

GMLC	Gateway MLC

GMSC	Gateway MSC

GPRS	General Packet Radio Service

GSM	Global System for Mobile communication

gsmSCF	GSM Service Control Function

gsmSSF	GSM Service Switching Function

GSN	GPRS Support Node (either SGSN or GGSN)

HLR	Home Location Register

HPLMN	Home PLMN

HSCSD	High Speed Circuit Switched Data

IBCF	Interconnect Border Control Function

ICS	IMS Centralized Services

IE	Information Element

IEC	Immediate Event Charging

IETF	Internet Engineering Task Force

IMEI	International Mobile Equipment Identity

IMS GWF	IMS GateWay Function

IMS	IP Multimedia Subsystem

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

ISC	IMS Service Control

ISDN	Integrated Services Digital Network

ITU-T	International Telecommunication Union - Telecommunications standardization sector

LAC	Location Area Code

LAN	Local Area Network

LCS	Location Services

LRF	Location Retrieval Function

LTE	Long Term Evolution

MAP	Mobile Application Part

MBMS	Multimedia Broadcast and Multicast Service

MDAS	Management Data Analytics ServiceME	Mobile Equipment

MGCF	Media Gateway Control Function

MGW	Media GateWay

MLC	Mobile Location Center

MME	Mobility Management Entity

MMI	Man-Machine Interface

MMS	Multimedia Messaging Service

MMSE	Multimedia Messaging Service Environment

MMTel	MultiMedia Telephony

MNO	Mobile Network Operator

MnS	Management Service

MO	Mobile Originated

MOC	MO Call

MRF	Media Resource Function

MRFC	MRF Controller

MS	Mobile Station

MSC	Mobile Services Switching Centre

MSISDN	Mobile Station ISDN number

MT	Mobile Terminated

MTC	MT Call

MVNO	Mobile Virtual Netork Operator

NE	Network Element

NF	Network Function

NWDAF	Network Data Analytics Function

OCF	Online Charging Function

OCS	Online Charging System

OFCS	Offline Charging System

OMR	Optimal Media Routing

PCEF	Policy and Charging Enforcement Function

PCF	Policy Control Function

PCRF	Policy and Charging Rules Function

PDG	Packet Data Gateway

PDN	Packet Data Network

PDP	Packet Data Protocol, e.g. IP

PLMN	Public Land Mobile Network

PoC	Push-to-talk over Cellular

ProSe	Proximity-based Services

PS	Packet-Switched

PSPDN	Packet-Switched Public Data Network

QoS	Quality of Service

RF	Rating Function

RNC	Radio Network Controller

RNS	Radio Network Subsystem

RPC	Reduced Partial CDR

SBCF	Session Based Charging Function

SCCP	Signalling Connection Control Part

SCEF	Service Capability Exposure Function

SCF	Service Control FunctionSCS	Services Capability Server

SCUR	Session Charging with Unit Reservation

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMS	Short Message Service

SMF	Session Management Function

SSF	Service Switching Function

TAP	Transferred Account Procedure

TDF	Traffic Detection Function

TR	Technical Report

TRF	Transit and Roaming Function

TS	Technical Specification

TWAG	Trusted WLAN Access Gateway

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

UPF	User Plane Function

USIM	Universal SIM

VAS	Value Added Service

VLR	Visitor Location Register

VMSC	Visited MSC

VPLMN	Visited PLMN

WLAN	Wireless LAN

2nd modified section

### 4.2.3	Common architecture – service based interface

The following figures provide an overview of the logical ubiquitous charging architecture and the information flows for converged offline and online charging in service based interface variant for 5G systems and Edge Computing enabling sub-systems.

Figure 4.2.3.1 provides the overview in service based representation:

Figure 4.2.3.1: Logical ubiquitous charging architecture and information flows for 5G systems – service based representation

Figure 4.2.3.2 provides the overview in reference point representation:

Figure 4.2.3.2: Logical ubiquitous charging architecture and information flows for 5G systems – reference point representation

The reference points are defined in clause 4.4.3.

For the sake of simplicity, the SMF+PGW-C is not explicitly added in Figure 4.2.3.1 and Figure 4.2.3.2, and is represented by the SMF.The SMF+PGW-C uses Nchf for 5GS and EPC interworking as well as when enhanced to support GERAN/UTRAN.

The Nchf_SpendingLimitControl service exposed by CHF and consumed by the PCF is specified in TS 23.502 [214].

3rd modified section

### 4.4.3	Charging services Reference point

The common charging architectures are mapped into the specific domain/subsystem/service charging architectures in the respective middle tier TSs, which contain in their reference point representation, the following reference points:

N28:	Reference point between PCF and CHF defined in TS 23.501[215].

N40:	Reference point between SMF and the CHF in the same PLMN defined in clause 4.2 of TS 32.255 [15].

N41:	Reference point between AMF and CHF in HPLMN defined in clause 4.2.2 of TS 32.256 [16].

N42:	Reference point between AMF and CHF in VPLMN defined in clause 4.2.2 of TS 32.256 [16].

N44:	Reference point between NEF and CHF defined in clause 4.4 of TS 32.254 [14].

N45:	Reference point between IMS Node and CHF defined in clause 4.4 of TS 32.260 [20].

N46:	Reference point between SMSF and CHF defined in clause 4.4 of TS 32.274 [34].

N47: 	Reference point between SMF and the CHF in different PLMNs defined in clause 4.2 of TS 32.255 [15].

N49: 	Reference point between EES and CHF defined in clause 4.2.3 of TS 32.257 [17].

End of modified sections