3GPP TSG-SA WG6 Meeting #52	S6-223347

Toulouse, France 14th – 18th November 2022	(revision of S6-223152)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0323 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flows for the ad hoc group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Kontron Transportation France, Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2022-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TR 23.700-76 V1.3.0 has defined the information flows for the adhoc group communication. The conclusion in the clause 9 for the Key issue #1 indicates that the information flows in the clause 7.1.2 of solution 1 is agreed without the “AHGC share security material command”, and exluding the “MC service ad hoc group ID” & “Preconfigured ad hoc group identity” IEs from the ad hoc group communication request from the MCPTT client to the MCPTT server.This CR is trying to propose the corresponding normative text in the normative work phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the new clause to provide the information flows and procedures for the adhoc group call. Added the information flows as defined in the clause 7.1.2 of TR 23.700-76 V1.3.0 into new clause 10.19 of TS 23.379 as a normatie text. The information flows between MCPTT servers for “Ad hoc group call request” and “Ad hoc group call response” are added which were missing in the TR 23.700-76 V1.3.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ad hoc group call support will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (10.19, 10.19.1, 10.19.2, 10.19.2.1, 10.19.2.2, 10.19.2.1, 10.19.2.2, 10.19.2.3, 10.19.2.4, 10.19.2.5, 10.19.2.6, 10.19.2.7, 10.19.2.8, 10.19.2.9, 10.19.2.10) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 10.19	Ad hoc group call

### 10.19.1	General

This subclause contains procedures for ad hoc group call across a single MCPTT system, and associated functions such as emergency call, Imminent peril call, broadcast call and others.

The ad hoc group calls can use the participants list provided by either an initiator of the call or MCPTT server. The MCPTT server can use the criteria provided by the initiator of the call to determine the participants list along with its own criteria or local policies. The resulting ad hoc group uses the configuration of a separate preconfigured MCPTT group.

NOTE 1:	A preconfigured group which is intended only to provide configuration for the ad hoc group is identified by a parameter in the group configuration described in 3GPP TS 23.280 [16].

The preconfigured MCPTT group that provides the configuration is not used for the MCPTT group communication, it only provides configuration for one or more adhoc group calls. The MCPTT group ID of the ad hoc group call is provided by the MCPTT server when the ad hoc group call is originated. In order to establish security context for the end-to-end secured ad hoc group call, the security related information is used from this preconfigured group.

NOTE 2:	The configurations defined for the adhoc group call in user profile and service configuration document as described in the Annex A.3 and A.5 is applicable only for single MCPTT system.

Editor's note: The configurations required for supporting adhoc group call involving multiple MCPTT system is FFS.

### 10.19.2	Information flows

#### 10.19.2.1	Ad hoc group call request (MCPTT client – MCPTT server)

Table 10.19.2.1-1 describes the information flow ad hoc group call request from the MCPTT client to the MCPTT server.

Table 10.19.2.1-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MC service ad hoc group ID (see NOTE 1) | O | The MC service group ID which is generated by the MC service user to be associated with the ad hoc group communication |
| Encryption supported (see NOTE 2) | O | Indicates whether this ad hoc group call supports end-to-end encryption |
| MCPTT ID list (see NOTE 3, NOTE 6) | O | MCPTT IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Media parameters of MCPTT clients |
| Implicit floor request (see NOTE 4) | O | When originating client requests the floor, this element shall be included |
| Broadcast indicator (see NOTE 5) | O | Indicates that the group call request is for a broadcast group call |
| Imminent peril indicator (see NOTE 5) | O | Indicates that the ad hoc group call request is an MC service imminent peril call |
| Emergency Indicator (see NOTE 5) | O | Indicates that the ad hoc group call request is an MC service emergency call |
| MCPTT ID list (see NOTE 3) | O | List of participants required to acknowledge the ad hoc group call before start of the audio transmission |
| Location information | O | Location of the calling party. |
| Criteria for determining the participants (see NOTE 6) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: If this information element is not included, the MC service server assigns an MC service ad hoc group ID to be used for the ad hoc group communication. This information element is returned to the calling party to use in the ad hoc group communication. NOTE 2: This information element is present and set to true only if this ad hoc group call is encrypted. When the ad hoc group call is initiated with participants provided by the initiator this acts as an indicator that subsequent requests follow targeting the individual participants and carrying the relevant key material. If this information element is set to false or not present, then this ad hoc group call is unencrypted. NOTE 3: This element is included only when the originating client sends the list of participants. NOTE 4: This element is included only when the originating client requests the floor. NOTE 5: If used, only one of these information elements is present. NOTE 6: Only one of these information elements is present. |  |  |

#### 10.19.2.2	Ad hoc group call request return (MCPTT server – MCPTT client)

Table 10.19.2.2-1 describes the information flow ad hoc group call request return from the MCPTT server to the MCPTT client. This response could be intermediate response to provide the server assigned MCPTT ad hoc group ID.

Table 10.19.2.2-1 Ad hoc group call request return information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ad hoc group ID | O | The MCPTT group ID to be associated with the ad hoc group call which is assigned by the MCPTT server.This information element shall be present if the authorization result is success. |
| Authorization result | M | Indicate if authorization is success or failure |

#### 10.19.2.3	Ad hoc group call request (MCPTT server – MCPTT server)

Table 10.19.2.3-1 describes the information flow ad hoc group call request between the MCPTT servers.

Table 10.19.2.3-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| MCPTT ID list (see NOTE 1, NOTE 3) | O | MCPTT IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MC service imminent peril call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MC service emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

#### 10.19.2.4	Ad hoc group call request (MCPTT server – MCPTT client)

Table 10.19.2.4-1 describes the information flow ad hoc group call request from the MCPTT server to the MCPTT client.

Table 10.19.2.4-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is an MC service imminent peril call |
| Emergency Indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is an MC service emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| NOTE 1: If used, only one of these information elements is present. |  |  |

#### 10.19.2.5	Ad hoc group call response (MCPTT server – MCPTT client)

Table 10.19.2.5-1 describes the information flow ad hoc group call response from the MCPTT server to the MCPTT client.

Table 10.19.2.5-1 Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the group call request (success or failure) |

#### 10.19.2.6	Ad hoc group call response (MCPTT server – MCPTT server)

Table 10.19.2.6-1 describes the information flow ad hoc group call response between the MCPTT servers.

Table 10.19.2.6-1 Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MC service user |
| Functional alias | O | The functional alias of the target MC service user |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group call request (success or failure) |

#### 10.19.2.7	Ad hoc group call response (MCPTT client – MCPTT server)

Table 10.19.2.7-1 describes the information flow ad hoc group call response from the MCPTT client to the MCPTT server.

Table 10.19.2.7-1 Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MC service user |
| Functional alias | O | The functional alias of the target MC service user |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group call request (success or failure) |

#### 10.19.2.8	Ad hoc group call release request (MCPTT server – MCPTT client)

Table 10.19.2.8-1 describes the information flow ad hoc group call release request from the MCPTT server to the MCPTT client.

Table 10.19.2.8-1 Ad hoc group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCPTT ad hoc group ID | M | The MCPTT group ID of the ad hoc group call on which call is released |

#### 10.19.2.9	Ad hoc group call release response (MCPTT client – MCPTT server)

Table 10.19.2.9-1 describes the information flow ad hoc group call release response from the MCPTT server to the MCPTT client.

Table 10.19.2.9-1 Ad hoc group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCPTT group ID | M | The MCPTT group ID of the ad hoc group call on which call is released |

#### 10.19.2.10	Ad hoc group call notify (MCPTT server – MCPTT client)

Table 10.19.2.10-1 describes the information flow ad hoc group call notify from MCPTT server to MCPTT client.

Table 10.19.2.10-1: Ad hoc group call notify

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID associated with the ad hoc group call |
| MCPTT ID list (see NOTE) | O | The list of the invited MC service users who did not acknowledge the ad hoc group call request |
| NOTE: Only applicable to acknowledged group calls. |  |  |

* * * End of Changes * * * *