3GPP TSG-SA6 Meeting #52	S6-223569

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0074 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for VRU zone configuration and operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo Future Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-64, Solution #2 was concluded to be considered for the eV2XAPP_ph3 normative phase. This contribution provides the procedures, information flows and requirements for the new VAE layer feature related to VRU zone configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the new feature, a new requirement was added in 4.x, and the procedures/information flows were added in clause 9.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (new), 4.x.1 (new), 4.x.2 (new), 9.x (new), 9.x.1 (new), 9.x.2 (new), 9.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

## 4.X	Support for VRU zone configuration and operation

### 4.x.1	Description

This subclause specifies the VRU zone configuration and operation related requirements.

### 4.x.2	Requirements

[AR-4.x.2-a] The VAE capabilities shall support VRU zone configurations (e.g. distribution of VRU zone related configuration information) to the relevant V2X UEs.

* * * * * * * SECOND CHANGE * * * * * * *

## 9.X	Support for VRU zone configuration and operation

### 9.x.1	General

This capability supports the configuration and provisioning of the VRU high risk zone at the VAE layer based on a request from the V2X application specific server (VASS).

### 9.x.2	Information flows

#### 9.x.2.1	VRU zone management subscription request

Table 9.x.2.1-1 describes information elements for the VRU zone management subscription request from the VASS to the VAE server.

Table 9.x.2.1-1: VRU zone management subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the service requestor (VASS, VAL server, App ID) |
| VRU zone ID | O | The identifier of the VRU zone which may be provided by the requestor. |
| VRU zone information | M | This information identifies, based on the VRUP scenario, which type of UEs (V2X UEs, pedestrian UEs) are to be considered. Also, whether it is a static or dynamic zone can be identified. |
| VRU application requirements | M | This includes requirements include the types of supported messages and requirements (e.g. requirements for VAM messages), and application QoS requirement dedicated for the VRU zone (e.g. URLLC like). |
| List of VAL UE IDs | O | The identifiers of the VAL UEs for which the VRU zone is applicable |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

#### 9.x.2.2	VRU zone management subscription response

Table 9.x.2.2-1 describes information elements for the VRU zone management subscription response from the VAE server to VASS.

Table 9.x.2.2-1: VRU zone management subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the VRU zone management subscription request (positive or negative acknowledgement) |

#### 9.x.2.3	VRU zone info notification

Table 9.x.2.3-1 describes information elements for the VRU zone info notification from the VAE server to the VAE clients.

Table 9.x.2.3-1: VRU zone info notification

| Information element | Status | Description |
| --- | --- | --- |
| VRU zone ID | M | The identifier of the VRU zone |
| VRU zone information | M | This information identifies based on the VRUP scenario, which type of UEs (V2X UEs, pedestrians) are to be considered. Also, whether it is a static or dynamic zone. |
| VRU zone area | M | The geographical (coordinates) or topological area for the zone (cell IDs, TAs) |
| QoS requirements | M | The application QoS requirements for the services within the zone |
| VRU zone configuration parameters | M | These parameters include: - The list of value-add VAE services required within the zone (e.g. location tracking, V2X message bundling), - the supported transmission modes (unicast, groupcast, broadcast) within the applications within the zone, - the allowed interface selection within the zone (Uu, PC5), - the use of application relaying or not |
| VRU zone adaptation assistance info | O | If the VRU zone is dynamic, this IE provides assistance info on the configuration adaptation triggers or to provide the planning (e.g. based on a school bus route) to allow the network to adapt the configuration. |

#### 9.x.2.4	Enter/Leave Notification

Table 9.x.2.4-1 describes information elements for the Enter/Leave Notification from the VAE server to the VASS or the VAE client(s) within the zone.

Table 9.x.2.4-1: Enter/Leave Notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M (NOTE) | The identity of the UE of interest (GPSI, or external ID) |
| VAL group ID | M (NOTE) | The group UE identity (if this applies to a group of UEs). |
| VRU zone ID | M | The identifier of the VRU zone |
| VRU zone information | M | This information identifies based on the VRUP scenario, which type of UEs (V2X UEs, pedestrians) are to be considered. Also, whether it is a static or dynamic zone. |
| Enter/Leave timing info | M | The timing info for the UE enter / leave the zone. This includes for entering a start time of VRUP and an expected duration, and for leaving an expected leave time (if the UE is approaching the VRU zone boundaries) |
| UE/group mobility info | O | The expected mobility/speed/direction of the UE or group of interest |
| NOTE: One of these shall be present. |  |  |

### 9.x.3	 Procedure on VAL server - triggered VRU zone configuration

Figure 9.x.3-1 illustrates the procedure where the VAE server configures the VRU zone based on application requirement; and supports the run-time operation based on an expected UE entrance to the zone.

Pre-conditions:

1.	V2X application specific server or client has subscribed to VAE layer to provide support for V2P communication.

Figure 9.x.3-1: VAL server - triggered VRU zone configuration

1.	The V2X application specific server sends a subscription request to manage the configuration of a new high-risk area zone and requires the VAE server support to translate it to a network-related zone configuration and provisioning to the UEs within the requested area.

2.	The VAE server processes the request and stores the subscription. The processing includes the translation of the zone requirement of step 1 and determines a set of network-related zone parameters which indicates the configuration parameters.

3.	The VAE sever sends a VRU zone management subscription response to VASS.

4.	The VAE server initiates the SEAL LMS service for location tracking both for static location and dynamically changing location for all UEs within the VRU zone area. VAE server obtains and initiates tracking the V2X UEs location from the location management server 1 as specified in 3GPP TS 23.434 (clause 9.3.12) or, for static area, it obtains location area monitoring information based on clause 9.3.10 of 3GPP TS 23.434.

5.	The VAE server sends a notification message including the VRU zone configuration parameters. Parameters can be based on the parameters provided in step 2. This configuration will be provided to the V2X-UEs (and pedestrians) within the area that will be covered by the VRU zone.

6.	The VAE server keeps monitoring the VRU zone area based on monitoring SEAL LMS events. This includes events on whether a UE (e.g. UE1) is moving in or out a target area of interest.

The VAE server translates the UE1 mobility event to an expected entrance to a VRU high risk zone, and based on the configuration of the zone, it identifies whether to generate and send a notification to the VASS on the expected entrance / leave of UE1.

7.	The VAE server alerts the VASS that the UE1 is expected to move to the VRU zone area in a given time horizon and provides also information on its mobility as well as the UE1 capabilities (e.g. VRU capable).

8.	The VAE server may also alert the VAE client (if deployed at the UE1) that it is expected to enter the VRU zone and requests the confirmation for allowing the push of VRU messages within the zone.