TSG SA#98-e	SP-221289

13 - 19 December 2022, Electronic meeting	(Revision of SP-22xxxx)

Title:	[draft] Reply LS on 5G capabilities exposure for factories of the future – identified gaps

Response to:	SP-221005

Release:	Rel-18

Work Item:	-

Source:	3GPP TSG SA

To:	5G-ACIA, 3GPP TSG CT

Cc:	3GPP TSG SA WG1, 3GPP TSG SA WG2, 3GPP TSG SA WG3, 3GPP TSG SA WG5, 3GPP TSG SA WG6

Contact Person:

#### Name:	Patrice Hédé

E-mail Address:	patrice dot hede at huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA would like to thank 5G-ACIA for their LS (SP-221005). In response to the actions to 3GPP:

5G-ACIA kindly asks 3GPP to take note of the identified gaps and limitations in Rel-17 specifications for considerations in the ongoing and future activities of 3GPP.

SA would like to point out that 3GPP has analylized those identified gaps and limitations, and the result of our findings are outlined in the last column of the following tables:

| Gap ID | Description | Corresponding requirement in the 5G-ACIA whitepaper mentioned above | 3GPP Analysis |
| --- | --- | --- | --- |
| G.1 | Provisioning of single subscriptions or a bulk of subscriptions by a provisioning server (which is assumed to be an IIoT function outside of the NPN domain) is identified as a requirement from enterprises. This is not seen as a gap in 3GPP specifications, because a provisioning protocol is not currently in the scope of 3GPP. Provisioning via exposure reference points could be considered for future work by 3GPP. | [R-4.2.2-01] The 5G exposure reference points must support integration into and configuration of a device within a 5G system by provisioning the relevant UE information (e. g. UE IDs, network access authentication keys, subscriptions) to the 5G network so it will accept device connection when the device is activated. [R-4.2.2-02] The 5G exposure reference points must support provisioning and onboarding of individual devices and groups of devices | 3GPP SA2 had earlier considered this gap as an objective within the Rel-18 work, e.g. S2-2108574, however this functionality was finally taken out from Rel-18 work due to restricted scoping and considered that provisioning protocol is running transparently over PDU session. This gap can be a potential candidate for future 3GPP SA2 (e.g., Rel-19) . |
| G.2 | The 5G exposure reference point must be capable of acknowledging a service request within 100 ms. The acknowledgement may include the requested information or may be a service request acknowledgement followed by the actual requested information. In the latter case the requested information must be provided within a specified time. | [R-4.2.3-02] The 5G exposure reference points must be capable of acknowledging a communication service request within 100 ms. [R-4.2.4-06] The 5G exposure reference point must respond to a request to provide real-time QoS monitoring information within a specified time. | 3GPP specifications allow a request to contain "3gpp-Sbi-Mas_rsp" to indicate the maximum time period to complete the processing of the request, and the recipient of the request will check that the request has not already timed out during the processing of the request, e.g. upon receipt of a response from the next upstream NF service. However, actual processing time of the request depends on lots of factors, e.g. UE state, network congestion status, the requested information (e.g. QoS or Monitoring Event or Parameter Provisioning) etc. This gap can be a potential candidate for future 3GPP SA2 e.g. Rel-19. |
| G.3 | Provision of security event/logging information is identified as gap in 3GPP specifications. | [R-4.4-5] The 5G exposure reference point must be able to provide a history of communication events. | The history opertional event/logging information is typically managed by O&M functionalities and stored in equipment and/or in O&M systems. Security event/logging information requires to be further defined in details and investigation are needed on whether existing event/logging mechanisms could be reused. |
| G.4 | For providing a traffic profile applicable to the connections of a UE, and for knowing the status, the current specifications do not address the case of a group of devices in a single request. | [R-4.2.3-06] The exposure reference points must enable the IIoT application to provide a traffic profile applicable to a single connection, to all connections of a device, or to all connections of a group of devices. (...) The traffic profile may be provided when a new connection is requested, or when an existing connection is modified. | This gap has been addressed as part of 3GPP SA2 Rel-18 FS_GMEC, see clause 5.2, 5.3, 8.2 and 8.3 in 3GPP TR 23.700-74. |

| Limitation ID | Description | Corresponding requirement in the 5G-ACIA whitepaper mentioned above (if applicable) | 3GPP Analysis |
| --- | --- | --- | --- |
| L.1 | A solution for VLAN tag assignment that is not dependent on secondary authentication and a corresponding authentication server may be needed. Note: The only standardized way of assigning VLAN tags is by using an authentication server DN-AAA with secondary authentication. This means that an authentication server DN-AAA must be deployed, and authentication credentials for secondary authentication must be managed just for supporting VLAN tag assignment, even in cases where there is no other reason for using secondary authentication. | N/A | L.1 is not a limitation anymore as it has already been resolved. Accordingly, 3GPP specification allows an AF to provide Flow Descriptions via NEF exposure interface without needing any authentication server DN-AAA or without having to involve secondary authentication and the Flow Descriptions include Ethernet Packet Filters consisting of a VLAN tag, see 5.7.6.3 in 3GPP TS 23.501. |
| L.2 | Currently, application-centric QoS profiles such as those in clause 5 in TS 22.104 can only be translated into “5G-native” QoS profiles in a pre-defined manner. For instance, the factory owner describes these QoS profiles in an SLA. Thereafter, the NPN operator translates them into “5G-native” profiles and provides the owner with IDs for each of the profiles. However, it can be foreseen that not all QoS profiles are known, for instance if automation devieces intermittently enter the factory premise and request previously undefined QoS communication service. Therefore, a more dynamic solution is needed. | N/A | When there is a pre-defined QoS information, the AF can request the network to provide required QoS based on the service requirements with the help of a QoS Reference parameter. 3GPP SA2 Rel-17 IIoT has defined another way for the AF to provide the required QoS in a dynamic mananer, namely the AF provides individual QoS parameters and traffic characteristics associated to the Flow Description instead of the QoS Reference, the AF can also provide Alternative Service Requirements in a prioritized order in addition to the QoS Reference or individual QoS parameters so to allow network adjusting to different QoS parameter combinations. See clause 6.1.3.22 in 3GPP TS 23.503. |
| L.3 | Establishment of UE-to-UE QoS bearer with a single API call is considered as a limitation when NEF is used (currently one API call per each UE would be required). Note that SEAL already has support for UE-to-UE QoS bearer establishment with one API call only. | [R-4.2.3-01] The 5G exposure reference points must support: • on-demand UE-to-UE (UNU) or UE-to-data-network (UN) connections with a defined quality of service (QoS) • multiple connections per device, each characterized by QoS parameters • modification of an established UNU or UN connection (e. g. new QoS parameters) and termination of a connection | 3GPP SA2 Rel-18 FS_GMEC has considered to address this limitation, and concluded that the AF needs to support API exposed by SEAL feature which current allows to request UE-to-UE QOS request. When AF or SEAL consumed the API exposed by NEF for provisioning and monitoring UE-to-UE traffic the AF or SEAL shall break the request in 2 UE-to-Network requests managed separately. This limitation will still be there in 3GPP Rel-18 specifications for NEF, and can be a potential candidate for future 3GPP SA2 e.g. Rel-19. |
| L.4 | NEF only supports QoS monitoring for delay. For other QoS parameters, NEF only provides a binary notification about QoS degradation but does not provide additional information. It is considered beneficial to provide an indication about a degraded QoS and in what way it is degraded. | [R-4.2.4-01] The 5G exposure reference points must support monitoring of device connectivity, including the connection’s QoS | 3GPP SA2 Rel-18 FS_GMEC has considered to address this limitation, and concluded that the existing QoS Parameter Notification Control and existing AQP can be used for monitoring of specific performance characteristic. However, this conclusion assumes that the AF supports the AQP mechanism and can derive the degraded QoS and the way it is degraded. Hence providing to AF with an indication about a degraded QoS and in what way it is degraded will still not supported by 3GPP Rel-18 specifications, and can be a potential candidate for future 3GPP SA2 e.g. Rel-19. |
| L.5 | IP address assignment is not provided via the exposure interfaces. It is considered beneficial to expose this feature to Application Functions. | [R-4.2.1-02], Note 2 In a non-public stand-alone network, the 5G NPN and the IIoT application shall be able to use the UE static IP address for identifying a UE. Note 2: The static IP address of the UE is assigned by the IIoT application. This IP address can then be employed to identify the UE via the exposure reference point. | 3GPP specifications cannot support IP address assignment exposure to AF directly via NEF or OAM, however SA2 Rel-16 Vertical_LAN has specified a way to use the exposure interface for AF to assign the IP address indirectly: the AF provisions Information related with secondary authentication / authorization as part of the 5G VN group data when defining the 5G VN group via exposure interface, thus to enable the SMF to get the UE IP address from the DN-AAA server. This limitation will still be open in 3GPP Rel-18 specifications of NEF or OAM, and can be a potential candidate for future 3GPP e.g. Rel-19. |
| L.6 | Fine-grained authorization on the level of specific services and resources, e.g. restricting API requests from specific AFs for specific UEs, is not specified in the standards. | N/A | 3GPP specifications already support authorization within 5GC for NF authorization using the expected NF Service name(s) and optionally "additional scope" information (i.e. requested resources and requested actions (service operations) on the resources), see TS 33.501 clause 13.4.1. And any request for a specific UE is authorized by 3GPP system before to be performed considering the UE subscription per different services, network configuraton, operator policy, etc. 3GPP specifications already specify the authentication and authorization procedure based on vertical services e.g. apply QoS profile X for user UE1 is checked against the UE1 subscription. |

In summary, 3GPP SA considers that  G.4, L.1, L.2 and L.6 have been resolved, and G.1, G.2, G.3, L.3, L.4 and L.5 could be relevant for future 3GPP work. 3GPP SA would like to encourage the member companies of 5G-ACIA to directly engage in the ongoing and future activities in the pertinent 3GPP WGs for further  resolution of the identified gaps and limitations.

2. Actions:

To 5G Alliance for Connected Industries and Automation (5G-ACIA).

ACTION: 	SA respectfully asks 5G-ACIA to take into account the feedback above.

3. Date of Next TSG-SA Meetings:

3GPP SA#99				21 - 24 Mar 2023			Rotterdam, NL

3GPP SA#100-e				13 - 19 Jun 2023				Online

A similar requirement is found in clause 6.10.2 in TS 22.261.

A similar requirement is found in clause 6.10.2 in TS 22.261.