TSG SA Meeting #SP-98E	SP-221126rm_rev1

13 - 19 December 2021, Electronic meeting	(Revision of SP-220814)

# Source:	SA WG2

# Title:	Revised WID: Timing Resiliency and URLLC enhancements (TRS_URLLC)

Agenda Item:	6.3

TSG SA WG2 Meeting #154	S2-2211461

Toulouse, France, 14 - 18 November 2022	(e-mail revision of S2-2210400r03)

(Revision of SP-220814)

# Source:	Nokia, Nokia Shanghai Bell, NTT DOCOMO, Verizon, Huawei, HiSilicon, ZTE, Samsung, KDDI, AT&T, Oracle, NextNav, LG Electronics, Matrixx Software, Ericsson, NICT, Deutsche Telekom

# Title:	Revised WID: Timing Resiliency and URLLC enhancements (TRS_URLLC)

Agenda Item:	10.3

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title:	Timing Resiliency and URLLC enhancements

## Acronym:	TRS_URLLC

## Unique identifier:	970024

## Target Release:	Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940055 | Study on 5G Timing Resiliency and TSC & URLLC enhancements | Corresponding study item |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 880037 | Study on 5G Timing Resiliency System | Stage 1 work item (TR 22.878) |

## 3	Justification

Following are the justifications for the work item objectives:

1) SA1 is specifying requirements for 5G System to remain time resilient if there is GNSS failure and for 5G System to act as a backup and offer wireless and indoor-capable time synchronization service for other applications (e.g. financial, power grid systems).

2) Generic TSC and exposure enhancements to 5GS for IP and ETH applications are needed for the following reasons:

- Current Exposure framework enables AF to request QoS parameters, provide traffic characteristics but not reliability aspect which is important for some IP and ETH applications

3) In order to support low latency and low jitter, enhancements needed to interwork with TSN Transport networks. In such deployments, it may be beneficial to be able to interwork so that the transport network can also provide delay guarantees.

4)  Need for RAN feedback to align the burst transmission from the application.

## 4	Objective

Following are the objectives for this work item:

-	Support for 5G Timing Resiliency requirements defined by SA1.

-	Support reporting 5GS network timing synchronization status (such as divergence from UTC and 5GS network timing source degradation) to UEs and 3rd party applications (AFs):

- 	The ability for RAN and 5GC to learn about network 5GS network timing synchronization status to be able to inform UEs and AFs.

- 	Support providing timing synchronization status to UEs and AFs.

NOTE: RAN impacts are dependent on feedback from RAN WGs and SA3.

-	Enable AFs to request time synchronization service in a specific coverage area and enforce the coverage area.

- Control 5G time synchronization service based on subscription (i.e. introducing subscription parameter for time synchronization and enforcing it).

- Enable an AF to explicitly provide PER to NEF/PCF.

- Support for low latency:

-	Support mechanisms for interworking with TSN transport networks. Interworking mechanisms with TSN networks deployed in the transport network in order to support of E2E determinism and low latency communication and efficient N3 transmission;

-	Support mechanisms for applications to adapt downstream scheduling in order for 5GS to meet really low latency (e.g. 2msecs) requirement and if there is a need to have feedback from RAN (e.g. for application to consider DL packet transmission time slots to avoid buffering in the RAN) for this purpose.

For the listed objectives, this work item will address the normative specifications according to the conclusions agreed in TR 23.700-25 (clause 8).

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Rapporteur(s) (see note 2) | For info at TSG# | For approval at TSG# | Remarks |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.

Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |
| 23.501 | Architectural and functional updates |  |
| 23.502 | Procedural updates |  |
| 23.503 | Policy aspects related updates |  |

## 6	Work item Rapporteur(s)

Devaki Chandramouli, Devaki.chandramouli@nokia.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

Potential RAN impact to be covered by RAN WGs.

Potential security impact to be covered by SA3.

Potential charging and OAM impact to be covered by SA5.

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

NTT DOCOMO

Verizon

Huawei

HiSilicon

ZTE

Samsung

KDDI

AT&T

Oracle

NextNav

LG Electronics

Matrixx Software

Ericsson

NICT

Deutsche Telekom