3GPP TSG-SA2 Meeting # 155 	S2-2303392

# Athens, GR, Februrary 20 - 24, 2023						(revision of S2-2303383, 3360, 2334, 254)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0369 | rev | 4 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing 5G ProSe ph2 function for KI#7 (Support of Emergency for UE-to-Network Relaying) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is a companion CR to TS 23.304 CR0162 to capture the enhanced IMS related functionality concluded in KI#7 (Support of Emergency for UE-to-Network Relaying) of TR 23.700-33 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of IMS emergency service from 5G ProSe Remote UE via 5G ProSe UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex H.5, Annex x (x.1, x.2, x.3) (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 23.304 CR0162 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[2]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; General Packet Radio Service (GPRS); Service description; Stage 2".

[3]	CCITT Recommendation Q.65: "Methodology – Stage 2 of the method for the characterisation of services supported by an ISDN".

[4]	ITU Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[5]	3GPP TS 23.271: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Functional stage 2 description of LCS".

[6]	3GPP TS 25.301: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Interface Protocol Architecture".

[7]	Void.

[8]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[9]	IETF RFC 3825: "Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information".

[10]	IETF RFC 4676: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information ".

[11]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[12]	3GPP TS 23.002: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Network architecture".

[13]	3GPP TS 24.008: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[14]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[15]	OMA AD SUPL: "Secure User Plane Location Architecture", http://www.openmobilealliance.org.

[16]	OMA TS ULP: "User Plane Location Protocol", http://www.openmobilealliance.org.

[17]	NENA I2 architecture: "Interim VoIP Architecture for Enhanced 9-1-1 Services (i2)".

[18]	ETSI ES 282 004: "Protocols for Advanced Networking (TISPAN); NGN Functional Architecture; Network Attachment Sub-System (NASS)".

[19]	3GPP TS 24.229: "IP multimedia call control protocol based on SIP and SDP; stage 3".

[20]	3GPP TS 23.203: "Policy and Charging Control architecture".

[21]	3GPP TS 23.003: "Numbering, addressing and identification".

[22]	Void.

[23]	ANSI/J-STD-036-B: "Enhanced Wireless 9-1-1, Phase 2".

[24]	3GPP2 X.S0002-0: "MAP Location Services Enhancements".

[25]	3GPP2 X.S0060: "HRPD Support for Emergency Service".

[26]	3GPP TS 22.003: "Circuit Teleservices supported by a Public Land Mobile Network (PLMN)".

[27]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) multimedia core network subsystem; Stage 1".

[28]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[29]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SR VCC); Stage 2".

[32]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[33]	3GPP TS 24.301: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[34]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[35]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[36]	ETSI TS 181 019 V2.0.0: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Business Communication Requirements".

[37]	ETSI TS 182 024 v.2.1.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Hosted Enterprise Services; Architecture, functional description and signalling".

[38]	ETSI TS 182 025 v.2.1.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Business trunking; Architecture and functional description".

[39]	3GPP2 X.S0057-C v1.0: "E-UTRAN - eHRPD Connectivity and Interworking: Core Network Aspects".

[40]	NENA 08-002, Version 1.0 (i3): "NENA Functional and Interface Standards for Next Generation 9-1-1".

[41]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[42]	3GPP TS 26.267: "Digital cellular telecommunication systems (Phase 2+); Universal Mobile Telecommunication System (UMTS); eCall data transfer; In-band modem solution General description".

[43]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents".

[44]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[45]	ATIS-0700028: "Location Accuracy Improvements for Emergency Calls".

[46]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services".

[47]	IEEE Std 802.11-2012: "IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[48]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[49]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[50]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[51]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[52]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[x]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

* * * Next Changes * * *

# H.5	Domain Priority and Selection Rules for Emergency Session Attempts

This clause details the domain priority and selection (see clause 7.3) for UE that attempts to make an emergency session for UTRAN, E-UTRAN or NG-RAN radio access networks based on the UE attach status to CS or PS domains and the network support for IMS emergency and IMS voice over PS.

The following table (Table H.1) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an emergency session is initiated and when the UE is not in limited service state.

For NG-eCall (eCall over IMS) domain selection in clause H.6 applies. This clause is not applicable for NG-eCall.

When a 5G ProSe enabled UE (as specified in TS 23.304 [x]) does not have direct connection to the network for emergency service, the UE can obtain emergency service via 5G ProSe UE-to-Network Relay as specified in Annex x.

Table H.1: Domain Selection Rules for emergency session attempts for UTRAN, E-UTRAN or NG-RAN radio access networks

|  | CS Attached | PS Attached | VoIMS | EMS | First EMC Attempt | Second EMC Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | N | Y | Y | Y | PS | CS if available and supported (NOTE 7, NOTE 8) |
| B | N | Y | N | Y | PS or CS if the emergency session includes at least voice. PS if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS (NOTE 7, NOTE 8) |
| C | N | Y | Y or N | N | PS if ESFB is "Y" (NOTE 5). Else CS or PS for another 3GPP RAT with EMS or ESFB set to "Y" if available and supported and if the emergency session includes at least voice. Else PS for another 3GPP RAT with EMS or ESFB set to "Y" if available and supported if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS (NOTE 7, NOTE 8) |
| D | Y | N | Y or N | Y or N | CS if the emergency session includes at least voice. PS if available and EMS or ESFB is "Y" and emergency session contains only media other than voice. | PS if available and EMS or ESFB is "Y" |
| E | Y | Y | Y | Y | If the emergency session includes at least voice, follow rules in TS 22.101 [8] which say to use the same domain as for a non-EMC (NOTE 2) PS if the emergency session contains only media other than voice. | PS if first attempt in CS CS if first attempt in PS |
| F | Y | Y | Y or N | N | PS if ESFB is "Y" (NOTE 5). Else PS for another 3GPP RAT with EMS if available and supported or CS, if the emergency session includes at least voice. | CS if first attempt in PS PS for another 3GPP RAT if available and supported and EMS or ESFB is "Y" if first attempt in CS. |
| G | Y | Y | N | Y | CS if the emergency session includes at least voice. PS if the emergency session contains only media other than voice. | PS |
| EMC = Emergency Session. EMC includes also normal calls initiated in the CS domain that are treated by the CS CN as emergency calls. VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28], TS 23.060 [2] and TS 23.502 [49]. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28], TS 23.060 [2], TS 23.501 [48] and TS 23.502 [49]. ESFB = Emergency Services Fallback for 5GS as defined in TS 23.501 [48] and TS 23.502 [49]. NOTE 1: If the UE selects the CS domain and initiates a normal call using the dialled local emergency number (see clause 7.1.2), and the UE enters limited service state (e.g. due to a Location Registration failing), then the UE camps on an acceptable cell (see TS 23.122 [41]) and may proceed with the EMC by initiating an emergency call in limited service state. NOTE 2: Use of the same domain as for a non-EMC is restricted to UTRAN, E-UTRAN and NG-RAN access (e.g. excludes WLAN). NOTE 3: This NOTE applies to a UE in dual registration mode as defined in TS 23.501 [48]. A dual registration mode UE that is registered to both EPC and 5GC assumes attachment, for the purpose of the "PS Attached" column, to whichever of EPC or 5GC indicates EMS as "Y". When both EPC and 5GC indicate EMS as "Y", the UE shall assume attachment to either EPC or 5GC based on implementation. A UE that is registered to both EPC and 5GC does not use emergency services fallback and ignores the ESFB condition when performing domain selection. NOTE 4: The other 3GPP RAT for row C and row F can be any of UTRA, E-UTRAN connected to EPC, E-UTRA connected to 5GC or NR connected to 5GC that is supported by the UE and differs from the RAT to which the UE is currently attached in the PS domain (or is assumed to be attached based on NOTE 3). NOTE 5: The condition 'ESFB is "Y"' only applies for a UE that is camped on or connected to 5GS via NR or via E-UTRA and that supports Emergency Services Fallback. In that case the emergency call will be provided over E-UTRAN or E-UTRA connected to 5GC as defined in procedures in TS 23.502 [49]. The condition 'ESFB is "Y"' is taken into consideration by the UE only when the network has indicated EMS = "N" for the RAT on which the UE is camping or connected. NOTE 6: For 5GS, the value of the column "EMS" is for the RAT that UE is camped on or is connected to. NOTE 7: As an implementation option, when the first attempt uses PS and fails for reasons other than related to IMS, the second attempt may use PS with a different 3GPP RAT. In this case the UE can make a third attempt using CS. NOTE 8: As an implementation option, when the first attempt uses PS and fails because the first discovered P-CSCF is not available, additional attempts may use PS with alternate P-CSCFs (see clause E.1.1.1 of TS 23.228 [1] and clause 4.3.2.2.1 of TS 23.502 [49]). In this case the UE can make an attempt using CS after making failed PS attempts on all discovered alternate P-CSCFs. |  |  |  |  |  |  |

* * * Next Changes * * *

Annex x (normative):
IMS emergency service from 5G ProSe Remote UE via 5G ProSe UE-to-Network Relay

# x.1	General

For a 5G ProSe enabled UE as specified in TS 23.304 [x], if the UE does not have access to emergency service via any of the IP-CANs in table E.1, the UE (in the role of 5G ProSe Remote UE) may attempt to obtain emergency service via 5G ProSe UE-to-Network Relay if available.

Refer to TS 23.304 [x] for further details how the 5G ProSe Remote UE initiates emergency service via the 5G ProSe UE-to-Network Relay.

# x.2	UE specific behaviour

A 5G ProSe Remote UE identifies that the dialled digits correspond to an emergency call as specified in TS 22.101 [8].

For 5G ProSe Layer-3 UE-to-Network relaying, the 5G ProSe Remote UE may obtain P-CSCF address from the 5G ProSe UE-to-Network Relay via DHCP.

NOTE 1:	Remote UE obtaining P-CSCF address via DHCP is specified in clause 14A.2.1 of TS 24.379 [26].

The 5G ProSe Remote UE sets the access type "3GPP-NR-ProSe-L2UNR" or "3GPP-NR-ProSe-L3UNR" to P-CSCF for the access via 5G ProSe UE-to-Network Relay depending on whether Layer-2 or Layer-3 relaying is applied.

NOTE 2:	The access types are specified in clause 7.2.15.3 of TS 24.229 [19].

# x.3	Location handling

When a 5G ProSe Remote UE performs an emergency registration or initiates an emergency session over a 5G ProSe UE-to-Network Relay, if the operator policy requires network provided location using PCC-based solutions for the UE location, the P-CSCF may retrieve location information from the access network as defined in TS 23.503 [51] and include it in the emergency session request as described in TS 23.228 [1].

NOTE: The user location information contains the 5G ProSe Relay UE’s location.

When a 5G ProSe Remote UE establishes an emergency call over a 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE shall include in the IMS emergency request the NR Cell Global Identity (NCGI) obtained from the 5G ProSe UE-to-Network Relay e.g., during the 5G ProSe UE-to-Network Relay Discovery procedure specified in TS 23.304 [x].

* * * End of Changes * * *