3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301569

# Elbonia, January 16 – 20, 2023		(revision of S2-2301293r01)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0212 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IP address allocataion for communication with a 5G ProSe Layer-3 UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution adds the description of IP address allocation for communication with a 5G ProSe Layer-3 UE-to-UE Relay. Reference for the IP address allocation in other clauses for 5G ProSe Layer-3 UE-to-UE Relay are also corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of IP address allocation mechanism for 5G ProSe Layer-3 UE-to-UE Relay operation into a new clause 5.5.1.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Layer-3 UE-to-UE Relay operation cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1.4 (new); 6.7.1; 6.7.3.2; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.5.1.4	IP address allocation for communication with a 5G ProSe Layer-3 UE-to-UE Relay

For communication with a 5G ProSe Layer-3 UE-to-UE Relay, the following mechanism for IP address/prefix allocation applies:

-	IP address allocation mechanisms of unicast mode of 5G ProSe direct communication as described in clause 5.5.1.1 can be reused on each hop between a 5G ProSe End UE and the 5G ProSe Layer-3 UE-to-UE Relay.

-	The 5G ProSe Layer-3 UE-to-UE Relay may provide the IP address of the target 5G ProSe End UE to the source 5G ProSe End UE in the Direct Communication Accept message, if the target 5G ProSe End UE IP address is available at the time.

-	The 5G ProSe End UE may obtain the IP address of other 5G ProSe End UEs via the 5G ProSe Layer-3 UE-to-UE Relay using DNS query.

NOTE: 5G ProSe Layer-3 UE-to-UE Relay may support 5G ProSe End UEs of different IP versions by performing IP convertions.

# * * * * next change * * * *

### 6.7.1	5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay

Figure 6.7.1-1 shows the procedure for 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay.

Figure 6.7.1-1: 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay

1.	Service authorization and provisioning are performed for source 5G ProSe Layer-3 End UE, target 5G ProSe Layer-3 End UE and 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.2.

2.	The source 5G ProSe Layer-3 End UE performs discovery of a 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.3.2.4.

3.	The source 5G ProSe Layer-3 End UE sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the 5G ProSe Layer-3 UE-to-UE Relay. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Source Layer-2 ID of the DCR message is self-assigned by the source 5G ProSe Layer-3 End UE, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the discovery message of the 5G ProSe Layer-3 UE-to-UE Relay.

4.	If the User Info ID of target 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay and RSC are included in the Direct Communication Request, the 5G ProSe Layer-3 UE-to-UE Relay responds by establishing the security with the source 5G ProSe Layer-3 End UE. When the security protection is enabled, the source 5G ProSe Layer-3 End UE sends IP Address Configuration, QoS Info (as described in clause 6.4.3.1) to the 5G ProSe Layer-3 UE-to-UE Relay.

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the 5G ProSe Layer-3 UE-to-UE Relay, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the source 5G ProSe Layer-3 End UE obtains the 5G ProSe Layer-3 UE-to-UE Relay's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

5.	After the Security Establishment procedure in step 4 is completed, the 5G ProSe Layer-3 UE-to-UE Relay sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the target 5G ProSe Layer-3 End UE. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Source Layer-2 ID of the DCR message is self-assigned by the 5G ProSe Layer-3 UE-to-UE Relay, and the Destination Layer-2 ID is the unicast Layer-2 ID of target 5G ProSe Layer-3 End UE associated with the User Info ID of target 5G ProSe Layer-3 End UE.

6.	If the User Info ID of target 5G ProSe Layer-3 End UE and RSC are included in the Direct Communication Request, the target 5G ProSe Layer-3 End UE responds by establishing the security with the 5G ProSe Layer-3 UE-to-UE Relay. When the security protection is enabled, the 5G ProSe Layer-3 UE-to-UE Relay sends IP Address Configuration, QoS Info (as described in clause 6.4.3.1) to the target 5G ProSe Layer-3 End UE.

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the target 5G ProSe Layer-3 End UE, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the 5G ProSe Layer-3 UE-to-UE Relay obtains the target 5G ProSe Layer-3 End UE's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

7.	The target 5G ProSe Layer-3 End UE sends a Direct Communication Accept message to the 5G ProSe Layer-3 UE-to-UE Relay that has successfully established security with. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

8.	For IP traffic, IPv6 prefix or IPv4 address is allocated for the target 5G ProSe Layer-3 End UE as defined in clause 5.5.1.4.

9.	After receiving the Direct Communication Accept message from the target 5G ProSe Layer-3 End UE, the 5G ProSe Layer-3 UE-to-UE Relay sends a Direct Communication Accept message to the source 5G ProSe Layer-3 End UE that has successfully established security with. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

10.	For IP traffic, IPv6 prefix or IPv4 address is allocated for the source 5G ProSe Layer-3 End UE as defined in clause 5.5.1.4.

11.	For IP communication, the 5G ProSe Layer-3 UE-to-UE Relay may store an association of User Info ID and the IP address of target 5G ProSe Layer-3 End UE into its DNS entries, and the 5G ProSe Layer-3 UE-to-UE Relay may act as a DNS server to other UEs. The source 5G ProSe Layer-3 End UE may send a DNS query to the 5G ProSe Layer-3 UE-to-UE Relay to request IP address of target 5G ProSe Layer-3 End UE after step 10 if the IP address of target 5G ProSe Layer-3 End UE is not received in step 9, and the 5G ProSe Layer-3 UE-to-UE Relay returns the IP address of the target 5G ProSe Layer-3 End UE to the source 5G ProSe Layer-3 End UE.

12.	The source 5G ProSe Layer-3 End UE communicates with the target 5G ProSe Layer-3 End UE via the 5G ProSe Layer-3 UE-to-UE Relay.

Editor's note:	How to support Ethernet and Unstructured traffic is FFS.

In the case of one source 5G ProSe Layer-3 End UE communicates with multiple target 5G ProSe Layer-3 End UEs, the PC5 link between the source 5G ProSe Layer-3 End UE and the 5G ProSe Layer-3 UE-to-UE Relay can be shared for multiple target 5G ProSe Layer-3 End UEs per RSC while the PC5 links may be established individually between the 5G ProSe Layer-3 UE-to-UE Relay and target 5G ProSe Layer-3 End UEs per RSC. For the shared PC5 link, the Layer-2 link modification procedure can be used.

In the case of multiple source 5G ProSe Layer-3 End UEs communicate with one target 5G ProSe Layer-3 End UE, the PC5 link between the 5G ProSe Layer-3 UE-to-UE Relay and the target 5G ProSe Layer-3 End UE can be shared per RSC while the PC5 links may be established individually between the source 5G ProSe Layer-3 End UEs and the 5G ProSe Layer-3 UE-to-UE Relay per RSC. For the shared PC5 link, the Layer-2 link modification procedure can be used.

# * * * * Next change * * * *

#### 6.7.3.2	Procedure

Figure 6.7.3.2-1: 5G ProSe Discovery integrated into PC5 unicast link establishment procedure

0.	UEs are authorized and provisioned with parameters to use the service provided by the UE-to-UE relays. UE-to-UE relays are authorized and provisioned with parameters to provide service of relaying traffic among UEs.

1.	UE-1 wants to establish a unicast communication with UE-2 and broadcasts a Direct Communication Request. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The relay_indication in the Direct Communication Request is used to indicate whether UE-to-UE relay can forward the Direct Communication Request message or not. It is also used to limit the number of hops of UE-to-UE relay by removing relay_indication in the Direct Communication Request message from the UE-to-UE Relay.

NOTE:	The data type of relay_indication can be determined in Stage 3.

2.	When receiving Direct Communication Request with relay_indication from UE-1, Relay (here, relay-1 and relay-2) may decide to participate in the procedure and broadcast a Direct Communication Request message in its proximity without relay_indication. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Direct Communication Request message uses relay's Layer-2 ID as the source Layer-2 ID.

3.	When UE-2 receives a Direct Communication Request from one or multiple UE-to-UE relays, UE-2 select a UE-to-UE Relay which UE-2 will respond. UE-2 may select UE-to-UE relay according to the signal strength, local policy, operator policy per Relay Service Code if any.

4.	security establishment happens between UE-2 and the selected 5G ProSe UE-to-UE Relay (here relay-1), if needed.

Editor's note:	It is FFS how to perform PC5 link modification between UE-2 and relay-1.

5.	UE-2 replies Direct Communication Accept message to relay-1. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

6. For IP traffic, IPv6 prefix or IPv4 address is allocated for the target 5G ProSe Layer-3 End UE as defined in clause 5.5.1.4.

7.	security establishment happens between UE-1 and relay-1, if needed.

Editor's note:	It is FFS how to perform PC5 link modification between UE-1 and relay-1.

8.	Relay-1 responds with Direct Communication Accept to the UE-1. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

9. For IP traffic, IPv6 prefix or IPv4 address is allocated for the source 5G ProSe Layer-3 End UE as defined in clause 5.5.1.4.

Editor's note:	It is FFS how to support non-IP case.

# * * * * End of changes * * * *