3GPP TSG-SA2 Meeting # 155	S2-2303384

# Athens, GR, Februrary 20 - 24, 2023			      (rev of S2-2303361, S2-2302331+02861+02948)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0216 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of multi-path transmission for U2N Relay (KI#5): structure for high level description and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, MediaTek Inc, ZTE, Nokia, Nokia Shanghai Bell,CATT, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To specify the multi-path functionality based on the conclusion of KI#5 (Support of multi-path transmission for UE-to-Network Relay) in TR 23.700-33 v18.0.0, this CR proposes a structure on how to document the functionality as follows: 5. High level functionality and features 5.X Multi-path transmission 5.X.1 General 5.X.2 Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay 5.X.3 Multi-path transmission via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay 6 Functional description and information flows 6.X Multi-path transmission 6.X.1 General 6.X.2 Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay 6.X.3 Multi-path transmission via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce document structure to specify high level feature description and procedures. Provide high level feature description: General and Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-path transmission is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new), 5.X.1 (new), 5.X.2 (new), 5.X.3 (new), 6.X (new), 6.X.1 (new), 6.X.2 (new), 6.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5. X	Multi-path transmission via Uu and via 5G ProSe UE-to-Network Relay

### 5.X.1	General

Multi-path transmission uses one direct network communication path and one indirect network communication path with UE-to-Network Relay as illustrated in Figure 5.X.1-1, where path #1 shows the direct communication path via Uu and path #2 is shows the indirect communication path via 5G ProSe UE-to-Network Relay.

Figure 5.X.1-1: Multi-path transmission via Uu and via 5G ProSe UE-to-Network Relay

The indirect communication path may be via a 5G ProSe Layer-2 UE-to-Network Relay, or via a 5G ProSe Layer-3 UE-to-Network Relay with or without N3IWF.

Editor’s Note: The term "multi-path transmission" is to be revisited.

### 5.X.2	Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay

A 5G ProSe Layer-3 Remote UE may access the network via a 5G ProSe Layer-3 UE-to-Network Relay with or without N3IWF.

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, the multi-path transmission via direct Uu path and via the UE-to-Network Relay is decided by the application layer mechanisms.

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the Layer-3 Remote UE connection via Layer-3 UE-to-Network Relay with N3IWF is considered as "untrusted non-3GPP access to 5GC via N3IWF", therefore the multi-path transmission can be achieved using MA PDU Session in ATSSS features specified in clause 5.32 of TS 23.501 [4].

NOTE:	MA PDU Session requires ATSSS Information in SM subscription data as specified in TS 23.502 [5].

### 5.X.3	Multi-path transmission via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay

Editor’s Note: Alignment with RAN WGs is to be done.

* * * Next Changes * * *

## 6.X	Multi-path transmission via Uu and via 5G ProSe UE-to-Network Relay

### 6.X.1	General

This clause describes the procedures to support multi-path transmission via direct Uu path and via 5G ProSe UE-to-Network Relay.

### 6.X.2	Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, if the UE decides to establish the multi-path via direct Uu path and via the UE-to-Network Relay by evaluating the matched URSP rule for application traffic as specified in clause 6.5.4, the following is performed:

-	The UE sets up path over Uu by establishing a new PDU Session as specified in clause 4.3.2 of TS 23.502 [5] or modifying an existing PDU session as clause 4.3.3 of TS 23.502 [5].

-	The Remote UE establishes 5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay without N3IWF as specified in clause 6.5.1.1.

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, if the UE decides to establish the multi-path via direct Uu path and via the UE-to-Network Relay, the UE establishes a MA PDU Session as specified in clause 4.22 of TS 23.502 [5].

### 6.X.3	Multi-path transmission via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay

Editor’s Note: Alignment with RAN WGs is to be done.

* * * End of Changes * * *