3GPP TSG-WG SA2 Meeting #155	S2-2303320

# Athens, Greece, February 20 – 24, 2023		(revision of S2-2302887)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0224 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General description of 5G ProSe UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution proposes to add General description of 5G ProSe UE-to-UE Relay based on the KI#1 conclusion in TR 23.700-33. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description of 5G ProSe UE-to-UE Relay is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | General description of 5G ProSe UE-to-UE Relay is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.X (new), 4.3.X.1 (new), 4.3.X.2 (new), 4.3.X.3 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

### 4.3.X	5G ProSe UE-to-UE Relay

#### 4.3.X.1	General

Both 5G ProSe Layer-2 and Layer-3 UE-to-UE Relay provides the relaying functionality to support connectivity between 5G ProSe End UEs. 5G ProSe UE-to-UE Relay can be used for both public safety services and commercial services (e.g. interactive service).

Both 5G ProSe Layer-2 and Layer-3 UE-to-UE Relay supports the following functions to enable connectivity between 5G ProSe End UEs:

-	5G ProSe UE-to-UE Relay Discovery service as defined in clause 6.3.2.4, to allow discovery by the 5G ProSe End UE;

-	5G ProSe Discovery integrated into PC5 unicast link establishment as specified in clause 6.7.3;

-	Access 5GS as a UE as defined in TS 23.501 [4] with the enhancements as specified in clauses 6.2;

-	Relay unicast traffic between the 5G ProSe End UEs, supporting IP, Ethernet or Unstructured traffic type.

NOTE:	Relaying groupcast and broadcast traffic to a 5G ProSe End UE by a 5G ProSe UE-to-UE Relay is not supported in this release of the specification.

#### 4.3.X.2	5G ProSe Layer-3 UE-to-UE Relay

In addition to the common 5G ProSe UE-to-UE Relay functionality defined in clause 4.3.X.1, a 5G ProSe Layer-3 UE-to-UE Relay supports the following functions:

-	5G ProSe Direct Communication via 5G ProSe Layer-3 UE-to-UE Relay as specified in clause 6.7.1, for the communication between 5G ProSe Layer-3 End UEs for relay operations;

-	QoS handling and end-to-end QoS treatment for the 5G ProSe Layer-3 End UE's traffic as defined in clause 5.6.3.1;

-	IP address management for the 5G ProSe Layer-3 End UE as defined in clause 5.5.1.4 when the 5G ProSe Layer-3 End UE uses IP traffic type;

-	5G ProSe UE-to-UE Relay reselection as specified in clause 6.7.4.3.

#### 4.3.X.3	5G ProSe Layer-2 UE-to-UE Relay

In addition to the common 5G ProSe UE-to-UE Relay functions defined in clause 4.3.X.1, 5G ProSe Layer-2 UE-to-UE Relay supports the following functions:

-	5G ProSe Direct Communication via 5G ProSe Layer-2 UE-to-UE Relay as specified in clause 6.7.2, for the communication between 5G ProSe Layer-2 End UEs for relay operations;

-	QoS handling and end-to-end QoS treatment for 5G ProSe Layer-2 End UE's traffic as defined in clause 5.6.3.2;

-	5G ProSe UE-to-UE Relay reselection as specified in clause 6.7.4.2.

# * * * * End of changes * * * *