3GPP TSG-SA2 Meeting #155	S2-2303546

# Athens, Greece, Feb 20-24, 2023																revision of S2-2303522

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0689 | rev | 8 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MTLF-based ML Model Accuracy Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, ETRI, vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MTLF-based ML model accuracy monitoring has been agreed in the conclusions of TR 23.700-81 and needs to be implemented in TS 23.288 rel 18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2E (new) introduces MTLF-based ML model accuracy monitoring, and it is further split into: - 6.2E.1 General, to provide overview of 2 procedures - 6.2E.2 Procedure for MTLF-based ML Model Accuracy Mornitoring, to specify the procedure without AnLF assistance On the top of revision 1, futher clarify - that the DataSetTag is defined in Clause 6.2B.1 so to resolve the following EN:. Editor’s Note: The definition and contents of the DataSetTag is FFS. - how to use DataSetTag and resolve the following EN. Editor’s Note: It is FFS how the NWDAF containing MTLF collect the historical data from the ADRF if the AnLF does not provides a DataSettag e.g. the NWDAF containing MTLF determine to monitor ML model accuracy and ML model retraining based on local policy or the previous ML Model subscribtion/request does not include DataSetTag. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MTLF-based ML Model Accuracy Monitoring is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E (new clause) 6.2E.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change – All new text * * * *

## 6.2E	MTLF-based ML Model Accuracy Monitoring

### 6.2E.1	General

MTLF-based ML Model Accuracy Monitoring procedure is where an NWDAF containing MTLF determines ML Model degradation based on newly collected test data and retrain or reprovisioning the existing ML Model.

### 6.2E.2	Procedure for MTLF-based ML Model Accuracy Mornitoring

Figure 6.2E.2-1 illustrates the procedure for monitoring the accuracy of the provisioned ML model using newly collected data. NWDAF containing AnLF may provide inference data to NWDAF containing MTLF for model accuracy monitoring and the NWDAF containing MTLF determines retraining or reprovisioning of the ML model.

Figure 6.2E.3-1: Procedure for MTLF-based ML Model Accuracy Monitoring

1.	The NWDAF containing AnLF requests an ML model from the appropriate NWDAF containing MTLF, using the MLModelProvision_Subscribe service operation. The NWDAF containing AnLF may include an analytics accuracy threshold which is used as an indicator to execute the accuracy monitoring operations., NWDAF containing AnLF may include a DataSetTag (see Clause 6.2B.1) and/or ADRF ID, which is used to store and fetch the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) from ADRF which are relevant for the accuracy monitoring and re-training/re-provisioning of ML model.

Editor’s Note: The call flow only shows a subscription/notification model, however it is FFS how to support request-response model.

2.	The NWDAF containing MTLF provides a trained ML model to the NWDAF containing AnLF. The NWDAF containing MTLF may include an accuracy information which is used to indicate the accuracy of ML model during the training.

3a-3f.	The NWDAF containing MTLF, based on the request(s) from one or more NWDAF containing AnLF or its local policy, determines whether to perform ML model accuracy monitoring and re-training/re-provisioning of ML model by collecting new data from various data sources:

- The NWDAF containing MTLF may collect new data for ML Model Accuracy monitoring, re-training, and re-provisioning from the data source NFs and DCCF by invoking Nnf_EventExposure_Subscribe and Ndccf_DataManagement_Susbscribe service operation, respecrively.

-	When ADRF ID and/or DataSetTag is given by step 1, theNWDAF containing MTLF may retrieve historical data from the ADRF indicated by the NWDAF containing AnLF at step 1. by invoking Nadrf_DataManagementRetrievalRequest or Nadrf_DataManagementRetrieval_Subscribe service operation. Otherwise, the NWDAF containing MTLF may retrieves the historical data from the DCCF or the NWDAF containing AnLF by invoking Ndccf_DataManagement_Subscribe or Nnwdaf_DataManagement_Subscribe service operation, respectively.

-	If the NWDAF containing AnLF does not include a DataSetTag with ADRF ID at step 1, the NWDAF containing MLTF may request ADRF to subscribe for the collection of the analytics and data that correspond to the analytics generated by the ML model provisioned in step 2, using the procedures defined in clause 6.2B.3.

-	The NWDAF containing MTLF may subscribe to UDM to get notification on change in the subscription data for Target of ML Model Reporting by invoking Nudm_SDM_Subscribe service operation, and the UDM subscribes to the UDR to get notifications of the modification on UE subscription data by invking Nudr_DM_Subscribe service operation.

- The NWDAF containing MTLF may consider the data quality into the accuracy monitoring by collectting fault prediction analytics data from MDAS to determine the status of Data Source NFs, using MDA Request.

In case the NWDAF containing MTLF has already collected new test data and performed ML model accuracy monitoring and retraining which is triggered by other NWDAF containing AnLF(s) (for ML model accuracy monitoring and retraining), the NWDAF containing MTLF, based on its internal logic, determines whether to use the data for the subscription or not.

4a-4f.	The NWDAF containing MTLF receives the requested data from various sources as requested in step 3a-3f.

5.	Based on the collected analytics and data from step 4a-4f, the NWDAF containing MTLF computes the accuracy using the predictions and the actual measured data observed at the time which the prediction refers to. The NWDAF containing MTLF discards data from data sources if it detects the data quality of that source is not good. The NWDAF containing MTLF may generate prediction with the collected input data to calculate the accuracy if only input data and ground truth data are available.

6.	An accuracy report is sent to the NWDAF containing AnLF, e.g., when the reporting threshold is met by invoking Nnwdaf_MLModelProvision_Notify service operation.

Editor’s Note: How the accuracy report is requested by and delivered to AnLF is FFS.

7.	Based on the computed accuracy, the NWDAF containing MTLF may decide to re-train/re-provision the ML model.

8.	When the newly generated or re-trained ML model is ready, the NWDAF containing MTLF sends new or re-trained ML model to the NWDAF containing AnLF by invoking Nnwdaf_MLModelProvision_Notify service operation. The NWDAF containing MTLF may send the accuracy report of the new or re-trained ML model to the NWDAF containing AnLF.

* * * * End of Changes * * * *