3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300960

# Elbonia, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-58 | CR | 0002 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update solution#5 that DAA payload is carried by PC5-U message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solution#5 has been concluded as key issue 2 and key issue 3 baseline solution. However, the procedure of solution are confusing in step 1 and step 6. Step 1 assumes the DAA paypload is generated by application layer. The payload is transmitted by a PC5-U message, according to existing design. However step 6 writes it can be a PC5-S message, which is not aligned with step 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update step 6, changing the DAA boardcast message from PC5-S to PC5-U message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-alignment in step 1 and step 6 message in solution 5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.5.5.1	Broadcast over PC5 for DAA deconfliction

Figure 6.5.5.1-1: Broadcast over PC5 for DAA deconfliction

Pre-requisite: the UAV are provisioned with a the U2X Policy which includes a DAA deconflicting policy (e.g. unicast or broadcast communication for deconflicting, communication frequency).

1.	UAV1 receives broadcast messages from UAV2, that may include application layer DAA payload, e.g. CAA-level UAV ID, UAV2's USS address, velocity, heading direction, position.

NOTE 1:	The USS address is not needed if the conflict is resolved locally between UAVs, but may be helpful if it requires the coordination of the USSes of the UAVs involved in the conflict. The interaction between the USSes for DAA deconfliction is out of scope of this study.

NOTE 2:	The exact content of DAA payload is out of scope of this study.

2.	UAV1's passes the DAA payload to the upper layer. The application layer detects a conflict, based on the broadcast messages received from UAV2, e.g. by comparing it with its own trajectory and location. If the application layer in the UAV1 detects a collision, it initiates a collision avoidance/conflict resolution procedure with UAV2.

3.	Optionally, UAV1 may inform its own USS about the detected collision by including peer UAV 2s' ID(s).

4.	. UAV1 selects a communication mode (broadcast or unicast) for DAA deconfliction based on the input received from the application layer and DAA policy. If broadcast deconfliction method is selected then the following messages are exchanges between the UAVs.

5.	UAV1 broadcasts a message (e.g. PC5-U message), e.g. deconfliction request message and may include DAA capability, which is part of U2X capability and indicates whether the UAV is able to engage in communication for deconflicting protocol, DAA deconflicting policy (broadcast based, deconflicting message frequency), collision detection alert, its CAA-level UAV IDs and the one(s) from other detected conflicting UAV(s), and deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

NOTE 3:	The deconflicting specific parameters are application layer content and is out of scope of this study.

6.	UAV2 broadcasts a message (e.g. PC5-U message), to provide agreed DAA deconflicting policy, its updated trajectory and other info, e.g. message deconfliction status response, conflict resolved alert, CAA-level UAV IDs of participating UAVs from the receiving UAV. Subsequent broadcast messages can be exchanged between UAVs until traffic conflict resolution is reached (e.g. for mutual position/trajectory monitoring) based on agreed message frequency.

# * * * * End of changes * * * *