3GPP TSG-SA5 Meeting #147	S5-233069

Athens, Greece, 27th Feb 2023 - 3rd Mar 2023                   Revised by S5-232278

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0019 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel17 CR TS 28.105 correct attribute definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 28.105, the definition and description of some attributes needs updated. - In clause 7.3.3, the support qualifier of attribute modelPerformanceTraining and areNewTrainingDataUsed has some misunderstanding. For attribute modelPerformanceTraining, since performance evaluation can be performed in producer, the performance information can also be included in traning report. For attribute areNewTrainingDataUsed, since new training data could also be used in case of "AI/ML training initiated by consumer". Therefore, the attribute modelPerformanceTraining and areNewTrainingDataUsed are applicable to both "AI/ML training initiated by consumer" and "AI/ML training initiated by producer" scenarios. - In clause 7.4.3, the MLContext is defined and it may be one of three types of context including ExpectedRunTimeContext, TrainingContext and RunTimeContext. According to the attributes defnition in Table 7.5.1-1, TrainingContext is applicable only when MLEntity has been trained, so inferenceEntityRef is not applied to this kind of context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 7.3.3, the Support Qualifier of attribute modelPerformanceTraining and areNewTrainingDataUsed in Table 7.3.3.2-1 is changed to be "M". In clause 7.4.3, the Support Qualifier of attribute inferenceEntityRef in Table 7.4.3.2-1 is changed to be "CM". The corresponding attribute constraints is added in clause 7.4.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors and misleading of attributes definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3, 7.4.3, 7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

### 7.3.3	MLTrainingReport

#### 7.3.3.1	Definition

The IOC MLTrainingReport represents the ML model training report that is provided by the training MnS producer.

The MLTrainingReport MOI is contained under one MLTrainingFunction MOI.

#### 7.3.3.2	Attributes

Table 7.3.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLEntityId | M | T | F | F | T |
| areConsumerTrainingDataUsed | M | T | F | F | T |
| usedConsumerTrainingData | CM | T | F | F | T |
| confidenceIndication | O | T | F | F | T |
| modelPerformanceTraining | M | T | F | F | T |
| areNewTrainingDataUsed | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingProcessRef | M | T | F | F | T |
| lastTrainingRef | CM | T | F | F | T |

#### 7.3.3.3	Attribute constraints

Table 7.3.3.3-1

| Name | Definition |
| --- | --- |
| usedConsumerTrainingData Support Qualifier | Condition: The value of areConsumerTrainingDataUsed attribute is ALL or PARTIALLY. |
| trainingRequestRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was requested by the MnS consumer (via AIMLTrainingRequest MOI). |
| lastTrainingRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was not initial training (i.e. the model has been trained before). |

#### 7.3.3.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

2nd modified section

### 7.4.3	MLContext <<dataType>>

#### 7.4.3.1	Definition

The MLContext represents the status and conditions related to the MLEntity. Specially it may be one of three types of context - the ExpectedRunTimeContext, the TrainingContext and  the RunTimeContext.

#### 7.4.3.2	Attributes

Table 7.4.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceEntityRef | CM | T | F | F | F |
| dataProviderRef | M | T | F | F | F |

#### 7.4.3.3	Attribute constraints

Table 7.4.3.3-1

| Name | Definition |
| --- | --- |
| inferenceEntityRef Support Qualifier | Condition: The MLContext is used for ExpectedRunTimeContext or RunTimeContext. |

#### 7.4.3.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

End of modified section