3GPP TSG-SA5 Meeting #147 	S5-232056

# Athens (GR),  27 Feb. - 03 Mar. 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0026 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct measurement used for eMBB and URLLC EE KPIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE5GPLUS |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The measurement GTP.OutDataOctN3UPF, used in the eMBB and URLLC EE KPI definitions, is wrongly described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct how the measurement GTP.OutDataOctN3UPF is described. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible wrong implementations of the eMBB and URLLC EE KPIs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.3, 6.1.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR mirrors CR#0025. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

#### 6.1.2.3	Energy efficiency of eMBB network slice

This KPI is obtained by the sum of UL and DL data volumes at N3 interface(s) of the network slice, divided by the energy consumption of the network slice. The unit of this KPI is bit/J.

The following measurements are used:

- GTP.InDataOctN3UPF.SNSSAI: Number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.4.1.3;

- GTP.OutDataOctN3UPF.SNSSAI: Number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.4.1.4.

Next Change

##### 6.1.2.4.3	Based on both latency and Data Volume (DV) of the network slice

This KPI is obtained by the product of the sum of UL and DL traffic data volumes at N3 interface(s) of the network slice multiplied by the inverse of the end-to-end User Plane (UP) latency of the network slice, divided by the energy consumption of the network slice. The unit of this KPI is bit/(0.1ms*J).

In this KPI variant, data volume and latency are two factors considered for evaluating the performance of network slice, i.e. the performance of network slice (Pns) for URLLC type of network slice is the sum of UL and DL traffic data volumes at N3 interface(s) multiplied by the inverse of the end-to-end User Plane (UP) latency of the network slice. This solution is applicable for the case of URLLC network slice is deployed and operators want to evaluate the slice EE KPI for different periods of time such as busy time slots (more DV) and idle time slots (less DV).

The following measurements, defined in TS 28.552 [15], are used to calculate the sum of UL and DL traffic data volumes at N3 interface(s) of the network slice:

- GTP.InDataOctN3UPF.SNSSAI: Number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.4.1.3;

- GTP.OutDataOctN3UPF.SNSSAI: Number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN), where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.4.1.4.

The following KPIs, defined in TS 28.554 [18], are used to calculate the latency of the network slice:

- DelayE2EUlNs: Average e2e uplink delay for a network slice, defined in TS 28.554 [18] clause 6.3.1.8.1 as the average e2e UL packet delay between the PSA UPF and the UE for a network slice;

- DelayE2EDlNs: Average e2e downlink delay for a network slice, defined in TS 28.554 [18] clause 6.3.1.8.2 as the average e2e DL packet delay between the PSA UPF and the UE for a network slice.

End of Change