3GPP TSG-SA5 Meeting #147 	S5-232568

Athens, Greece, 27th February - 3rd March 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0111 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of integrity KPIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The upstream and downstream throughput for network and network slice instance KPI is defined as kbit/s while the calculation sums the octets. The upstream and downstream GTP data throughput at N3 interface KPI is not calculated for per network slice but per UPF, it sums octets and divides it by an undefined “time period”. The values calculated will be a mean value of the throughput over the granularity period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add conversion from octets to bits (times by 8). Change naming of upstream and downstream GTP data throughput at N3 interface KPI to reflect that it’s not per network slice. Add division by granularity period in milliseconds, also used in TS 28.552. Changing the type of KPI to MEAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KPIs will not be clearly specified which may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3, 6.3.4 and 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 6.3.2	Upstream throughput for network and Network Slice Instance

a)	Upstream throughput for network and network slice instance.

b)	This KPI describes the upstream throughput of one single network slice instance by computing the packet size for each successfully transmitted UL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by measuring the total number of upstream octets provided by N3 interface from NG-RAN to all UPFs, related to the single network slice instance, divided by the granularity period (in milliseconds).

d)	UTSNSI= UPFGTP.InDataOctN3UPF.SNSSAIGranularityPeriod×8UTSNSI=UPFGTP.InDataOctN3UPF

e)	GTP.InDataOctN3UPF.

f)	5GS.

g)	Integrity.

h)	kbit/s.

i)	MEAN.

Second change

### 6.3.3	Downstream Throughput for Single Network Slice Instance

a)	Downstream throughput for network and network slice instance.

b)	This KPI describes the downstream throughput of one single network slice instance by computing the packet size for each successfully transmitted DL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by measuring the total number of downstream octets provided by N3 interface from all UPFs to NG-RAN, related to the single network slice instance, divided by the granularity period (in milliseconds).

d)	DTSNSI= UPFGTP.OutDataOctN3UPF.SNSSAIGranularityPeriod×8

e)	GTP.OutDataOctN3UPF.

f)	5GS.

g)	Integrity.

h)	kbit/s.

i)	MEAN.

Third change

### 6.3.4	Upstream Throughput at N3 interface

a)	Upstream GTP data throughput at N3 interface.

b)	This KPI describes the total number of octets of all incoming GTP data packets on the N3 interface (measured at UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate upstream GTP throughput integrity performance at the N3 interface.

c)	This KPI is obtained by measuring the total number of octets GTP data packets upstream provided by N3 interface from NG-RAN to UPF, divided by the granularity period (in milliseconds).

d)	UGTPTN= GTP.InDataOctN3UPFGranularityPeriod×8

e)	GTP.InDataOctN3UPF

f)	5GS

g)	Integrity

h)	kbit/s

i)	MEAN

Fourth change

### 6.3.5	Downstream Throughput at N3 interface

a)	Downstream GTP data throughput at N3 interface.

b)	This KPI describes the total number of octets of all downstream GTP data packets on the N3 interface (transmitted downstream from UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate integrity performance at N3 interface.

c)	This KPI is obtained by measuring the total number of octets GTP data packets downstream provided by N3 interface from UPF to NG-RAN, divided by the granularity period (in milliseconds).

d)	DGTPTN=  GTP.InDataOctN3UPFGranularityPeriod×8

e)	GTP.OutDataOctN3UPF

f)	5GS

g)	Integrity

h)	kbit/s

i)	MEAN

End of changes