| 3GPP TR 22.843 V1.0.0 (2023-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group TSG SA; Study on Uncrewed Aerial Vehicle (UAV) Phase 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	8

4	Overview	8

5	Use cases	8

5.1	Use case on UAV detection	8

5.1.1	 Description	8

5.1.2	 Pre-conditions	8

5.1.3	 Service Flows	8

5.1.4	 Post-conditions	9

5.1.5	 Existing features partly or fully covering the use case functionality	9

5.1.6	 Potential New Requirements needed to support the use case	9

5.2	Use case on supporting UAV pre-flight preparation	9

5.2.1	 Description	9

5.2.2	 Pre-conditions	9

5.2.3	 Service Flows	10

5.2.4	 Post-conditions	10

5.2.5	 Existing features partly or fully covering the use case functionality	10

5.2.6	 Potential New Requirements needed to support the use case	10

5.3	Use case: Geofencing for Visual Line-of-Sight UAV missions	11

5.3.1	 Description	11

5.3.2	 Pre-conditions	11

5.3.3	 Service Flows	11

5.3.4	 Post-conditions	11

5.3.5	 Existing features partly or fully covering the use case functionality	12

5.3.6	 Potential New Requirements needed to support the use case	12

5.4.	Use case on network-assisted UAV DAA	12

5.4.1	 Description	12

5.4.2	 Pre-conditions	12

5.4.3	 Service Flows	12

5.4.4	 Post-conditions	13

5.4.5	 Existing features partly or fully covering the use case functionality	13

5.4.6	 Potential New Requirements needed to support the use case	13

5.5	Use case of the 3GPP network as an information source to the UTM	14

5.5.1	 Description	14

5.5.2	 Pre-conditions	15

5.5.3	 Service Flows	15

5.5.4	 Post-conditions	16

5.5.5	 Existing features partly or fully covering the use case functionality	16

5.5.6	 Potential New Requirements needed to support the use case	16

5.6	Use case on supporting UAV inflight operations	16

5.6.1	 Description	16

5.6.2	 Pre-conditions	17

5.6.3	 Service Flows	17

5.6.4	 Post-conditions	17

5.6.5	 Existing features partly or fully covering the use case functionality	18

5.6.6	 Potential New Requirements needed to support the use case	18

5.7	Use case on UAV flight route tracking at Rendezvous points	18

5.7.1	Description	18

5.7.2	Pre-conditions	19

5.7.3	Service Flows	19

5.7.4	Post-conditions	20

5.7.5	Existing features partly or fully covering the use case functionality	20

5.7.6	Potential New Requirements needed to support the use case	20

6	Consolidated potential requirements	20

7	Conclusion and recommendations	20

Annex A (informative): Change history	20

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document provides additional use cases of UAV and identifies potential requirements to improve 5G system’s support of UAV applications, drone operations and management, including:

- Provide additional information to the UAV operator/USS to execute pre-flight preparations and inflight operation (e.g, flight mission application, flight path recommendation, flight monitoring and control);

- Use 5G system to support enhancing the UAV flight/route management based on network capacity and QoS information along the planned route;

- Use 5G system to further enhance the safety and security of drone operations, e.g. supporting UTM in UAV    detection and DAA(Detect and Avoid);

- Identify potential new security, charging and regulatory requirements on 5GS, when used for UAS operation.

- Identify potential new requirements related to redundancy and reliability of command and control (C2) traffic for UAV.

For the identified use cases, a gap analysis is performed between potential new service requirements and existing 3GPP requirements and functionalities.

NOTE:	This study investigates requirements for additional 5GS capabilities to assist UTM, with the assumption that   liability and responsibility for drones operation remain under the domain of the UTM/drone operator.

NOTE:	Potential overlaps with ongoing stage-2 work (on UAS), and other S1 studies (e.g. FS_Sensing) have been    considered and avoided.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	Global UTM Association UTM Architecture v1: https://www.gutma.org/docs/Global_UTM_Architecture_V1.pdf

[3]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP".

…

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BRID	Broadcast Remote Identification

BVLOS	Beyond Visual Line of Sight

# 4	Overview

The present document focuses on improving the 3GPP support for the applications and security for various scenarios using low altitude UAVs. The present document provides potential use cases and defines potential new service level requirements and KPIs for supporting identified use cases by 3GPP system.

# 5	Use cases

## 5.1	Use case on UAV detection

### 5.1.1		Description

3GPP aerial features are activated based on "aerial subscription" flag specified in Rel-15. This implies that an operator using these features will provide drone users with USIMs that are associated to a user profile with the "aerial subscription" flag set.

It is however technically possible for a drone user to insert a USIM not meant for UAV in a UE that is on board a UAV (this UE being used for C2C or for other purpose). The user could do that either intentionally (e.g. to have a lower tariff or to bypass some restrictions applied to UAVs) or not (e.g. because he/she is not aware that UAVs need to be used with specific USIMs).

In this case, with current specifications, the 3GPP network is not able to recognize that the UE is on board a UAV and therefore cannot activate aerial features. More importantly, the 3GPP network will not perform UAV Authentication and Authorization procedures with the UTM, and may cause the UE to use a frequency band that is not allowed for aerial usage where the UAV is located due to the risk of interference to users or adjacent frequency bands (e.g. military installation, weather radar). This has security implications. In addition, the 3GPP network cannot fulfill the requirements of TS 22.125 applicable to UE on board a UAV.

From Rel-17 onwards, the 3GPP network is able to reject registration when the UE provides a "CAA-level UAV ID" and does not have the "aerial subscription" flag set in its subscription. This mitigates the above issue, but the issue still exists if the UE is pre-R17 or is not configured with a CAA-level UAV ID.

There is therefore a need to enable the 5G System network to detect that a UE is on board a UAV, without relying on subscription information or on indication provided by the UE.

### 5.1.2		Pre-conditions

A UAV user has inserted in a UE on board a UAV, a USIM that is associated to a user profile without the "aerial subscription" flag set.

### 5.1.3		Service Flows

1.	The UE registers in 5GS and starts communicating. The UAV takes off.

2.	The network detects that the UE is on board a UAV.

3.	The network takes action, e.g. activates aerial features, changes the QoS, populates charging records, alert the UTM and/or let the UTM take over the communication used to control the UAV.

### 5.1.4		Post-conditions

The UTM may take over the communication used to control the UAV and force the UAV to land safely.

Later, the UTM and/or network operator informs or reminds the user that a specific USIM needs to be used for proper operation of a UE on board a UAV.

### 5.1.5		Existing features partly or fully covering the use case functionality

-	"aerial subscription" flag in subscription.

-	Provision of "CAA-level UAV ID" by the UE.

-	detection of UE above altitude thresholds by the eNB / gNB (per TS 36.331 & 38.331)

### 5.1.6		Potential New Requirements needed to support the use case

[PR 5.1.6-1] The 5G system shall be able to detect that a connected UE is airborne, while UE’s subscription does not include “aerial.

## 5.2	Use case on supporting UAV pre-flight preparation

### 5.2.1		Description

During the operation of UAVs using the 3GPP network access, the network needs to ensure that the UAVs can access the network smoothly and timely. And UAV flight mission need to be ensured for the real-time performance and stability of flight control link to guarantee the continuity of the flight.

5G network can provide wide-area, high-quality and secure connectivity that can enable cost-efficient drone operations beyond visual line-of-sight (BVLOS) range. In some industrial UAV scenarios, the UAV would be provided pre-scheduled flight plan for autonomous flying. When UAV uses 3GPP network as the access type, the UTM/drone operator may need to acknowledge the network status for flight path scheduling with the help of 3GPP network .

### 5.2.2		Pre-conditions

In the logistics company, UAV A and UAV B are both individual terminals for express delivery and subscribe the communication service from 5G network.

Operator A provides 5G network coverage and communication services for remote commands and control (C2) communication between UAV and UAV control platform .

The UTM has good cooperation with Operator A to have UAV-related information which is fetched from 5G network.

### 5.2.3		Service Flows

Figure 5.2.3-1: Supporting UAV pre-flight preparation

1.	The logistics company plans to use UAV A to deliver express from X to Y and use UAV B to deliver express from X to Z.

2.	UAV/UAV-C A and UAV/UAV-C B register in 5G network and can communicate with UTM though 5G network.

3.	The UAV-C provides planned flight path information to UTM (e.g. flight path and required QoS along the planned flight path) .

4.	UTM requests 5G network for QoS information/predictions/statistics along the flight path or alternative flight paths..

5.	The 5G network provides the requested statistics/predictions (e.g. end-to-end latency, QoE) for specific requested area/areas.

6.	UTM/UAS determines the final flight paths based on the information from Operator A and sends to UAU/UAV-C A and UAV/UAV-C B separately.

7.	UAV A and UAV B receive the flight path information from UTM/UAS via 5G network and start flight mission.

### 5.2.4		Post-conditions

UAV A and UAV B can finish their delivery tasks with the support of 5G network.

### 5.2.5		Existing features partly or fully covering the use case functionality

None.

### 5.2.6		Potential New Requirements needed to support the use case

[P.R 5.2.6-001] Based on operator’s policy, the 5G system shall be able to provide a method to monitor and predict network condition (e.g. bitrate, latency, reliability) along the UAV continuous geographic path provided from UTM.

[P.R 5.2.6-002] The 5G system shall  be able to provide UTM with the information obout geographic areas where UAV service requirements could be met based on predicted QoS and network conditions (e.g. bitrate, latency, reliability) .

## 5.3	Use case: Geofencing for Visual Line-of-Sight UAV missions

### 5.3.1		Description

3GPP has specified a wide range of aerial features, activated through an "aerial subscription". Three types of C2 communications have been defined for R17, in TS.22.125:

- Direct C2 communications (when the UAV controller and UAV establish a direct C2 link to communicate with each other)

- Network-Assisted C2 communications (when the UAV controller and UAV communicate with each other via 5G network)

- UTM-Navigated C2 communication (for pre-scheduled autonomous flights)

3GPP features for Network-Assisted C2 communications and UTM-Navigated C2 communication mostly cover Beyond Visual Line-of-Sight (BVLoS) UAV operations, and Direct C2 communications are constrained by their radio coverage range, which is generally much wider than the VLoS constraints imposed by airspace authorities.

In all three cases, it is thus technically possible that an UAV controller does not comply with airspace regulatory requirements related to VLoS operations.

Ensuring that an aerial UE is complying with regulatory Visual Line-of-Sight constraints, by supporting pre-flight preparations and inflight operation flight monitoring and control, enhances the safety and security of drone operations.

There is a need to enable the 5G system to be aware if VLoS constraints should be applied to UAV operations, and if so, to monitor the relative positions of an aerial UE with respect to its pilot to ensure that this aerial UE is complying with Line-of-Sight constraints.

NOTE:	The pilot, or UAV-C, can be an EU (connected to the same PLMN or to another PLMN), or not, but remains attached to the DN using other technologies..

### 5.3.2		Pre-conditions

The UAV is associated with an aerial UE profile and a “VLoS” option is activated together with its subscription. The UAV is associated with an UAV Controller (UAV-C). The 5G system is informed by the UTM of the maximal distance authorized between the UAV and the UAV-C (VLoS distance threshold, usually around 200 or 300m), which usually depends on airspace regulation and may potentially account for other conditions (e.g. weather, type of environment, type of UAV, temporary or localized special authorization, etc.).

### 5.3.3		Service Flows

1.	The UAV registers in 5GS as an aerial UE with the VLoS option activated.

2.	The UAV is associated with an UAV-C. The UAV takes off.

3.	The network tracks the geolocalization of both the UAV and UAV-C, and deduces the distance between both. If the UAV-C does not belong to the 3GPP system, its geolocalization may be obtained, for example, from the UTM/USSP.

4.	In case this distance exceeds the VLoS threshold, the network takes action, e.g. alerts the UTM and/or lets the UTM takes over the communication used to bring back the UAV within the authorized VLoS constraints, or sends a new waypoint within the authorized VLoS constraints.

### 5.3.4		Post-conditions

The UTM may take over the communication used to control the UAV and force the UAV to land safely.

Later, the UTM and/or network operator informs or reminds the user that a specific USIM needs to be used for proper operation of a UE on board a UAV.

### 5.3.5		Existing features partly or fully covering the use case functionality

-	"Aerial subscription" option in subscription, as specified in Rel-15.

-	UAV / UAV-C association, as in TS 23.256 for example.

-	Tracking of the geolocalization of the UAV as in TS 23.256 for example.

### 5.3.6		Potential New Requirements needed to support the use case

[PR 5.3.6-1] The 5G system shall be able to be informed of VLoS regulatory requirements (e.g. maximal distance threshold) and shall be informed if the UTM needs to be supported in meeting such requirements for a given UAS.

[PR 5.3.6-2] The 5G system shall be able to track the UAV-Controller, regardless of the type of connection.

Editor’s Note:	this requirement is FFS

## 5.4.	Use case on network-assisted UAV DAA

### 5.4.1		Description

Collection of information for UAV detection and avoidance in order to support tactical deconfliction is one of the key functions to guarantee UAV service safety. Previous studies on 3GPP mostly focused on the aspect that the detection and resolution of collisions is locally performed between UAVs using direct UAV to UAV communication over PC5. However, UAV to UAV DAA is only one component of the overall solution and, as in current aviation system, support by ground systems for tactical deconfliction and collision avoidance is an essential complement. With the development of network capabilities for the support of UAVs, new system capabilities like sensing in the 3GPP network can also effectively assist the UAV DAA scenario to ensure the flight safety by providing a ground-assisted component.

### 5.4.2		Pre-conditions

UAV A and UAV B are 3GPP aerial UEs and subscribe the communication service from 5G network.

UAV C is an aerial vehicle not capable of 3GPP communications, with C2 provided by a non-3GPP UAV controller and communication with USS/UTM directly.

UAV A and UAV B can setup remote commands and control (C2) communication to a UAV controller within 5G network with UTM.

### 5.4.3		Service Flows

Figure 5.4.3-1: Network-assisted UAV DAA

1.	UAV A and UAV B register in 5G network and can setup communications with UTM though 5G network. UAV C setup communicate with UTM directly.

2.	The UTM subscribes network-assisted UAV DAA service from Operator M.

3.	Operator M could use base station(s) to perform sensing to collect information about the UAV A, B and C in certain area and provide the collected information to UTM for DAA operation.

4.	Operator M could also collect information from UAV A and UAV B about the flight status and reports to UTM.

5.	UTM could trigger DAA notifications for the flying UAV A, B and C under its area based on the information from network.

6.	UAV A, B and C receive the notification and take actions accordingly.

### 5.4.4		Post-conditions

UAV A, B and C can fly safely under the control of UTM with the support of 5G network.

### 5.4.5		Existing features partly or fully covering the use case functionality

SA1 has performed study on UAV DAA in previous releases, where related normative stage 1 requirements are introduced in TS 22.125 [3].

TS 22.125 clause 6.2 introduces the network support for UAV detection as below:

[R-5.2.1-001] The 3GPP system shall provide a mechanism for a UTM to provide route data, along with flight clearance, to a UAV.

[R-5.2.1-002] The 3GPP system shall be able to deliver route modification information received from a UTM to a UAS with a latency of less than 500ms.

[R-5.2.1-003] The 3GPP system shall be able to deliver the notifications received from a UTM to a UAV controller with a latency of less than 500ms.

[R-5.2.1-004] Based on MNO policies and/or regulatory requirements, the 3GPP system shall enable the UTM to take over the communication used to control the UAV.

TS 22.125 clause 5.2 introduces the introduces network support for UAV avoidance as below:

[R-6.2-001] The 3GPP system shall provide means to allow a 3rd party to request and obtain real-time monitoring the status information (e.g., location of UAV, communication link status) of a UAV.

[R-6.2-002] Based on operator 's policy, the 3GPP system shall provide means to provide a 3rd party with the information regarding the service status for UAVs in a certain geographical area and/or at a certain time.

### 5.4.6		Potential New Requirements needed to support the use case

[P.R 5.4.6-001] Based on operator’s policy, the 5G system shall be able to provide a method to provide UTM with the information collected or generated by 5G system regarding to the relative distance between 3GPP and/or non-3GPP UAVs (based on sensing result), to support network-assisted DAA.

[P.R 5.4.6-002] Based on operator’s policy, the 5G system shall be able to provide a method to provide UTM with the information collected or generated by 5G system regarding to location of non-3GPP UAVs (based on sensing result) , to support network-assisted DAA.

Editor’s Note:	Both requirements are for FFS.

## 5.5	Use case of the 3GPP network as an information source to the UTM

### 5.5.1		Description

The UTM is a complex system in which actors outside the 3GPP System work to ensure the required safety level of UAS operations. For this reason, the UTM is defined by GUTMA (Global UTM Association) as “a system of stakeholders and technical systems collaborating in certain interactions, and according to certain regulations, to maintain safe separation of unmanned aircraft, between themselves and from ATM users, at very low level, and to provide an efficient and orderly flow of traffic”

A system like UTM requires a high-quality set of data flows to remain fully aware of the system it is operating within and to effectively meet the mission brief. In figures 1 and 2 below, the GUTMA architecture [2] shows that a high number of data sources are considered to keep the UTM functional.

Figure 5.5.1-1: A view of a UTM system (source: GUTMA Global UTM Architecture v1 [2])

Figure 5.5.1-2: NASA UTM system (source: GUTMA Global UTM Architecture v1 [2])

The 3GPP System can act as a data source towards the UTM in order to provide spatial, surveillance, meteorological, and real-time positioning information,

### 5.5.2		Pre-conditions

A UTM exists which is capable of accepting a 3GPP network as data source. A partner 3GPP operator is prepared to supply information to the UTM based on the capabilities of the network. Agreements on trust, responsibility, liability, privacy, and data management have been made.

### 5.5.3		Service Flows

- A UTM determines what data is required from the 3GPP network which can act as an information source.

- Based on the capabilities of the 3GPP network, an operator begins to gather data according to agreements made with a UTM operator where the UTM provides the type of information and optional conditions regarding such information (e.g. reporting for a specific area, reporting specific events, etc.). This data is gathered from a number of sources which may include the following: RAN awareness of UE altitude (detect UEs above a certain altitude from the ground), Wireless Sensing results, UE registration, UE location reporting, network-based UE location determination.

- The operator begins to feed data towards a UTM according to the information requested by the UTM.

- The UTM accepts the data from the operator and is responsible for processing (corelating data sets, identifying objects, deduplication) and acting on (warnings, notifications) this data according to mechanisms outside the scope of 3GPP.

### 5.5.4		Post-conditions

The UTM has an increased situational awareness because of the increased data provided by the 3GPP network.

### 5.5.5		Existing features partly or fully covering the use case functionality

Related requirements in TS 22.125:

[R-5.1-003] The 3GPP system shall enable a UAS to send UTM the UAV data which can contain: unique identity (this may be a 3GPP identity), UE capability of the UAV, make & model, serial number, take-off weight, position, owner identity, owner address, owner contact details, owner certification, take-off location, mission type, route data, operating status.

[R-5.1-004] The 3GPP system shall enable a UAS to send UTM the UAV controller data which can contain: unique identity (this may be a 3GPP identity), UE capability of the UAV controller, position, owner identity, owner address, owner contact details, owner certification, UAV operator identity, UAV operator license, UAV operator certification, UAV pilot identity, UAV pilot license, UAV pilot certification and flight plan.

[R-5.1-006] The 3GPP system shall support capability to extend UAS data being sent to UTM with the evolution of UTM and its support applications in future.

[R-5.1-009] The 3GPP system should enable an MNO to augment the data sent to a UTM with the following: network-based positioning information of UAV and UAV controller.

[R-5.1-017] The 3GPP system shall support the UTM in detection of UAV operating without authorization.

[R-6.3-001] The 3GPP network shall be able to support network-based 3D space positioning (e.g., with altitude 30~300m) of a UE onboard UAV.

Table 7.3-1 lists typical scenarios and the corresponding positioning requirements for horizontal and vertical accuracy, availability, heading, latency, and UE speed.

Related requirements in TS 22.261:

The 5G system shall be able to make the position-related data available to an application or to an application server existing within the 5G network, external to the 5G network, or in the User Equipment.

NOTE 3: 		the position service latency can be tailored to the use cases.

### 5.5.6		Potential New Requirements needed to support the use case

[PR 5.5.6-1] Based on operator policy and UTM’s request, the 5G system shall be able to provide UTM with a UEs’ location and identity.

Editor’s Note:	this requirement is FFS

[PR 5.5.6-2] Subject to user consent and national or regional regulation, based on operator policy, and the UTM’s request, the 5G system may be able to provide Sensing results to the UTM.

Editor’s Note:	the relationship of Sensing and location in the above 2 requirements is FFS

## 5.6	Use case on supporting UAV inflight operations

### 5.6.1		Description

For UAVs operated using the 3GPP network access, the network needs to ensure that the UAVs can use the network reliably and with adequate performance. UAV flight mission need to be monitored for the real-time performance and stability of flight control link to guarantee the safety and continuity of the flight.

5G network can provide wide-area, high-quality and secure connectivity that can enable cost-efficient drone operations beyond visual line-of-sight. When UAV uses 3GPP network for communications, the UTM/drone operator may need to monitor the network status along the flight path with the help of 3GPP network.

### 5.6.2		Pre-conditions

In the logistics company, UAV A and UAV B are both individual terminals for express delivery and subscribe the communication service from 5G network.

Operator A provides 5G network coverage and communication services for remote commands and control (C2) communication between UAV and UAV control platform.

The UTM relies on Operator A to monitor and provide network status from the 5G network for UAV inflight related information.

### 5.6.3		Service Flows

Figure 5.6.3-1: Supporting UAV inflight monitoring

- The logistics company uses UAV A and UAV B to perform delivery operations. UAVs planned flight paths shared a common portion, along waypoint#1 and waypoint#2.

- UAV/UAV-C A and UAV/UAV-C B register in 5G network and can communicate with UTM though 5G network.

- The UAV-C A provides planned flight path #A information to UTM (e.g., flight path and required QoS along the planned flight path).

- UAV A receive the flight path information from UTM/UAS via 5G network and start flight mission.

- UTM requests 5G network for network condition information (e.g., bitrate, latency, reliability) along the flight path of UAV A.

- The 5G network provides the requested statistics (e.g., end-to-end latency, QoE) for specific requested area/areas (e.g., degraded QoE at waypoint #2).

- The UAV-C B provides planned flight path #B (along waypoint #1 and waypoint #2) information to UTM.

- UTM/UAS determines the final flight path #B (e.g., adjusted to take waypoint #2') and scheduling for UAV/UAV-C B based on the information from Operator A along UAV A flight route, and sends it to UAV/UAV-C B.

- UAV B receive the flight path #B information from UTM/UAS via 5G network and start flight mission.

### 5.6.4		Post-conditions

UAV A and UAV B can finish their delivery tasks with the support of 5G network.

### 5.6.5		Existing features partly or fully covering the use case functionality

Existing related service requirements are captured in 3GPP TS 22.125 [3] clause  6.2 "Network exposure for UAV services".

[R-6.2-001] The 3GPP system shall provide means to allow a 3rd party to request and obtain real-time monitoring the status information (e.g., location of UAV, communication link status) of a UAV.

[R-6.2-002] Based on operator 's policy, the 3GPP system shall provide means to provide a 3rd party with the information regarding the service status for UAVs in a certain geographical area and/or at a certain time.

NOTE:	Service status is about the information of whether the communication service to the UAV can be provided with a certain QoS by the network.

These requirements do not include explicit requirements on QoS related network conditions like bitrate, latency and reliability.

### 5.6.6		Potential New Requirements needed to support the use case

[P.R 5.6.6-001] Based on operator’s policy, the 5G system shall be able to provide a method to monitor and provide a 3rd party with information about network conditions and QoS (e.g. bitrate, latency, reliability) along a  UAV flight path.

[P.R 5.6.6-002] Based on operator’s policy, the 5G system shall be able to provide a method to monitor and provide a 3rd party with information about deviations and violations along a UAV flight path and time.

Note:	Deviations can be e.g. in location and/or time with respect to the original flight plan. Violations can be e.g. with respect to exclusion zones provided together with the flight plan or known via other means.

## 5.7	Use case on UAV flight route tracking at Rendezvous points

### 5.7.1	Description

Commercial UAV flights are usually BVLOS. Tracking the UAV flights to ensure that they are sticking to the planned route is critical to the success of the mission and the safety of the public. For UAVs that have network communication capability, cellular networks can greatly assist in UAV flight route tracking thanks to their ubiquitous network coverage. However, there are UAVs in the markets today that lack 5G network connection capabilities and probably will remain so for a long time. For such non-networked UAVs, ground stations or stationary devices can provide means to UTM/drone operator in flight route tracking.

Ground stations or stationary devices can be operated by the 5G network operator which has service agreements with the USS/UTM or by a third-party service provider. The ground stations or stationary devices are deployed along popular flight routes. The UTM/drone operator can have a need to track the flight route of a non 3GPP capable UAVs using the assistance of 5G network and ground station devices to further enhance the safety and security of drone operations (e.g., for DAA) while also supporting legacy UAVs.

Figure 5.7.1-1: UAV flight route tracking at Rendezvous points

### 5.7.2	Pre-conditions

A USS/UTM provides environmental surveillance services in an area. It regularly sends out UAVs to fly a planned route collecting environmental data. The UAV is not the subscriber UE of the 5G cellular network operator and can be considered as non-networked UAV for the 5G operator.

A 5G cellular network operator provides 5G network services in the area.

A 5G cellular network operator provides UAV tracking service in the area. For example, it has several ground stations or stationary devices deployed in strategic areas (e.g., along popular flight routes, close to important infrastructure as airports). The ground stations or stationary devices are capable of 5G connectivity and also detection or communication with non-networked UAVs using e.g. 5G device-to-device communication technologies. The 5G network operator has access to information about the available ground stations.

The technologies for detection of any UAV along a flight route is outside the scope of the use-case, however the tracking mechanisms for specific UAV needs to support unambiguous identification of the UAVs to ensure reliable and accurate tracking data about the UAV.

### 5.7.3	Service Flows

1.	The USS/UTM requests tracking of one of more UAVs from the 5G operator.

2.	The 5G network to track those non-network UAVs. The USS/UTM provides the planned flight route information to the 5G network (e.g., waypoints, ETA).

3.	The 5G network start to track those UAV, such as requests selected ground stations to monitor the presence of the target UAVs in its proximity. The ground stations monitor the UAV flight route based on the available information about the planned flight route.

4.	When a ground station detects and identifies the target UAV in its proximity, e.g., by receiving BRID information from the target UAV, the ground station in the 5G network informs the UTM that the UAV is following the planned route.

5.	At estimated time-of-arrival (ETA), the relevant ground station(s) monitor the presence of the target UAV in its proximity and the 5G network informs the UTM that the UAV has completed its flight as scheduled.

### 5.7.4	Post-conditions

The UAV finishes the mission with USS/UTM reassured during the whole flight.

### 5.7.5	Existing features partly or fully covering the use case functionality

Identification of UAVs which don’t have subscription with the 5G operator for DAA has been specified in Rel-18. Tracking for such UAVs is currently not covered in Rel-18 because release 18 only supports the capability to allow the 5G operator tracking of those UAVs which have the subscription with the 5G operator.

### 5.7.6	Potential New Requirements needed to support the use case

[P.R 5.7.6-001] The 5G system shall be able to support a mechanism to enable a network operator to track a UAV which doesn’t have subscription with this network operator.

# 6	Consolidated potential requirements

# 7	Conclusion and recommendations

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | SA1#99-e | S1-222402 |  |  |  | TR skeleton | 0.0.0 |
| 2022-09 | SA1#99-e |  |  |  |  | Includes: S1-222403, S1-222404, S1-222405, S1-222406 | 0.1.0 |
| 2022-11 | SA1#100 |  |  |  |  | Includes: S1-223436; S1-223616; S1-223719; S1-223720; S1-223721 | 0.2.0 |
| 2023-02 | SA1#101 |  |  |  |  | Includes: S1-230357; S1-230786; S1-230787 | 0.3.0 |
| 2023-03 | SA#99 | SP-230221 |  |  |  | MCC clean-up for presentation to SA | 1.0.0 |