3GPP TSG-SA WG6 Meeting #53	S6-230958

Athens, Greece, 27th February 2023 – 3rd March 2023	(revision of S6-230665)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-64 | CR | 0001 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FS_eV2XAPP2 solution #4 naming correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_eV2XAPP2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution provides a naming update for the Solution #4 procedure in order to disambiguate it from the VRU zone feature. The change is in line with the agreements reached at SA6#52. Based on the change and the previous agreements, the last line of the solution evaluation can be removed, the overall evaulation and conclusion updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedure naming change, update of overall evaulation and conclusion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Two important procedures and features are not disambiguated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4, 7.3, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 corrects the CR cover page |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.4	Solution #4 - UE initiated request for protection zones.

### 6.4.1	Solution description

#### 6.4.1.1	General

A cyclist training for long-distance competitions is vulnerable to road collisions during the entire training route. The cyclist would like to create a protection zone that tracks their route for use as part of the services provided by the V2X service provider. The V2X infrastructure, including edge enabler servers, receives a request to create a protection zone and dynamically update the protection zone to follow the route of the cyclist. When an incoming vehicle is detected , the infrastructure can alert the vehicle and provision V2P communication configurations to enable the vehicle to communicate with the cyclist.

The following solution is provided for key issues #1 and 2 and complements solution #1. This solution also enhances solution #2 by providing an option to employ the VAE layer to receive protection zone configuration requests from the UE and notify VASS. The two solutions can coexist in any deployment, with the VRU application determining whether solution#4 may be used based on a policy specifying whether:

a.	Use of VAE service for protection zone notification is allowed; and

b.	Protection zone requests initiated by UEs are allowed.

#### 6.4.1.2	Procedure

Pre-conditions:

1.	VASS has subscribed to VAE server to provide support for V2P communication.

2.	VAE clients of V2X cyclist UE and vehicular UE are registered with VAE server.

3.	The VRU application (at VASS and Application client at the cyclist UE) are pre-configured so that:

a.	Use of VAE service for protection zone notification is allowed; and

b.	Protection zone requests be initiated by UEs are allowed.

NOTE 1:	Pre-condition 3 allows VAE server to determine that use of solution#4 mechanism is allowed.

Figure 6.4.1.2-1: UE request for protection zone

1.	Optionally, an application client on a cyclist UE makes a request to the VAE client to create a protection zone.

2.	A VAE client sends a request to a VAE server to create a protection zone. The request may include V2X Service ID, UE ID, application ID, UE location and location reporting configuration, destination location, request for dynamic protection zone, V2P communication configuration, user consent for using user application context information, user activity, active applications, etc.

3.	The VAE server sends a request to a VASS to authorize the creation of the protection zone for the cyclist UE and includes the information provided by the cyclist UE.

4.	The VASS authorizes the request and creates a protection zone based on information received from the VAE server. The VASS provides the VAE server with the protection zone and V2P communication configuration.

5.	The VASS responds with a an authorization response to the VAE server. The response includes the status of the authorization, the protection zone type, geographical area, time validity and initiation trigger, types of supported messages and requirements (e.g. requirements for VAM messages), application QoS requirement dedicated for the protection zone (e.g. URLLC like), whether it is a dynamic or static zone, an expiration for the protection zone, V2P communication configuration information, etc.

6.	The VAE server configures subscriptions to obtain UE location from the 5G system. Sources of UE mobility information may come from SEAL LMS servers, EES, an analytics function (e.g. from ADAES or NWDAF), and the 5G core network. For V2X UEs location obtained from the location management server, the procedures specified in 3GPP TS 23.434 (clause 9.3.12) apply.

7.	The VAE server sends a response message to the cyclist UE with a status for the request, the information of the protection zone, and updated V2P communication configuration. The VAE server may also subscribe to receive notifications from the VAE client about application context events such as user activity, active applications, application notifications, navigation information, etc.

8.	Optionally, the VAE client returns a response to the application client.

9.	The VAE server receives UE mobility information from the 5G network and/or from the UEs. The cyclist UE and vehicular UE may also provide application context information to the VAE server, such as battery level, user activity, active applications, and routing information from a navigation app. The battery level of the UE may help the VAE server determine the V2P communication configuration to provide to UEs in the protection zone. For example, if the battery level of a cyclist UE is low, the VAE server may provide V2P communication configurations to vehicular UEs to align with when the cyclist UE is able to receive V2P communication. Alternatively, the VAE server can provide V2P communication parameters to vehicular UEs to communicate with nearby UEs instead.

10.	The VAE server updates the protection zone based on information the VAE server has obtained about the cyclist UE. For example, the protection zone may be updated due to a change in UE location. In addition, the VAE server may broadcast the new protection zone information to all the UEs in the protection zone.

11.	The VAE server detects that a vehicular UE is now in the updated protection zone and sends a notification alert to the vehicular UE and includes the V2P communication configuration for use by the vehicular UE to communicate directly with the cyclist UE.

### 6.4.2	Solution evaluation

This solution addresses KI #1 and aspects of KI #2 and provides a procedure for a VAL client on a cyclist UE to request the creation of a dynamic protection zone via the VAE client. The VAE server checks with a VASS to authorize the request and if authorized, the VASS creates a protection zone based on the information provided by the VAE client. After the creation of the protection zone, the VAE server monitors the UE’s mobility information and updates the protection zone accordingly. If the VAE server detects the presence of other vehicular UEs in the updated protection zone, the VAE server sends a notification alert to the vehicular UE and includes V2P communication configuration information.

Further agreement is needed for the scenario of this solution where UEs can initiate VRU protection zone creation.

* * * First Change * * * *

## 7.3	Overall evaluation of key issue#1

Key issue#1 on support for high risk VRU zones discussed the following open issues:

a.	Whether and how to enhance VAE layer capabilities to support configuring the communication parameters for the zones.

b.	Whether and how to enhance VAE layer capabilities to translate the zone configurations to network requirements and zone specific provisioning parameters.

c.	Whether and how to assist the provisioning of zone specific parameters to the devices which can be potential VRUs and V2X-UEs.

Solution#2 provides a solution for network initiated VRU zone configuration and operation . It addresses the open issues in a, b and c.

Solution#4 provides a solution for UE initiated  configuration and operation of protection zones.

* * * Next Change * * * *

# 8	Conclusions

This technical report fulfills the objectives of the study on enhancements to application layer to support advanced V2X services like V2P, ToD, etc and edge computing deployments for V2X services. The study includes the following:

1)	Identification of key issues (clause 4) and corresponding architecture requirements (clause 5) for enhanced V2X application enabler capabilities.

2)	Individual solutions (clause 5) addressing the key issues.

3)	Overall evaluation (clause 7) of all the key issues and solutions.

The results from the study will be considered for follow-up normative work in Release 18 as follows:

1)	The architecture requirements (in clause 5) will be considered as the basis for technical specification with necessary enhancements and additions;

2)	The following key issues (clause 4) and individual solutions (clause 6) are considered to be the candidate solutions with necessary enhancements as appropriate, according to the overall evaluation (clause 7):

-	For key issue#1, solution#2 is considered, and solution#4 can be considered based on the agreement on the  use of protection zones.

- 	For key issue#2, solution#1 is considered subjected to capabilities enabled by SA2 and RAN2.

-	For key issue#3, solution#3 is considered.

-	For key issue#4, there is no solution proposed.

-	For key issue#5, solution#6 is considered.

-	For key issue#6, solution#5 is considered.