TSG SA Meeting #SP-99 Agenda	SP-230331

21 – 24 March 2023, Rotterdam, Netherlands	(revision of S2-2301393)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3666 | rev | 5 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of NAT exposure in 23.502 according to the conclusion in UPEAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, DISH Network, Intel, China Mobile, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-03-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the conclusion in TR 23.700-62, UPF event exposure is enhanced in R18 and this impacts to UPF service expoure and UPF discovery. Rev 5: In step 3 and 5 of 4.15.10, whether the DNN and S-NSSAI are needed together with the public IP address is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of NAT exposure Rev 5: adding an EN: whether the S-NSSAI and DNN can be used is FFS in step 3 and 5 of 4.15.10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This new defined capability of UPF in UPEAS is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.10, 4.17.6.1, 5.2.26.1, 5.2.26.2.1, 5.2.7.2.2, 5.2.7.3.2, 5.2.6.27.2, new: 5.2.26.3, 5.2.26.3.1, 5.2.26.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 3825 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************* Change 1 *****************

4.15.10	AF specific UE ID retrieval

This clause contains the detailed description and the procedures for the AF specific UE ID retrieval. The AF specific UE Identifier is represented by the External Identifier as defined in TS 23.003 [33].

NOTE 1:	After retrieving AF specific UE ID, the AF can invoke NEF provided services (e.g. location monitoring).

Figure 4.15.10-1: AF specific UE ID retrieval

1.	AF requests to retrieve UE ID via the Nnef_UEId_Get service operation. The request message shall include UE address (IP address or MAC address) and AF Identifier, it may include, Port Number associated with the IP address, MTC Provider Information, Application Port ID, IP domain. The MTC Provider Information identifies the MTC Service Provider and/or MTC Application. If available, the AF may also provide the corresponding DNN and/or S-NSSAI.

NOTE 2:	The MTC Provider Information can be used by any type of Service Providers (MTC or non-MTC) or Corporate or External Parties for, e.g. to distinguish their different customers.

NOTE 3:	The combination of IP address and Port Number can be used by 5GC to derive the UE address if the UE is behind a NAT, see steps 3-6 below.

NOTE 4:	The Application Port ID is as defined in Nnef_Trigger_Delivery.

NOTE 5:	The NEF can validate the provided MTC Provider Information and override it to a NEF selected MTC Provider Information based on configuration. How the NEF determines the MTC Provider Information, if not present, is left to implementation (e.g. based on the requesting AF).

2.	The NEF authorizes the AF request. If the authorisation is not granted, the NEF replies to the AF with a Result value indicating authorisation failure; otherwise the NEF proceeds with the following steps. The NEF determines corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or is determined by the NEF based on the requesting AF Identifier, MTC Provider Information.

If the NEF has received a Port Number in step 1, based on configuration, the NEF may recognize the address received is a public IP address different from the actual UE IP address (private IP address), i.e., the UE is behind a NAT. If so, the NEF performs steps 3 to 6. Otherwise, steps 3 to 6 are skipped.

3.	The NEF uses the Nnrf_NFDiscovery service operation to obtain the UPF address of the UPF hosting the UE (public) IP address. The request includes the UE (public) IP address. The NEF may also include the DNN and S-NSSAI associated with the AF ID, as well as the IP domain.

Editor's note:	Whether the S-NSSAI and DNN can be used is FFS.

4.	The NRF responds with a Nnrf_NFDiscovery response message including the UPF address of the UPF hosting the UE (public) IP address.

5.	The NEF uses the Nupf_GetPrivateUEIP_Get service operation to request UE’s (private) IP address from the UPF. The request includes the UE (public) IP address and Port Number, and optionally DNN and S-NSSAI associated with the AF ID.

Editor's note:	Whether the S-NSSAI and DNN can be used is FFS.

6.	The UPF responds with the Nupf_GetPrivateUEIP_Get response message including UE’s IP address and the IP domain. If the UPF has applied a NAT functionality, the UE’s IP address returned by the UPF is the private UE IP address. If IP domain of UE private IP address is returned from UPF, it always takes precedence regardless of whether the IP domain information also provided by AF when it invokes Nnef_UEId_Get service operation.

7-8.	The NEF uses the Nbsf_Management_Discovery service operation with UE address and IP domain and /or DNN and/or S-NSSAI to retrieve the session binding information of the UE. If no SUPI is received in the session binding information from the BSF, the NEF replies to the AF with a Result value indicating that the UE ID is not available.

9.	The NEF interacts with UDM to retrieve the AF specific UE Identifier via the Nudm_SDM_Get service operation. The request message includes SUPI and at least one of Application Port ID, MTC Provider Information or AF Identifier.

10.	The UDM responds to the NEF with an AF specific UE Identifier represented as an External Identifier for the UE which is uniquely associated with the Application Port ID, MTC provider Information and/or AF Identifier.

11.	The NEF further responds to the AF with the information (including the AF specific UE Identifier represented as an External Identifier) received from the UDM.

******************* Change 2 *****************

5.2.7.2.2	Nnrf_NFManagement_NFRegister service operation

Service Operation name: Nnrf_NFManagement_NFRegister.

Description: Registers the consumer NF in the NRF by providing the NF profile of the consumer NF to NRF and NRF marks the consumer NF available.

Inputs, Required: NF type, NF instance ID, FQDN or IP address of NF, Names of supported NF services (if applicable) and PLMN ID e.g. if NF needs to be discovered by other PLMNs/SNPNs.

NOTE 1:	for the UPF, the addressing information within the NF profile corresponds to the N4 interface.

NOTE 2:	For the purpose of the Nnrf_NFManagement service, the SCP is treated by the NRF in the same way as NFs. Specifically, the SCP is designated with a specific NF type and NF instance ID. However, the SCP does not support services and related NF profile parameters do not apply (e.g. NF Set ID, NF service set ID, Endpoint Address(es) of instance(s) of supported service(s)), see clause 6.2.6.3 of TS 23.501 [2].

Inputs, Optional:

-	If the consumer NF stores Data Set(s) (e.g. UDR): Range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers, Data Set Identifier(s).

-	If the consumer is BSF: Range(s) of SUPIs, range(s) of GPSIs, Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [37], Range(s) of SUPIs, range(s) of GPSIs.

NOTE 3:	Range of SUPI(s) is limited in this release to a SUPI type of IMSI as defined in TS 23.003 [33].

-	If the consumer is UDM, UDR, PCF, BSF or AUSF, they can include UDM Group ID, UDR Group ID, PCF Group ID, BSF Group ID, AUSF Group ID respectively.

-	For UDM and AUSF, Routing Indicator, or Routing Indicator and Home Network Public Key identifier; Home Network Identifier: PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN. Optionally, some NFs may additionally include a Home Network Identifier (including the identification of the CH with AAA Server or DCS with AAA Server) in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using a DCS with AAA Server.

-	For NSSAAF, Home Network Identifier in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using credentials from a DCS with AAA Server.

-	If the consumer is AMF, it includes list of GUAMI(s). In addition, AMF may include list of GUAMI(s) for which it can serve as backup for failure/maintenance.

-	If the consumer is CHF, it may include Range(s) of SUPIs, Range(s) of GPSIs, or Range(s) of PLMNs as defined in TS 32.290 [42].

-	If the consumer is CHF, primary CHF instance and the secondary CHF instance pair. If the CHF does not provide NF set ID or NF Service Set ID, it shall provide a primary CHF instance and the secondary CHF instance pair and otherwise it may do so.

-	If the consumer is P-CSCF, the P-CSCF IP address(es) to be provided to the UE by SMF.

-	If the consumer is HSS, IMPI range, IMPU range, HSS Group ID (as defined in TS 23.228 [55]) can be used as optional input parameters.

-	For the UPF Management by SMF: UPF Provisioning Information as defined in clause 4.17.6.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	DNN(s) if the consumer is PCF or BSF. DNN(s) per S-NSSAI if the consumer is SMF, UPF or TSCTSF.

-	If the consumer is a trusted AF it may include one or multiple combination(s) of S-NSSAI and DNN corresponding to the AF. In addition, it may include supported Application Id(s), Event ID(s) and Internal-Group Identifier. It may include an indication whether it supports mapping between UE IP address (IPv4 address or IPv6 prefix) and UE ID (i.e. SUPI).

-	Information about the location of the NF consumer (operator specific information, e.g. geographical location, data centre).

-	TAI(s).

-	NF Set ID.

-	NF Service Set ID.

-	If the consumer is PCF or SMF, it includes the MA PDU Session capability to indicate if the NF instance supports MA PDU session or not.

-	If the consumer is PCF, it includes the DNN replacement capability to indicate if the NF instance supports DNN replacement or not.

-	If the consumer is PCF, it may include the 5G ProSe Capability as specified in TS 23.304 [77].

-	If the consumer is PCF, it may include the V2X capability as specified in TS 23.287 [73].

-	If the consumer is NWDAF, it includes Analytics ID(s) (possibly per service), NWDAF Serving Area information and Supported Analytics Delay per Analytics ID(s) (if available). In addition, it includes Analytics aggregation capability and/ or Analytics metadata provisioning capability if such capability is provided by the NWDAF. If the consumer NWDAF contains MTLF, it may also include the ML model Filter information parameters S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s) and ML Model Interoperability indicator, if available (see clause 5.2 of TS 23.288 [50]). If the the consumer is NWDAF containing MTLF with Federated Learning (FL) capability, it includes FL capability type (i.e. FL client capability, FL server capability, if available), Time interval supporting FL, if available (see clause 5.2 of TS 23.288 [50]). It may also include NF Set ID and NF Type of the NF data sources, if data management service is available. Details about NWDAF specific information are described in clause 6.3.13 of TS 23.501 [2].

Editor's note:	It is FFS if the Interoperability indicator is per Analytics ID.

-	If the consumer is NEF, it may include Event ID(s) supported by AFs, the S-NSSAI and DNN corresponding to the untrusted AF served by the NEF, Application Identifier(s) supported by AFs, range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF. It may also include an indication whether the untrusted AF supports mapping between UE IP address (IPv4 address or IPv6 prefix) and external UE ID (i.e. GPSI). If the consumer is local NEF, it may include parameters of list of supported TAI or list of supported DNAI additionally.

-	If the consumer is NSACF, it includes S-NSSAI(s) of the NSACF located network and served by NSACF for which it manages the NSAC NSACF Serving Area information and NSACF service capabilities. Details about NSACF Service Area information and NSACF service capabilities are described in clause 6.3.22 of TS 23.501 [2].

-	Notification endpoint for default subscription for each type of notification that the NF is interested in receiving.

-	Endpoint Address(es) of instance(s) of supported service(s).

-	NF capacity information.

-	NF priority information.

-	If consumer is NF, SCP domain the NF belongs to.

-	If the consumer is SCP, it may include:

-	SCP domain(s) the SCP belongs to.

-	Remote PLMNs reachable through SCP.

-	Endpoint addresses or Address Domain(s) (e.g. IP Address or FQDN ranges) accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	If the consumer NF is MB-SMF, it may include MB-SMF service area and the MBS Session ID(s), Area Session ID(s), the corresponding MBS service area(s) if available, as specified in TS 23.247 [78].

-	If the consumer is DCCF, the request may include DCCF Serving Area information, NF type of the NF data source, NF Set ID of the NF data sources. Details about DCCF discovery and selection are described in clause 6.3.19 of TS 23.501 [2].

-	If the consumer is EASDF, it may include S-NSSAI, DNN, N6 IP address of the PSA UPF, location as per NF profile and DNAI (if exists).

-	For ON-SNPN, if the consumer is AMF, Capability to support SNPN Onboarding, or, if the consumer is SMF, Capability to support User Plane Remote Provisioning.

-	If the consumer is NEF, it includes the support for UAS NF functionality.

-	If the consumer is UPF and UPF can expose NAT information, it may include the range of public IP addresses the NAT uses.

Outputs, Required: Result indication.

Outputs, Optional: None.

See clause 5.21.2.1 in TS 23.501 [2], the AMF registers itself to NRF.

******************* Change 3 *****************

5.2.7.3.2	Nnrf_NFDiscovery_Request service operation

Service operation name: Nnrf_NFDiscovery_Request

Description: provides the IP address or FQDN of the expected NF instance(s) and, if present in NF profile, the Endpoint Address(es) of NF service instance(s) to the NF service consumer or SCP.

Inputs, Required: one or more target NF service Name(s), NF type of the target NF, NF type of the NF service consumer.

If the NF service consumer intends to discover an NF service producer providing all the standardized services, it provides a wildcard NF service name.

Inputs, Optional:

-	S-NSSAI and the associated NSI ID (if available), DNN, target NF/NF service PLMN ID (or realm in the case of network specific identifier type SUCI/SUPI, see clause 4.17.5a), NRF to be used to select NFs/services within HPLMN or Credentials Holder, Serving PLMN ID (or PLMN ID and NID in the case of SNPN, see clause 4.17.5a), the NF service consumer ID, preferred target NF location, TAI.

NOTE 1:	For network slicing the NF service consumer ID is a required input.

-	FQDN for the S5/S8 interface of the SMF+PGW-C, to discover the N11/N16 interface of the SMF+PGW-C in the case of EPS to 5GS mobility.

-	If the target NF stores Data Set(s) (e.g. UDR, BSF): SUPI, GPSI, IMPI, IMPU, Data Set Identifier(s). (UE) IPv4 address, IP domain or (UE) IPv6 Prefix.

NOTE 2:	GPSI is relevant for BSF.

NOTE 3:	If the request includes a subscriber identifier the NRF may need to use the association between the supplied subscriber identifier and the appropriate NF Group ID as described in clause 6.3.1 of TS 23.501 [2] to determine the applicable set of NF instances for the response.

NOTE 4:	The (UE) IPv4 address or (UE) IPv6 Prefix is provided for BSF discovery: in that case the NRF looks up for a match within one of the Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes provided by BSF(s) as part of the invocation of Nnrf_NFManagement_NFRegister operation. The NRF is not meant to store individual (UE) IPv4 addresses or (UE) IPv6 prefixes.

-	If the target NF is UDM or AUSF, the request may include the UE's Routing Indicator, or the UE's Routing Indicator and Home Network Public Key identifier.

-	If the target UDM or NF is AUSF, the request may include the UE's HNI: PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN. Optionally, some NFs may additionally include a Home Network Identifier in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using a DCS with AAA Server.

-	If the target NF is NSSAAF, the request may include Home Network Identifier in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using credentials from a DCS with AAA Server.

-	If the target NF is AMF and the consumer NF is MB-SMF for broadcast service, the request includes TAI(s) (see clause 7.3 of TS 23.247 [78]).

-	If the target NF is AMF and the consumer NF is other than MB-SMF, the request may include:

-	AMF region, AMF Set, GUAMI and Target TAI(s).

-	If the target NF is UDR or UDM or AUSF or PCF or BSF, the request may include UDR Group ID or UDM Group ID or AUSF Group ID or PCF Group ID or BSF Group ID respectively.

NOTE 5:	It is assumed that the corresponding NF service consumer is either configured with the corresponding Group ID or it received it via earlier Discovery output.

-	If the target NF is UDM, the request may include SUPI, GPSI, Internal Group ID and External Group ID.

-	If the target NF is UPF, the request may include SMF Area Identity, UE IPv4 Address/IPv6 Prefix, public (due to NAT) UE IPv4 address/IPv6 Prefix, supported ATSSS steering functionality, the supported UPF event exposure service and the supported Event IDs that can be subscribed.

NOTE 6:	If the UE's(public) IPv4 address or UE's(public) IPv6 Prefix is provided for UPF discovery, then the NRF looks up for a match within one of the Range(s) of (public) IPv4 addresses or (public) IPv6 prefixes provided by UPF in the NF profile at the invocation of Nnrf_NFManagement_NFRegister operation. The NRF is not meant to store the UE's individual (public) IPv4 addresses or (public) IPv6 prefixes.

NOTE 7:	Discovering UPF at PDU Session Establishment time and creating the N4 association assumes full connectivity between SMF and UPFs.

-	If the target NF is CHF, the request may include SUPI or GPSI as specified in TS 32.290 [42].

-	If the target NF is PCF or SMF, the request may include the MA PDU Session capability to indicate that a NF instance supporting MA PDU session capability is requested.

-	If the target NF is PCF, the request may include the DNN replacement capability to indicate that a NF instance supporting DNN replacement capability is preferred.

-	If the target NF is PCF, the request may include the 5G ProSe Capability as specified in TS 23.304 [77].

-	If the target NF is PCF, the request may include the V2X capability as specified in TS 23.287 [73].

-	If the target NF is NWDAF, the request may include Analytics ID(s) (possibly per service), TAI(s), Analytics aggregation capability, Analytics metadata provisioning capability, a Real-Time Communication Indication per Analytics ID, NF Set ID and NF Type of the NF data sources. The request may include the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and NF consumer information when the target is an NWDAF containing MTLF. The request includes required FL capability type (i.e. FL server capability, FL client capability, if available) and Time period of interest when the target is an NWDAF containing MTLF with FL capability. Details about NWDAF discovery and selection are described in clause 6.3.13 of TS 23.501 [2].

NOTE 8:	Analytics metadata provisioning capability is only applicable when NF service consumer is NWDAF.

NOTE 9:	NF consumer information such as vendor ID is defined in stage 3.

-	If the target NF is HSS, the request may include IMPI, and/or IMPU and/or HSS Group ID.

-	If the NF service consumer needs to discover NF service producer instance(s) within an NF instance, the request includes the target NF Instance ID and NF Service Set ID of the producer.

-	If the NF service consumer needs to discover NF service producer instance(s) in an equivalent NF Service Set within an NF Set, the request includes the identification of the equivalent NF service Set and NF Set ID of producer.

NOTE 10:	TS 29.510 [37] specifies the mechanism to identify equivalent NF Service Sets.

-	If the NF service consumer needs to discover NF service producer instance(s) in the NF Set, the request includes the target NF Set ID of the producer.

-	If the target NF is SMF, the request may include:

-	the UE location (TAI); or

-	TAI list.

-	If the target NF is P-CSCF, the request may include UE location information, UE IP address/IP prefix, Access Type.

-	If the target NF is NEF, the request may include Event ID(s) provided by AF and optional AF identification as described in clause 6.2.2.3 of TS 23.288 [50]. When the consumer is an AF, the request may include an External Identifier, External Group Identifier, or a domain name. If the target NF is local NEF, the request may include the parameters of list of supported TAI or list of supported DNAI additionally.

-	If the target NF is SMF, the request may include the Control Plane CIoT 5GS Optimisation Indication or User Plane CIoT 5GS Optimisation Indication.

-	If the target NF is NSACF, the request may include S-NSSAI(s) of the NSACF located network and served by NSACF, NSACF Serving Area information and NSACF service capability. Details about NSACF discovery and selection are described in clause 6.3.22 of TS 23.501 [2].

-	If the target NF is SCP, the request may include information about:

-	SCP domain(s).

-	Remote PLMN reachable through SCP.

-	Endpoint addresses or Address Domain(s) (e.g. IP Address or FQDN ranges) accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	If the target NF is MB-SMF, the request may include UE location (i.e. TAI), MBS Session ID and Area Session ID. Details about MB-SMF discovery and selection are described in TS 23.247 [78].

-	If the target NF is 5G DDNMF, the request may include SUPI, IP Address or FQDN of 5G DDNMF.

-	If the target NF is DCCF, the request may include TAI(s), NF type of the NF data sources, NF Set ID of the NF data sources. Details about DCCF discovery and selection are described in clause 6.3.19 of TS 23.501 [2].

-	If the target NF is EASDF, the request may include S-NSSAI, DNN, N6 IP address of the PSA UPF, location as per NF profile and DNAI(if exist). Details about EASDF discovery and selection are described in clause 6.3.23 of TS 23.501 [2].

-	If the target NF is AMF, the request may include the support of SNPN Onboarding to indicate whether the target NF instance supports SNPN Onboarding or not.

-	If the target NF is SMF, the request may include the support of User Plane Remote Provisioning to indicate whether the target NF instance supports User Plane Remote Provisioning or not as described in clause 5.30.2.10.4.3 of TS 23.501 [2].

-	If the target NF is NEF, the request may include the support of UAS NF functionality.

-	If the target NF is NSSAAF, the request may include SUPI or Internal Group ID.

Outputs, Required: A set of NF instances, a validity period for the discovery result, containing per NF Instance: NF type, NF instance ID, FQDN or IP address(es) of the NF instance and if applicable, a list of services instances, where each service instance has a service name, a NF service instance ID and optionally Endpoint Address(es)

Endpoint Address(es) may be a list of IP addresses or an FQDN for the NF service instance.

NOTE 11:	SCPs does not have any service instances.

Outputs, Optional: Per NF instance, other information in the NF profile listed in clause 6.2.6 in TS 23.501 [2] related to the NF instance, such as:

-	NF load information.

-	NF capacity information.

-	NF priority information.

-	If the target NF stores Data Set(s) (e.g. UDR): Range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers, Data Set Identifier(s). If the target NF is BSF or P-CSCF: Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, Range(s) of SUPIs, range(s) of GPSIs.

NOTE 12:	Range of SUPI(s) is limited in this release to a SUPI type of IMSI as defined in TS 23.003 [33].

-	If the target NF is UDM, UDR, PCF, BSF or AUSF, they can include UDM Group ID, UDR Group ID, PCF Group ID, BSF Group ID, AUSF Group ID respectively.

-	If the target NF is HSS, it can include HSS Group ID.

-	For UDM and AUSF, Routing Indicator, or Routing Indicator and Home Network Public Key identifier.

-	If the target NF is AMF, it includes list of GUAMI(s). In addition, it may include list of GUAMI(s) for which it can serve as backup for failure/maintenance.

-	If the target NF is CHF, it includes primary CHF instance and the secondary CHF instance pair(s), if configured in CHF instance profile.

-	For the UPF Management: UPF Provisioning Information as defined in clause 4.17.6.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	Information about the location of the target NF (operator specific information, e.g. geographical location, data centre).

-	TAI(s).

-	PLMN ID.

-	If the target is PCF or SMF, it includes the MA PDU Session capability to indicate if the NF instance supports MA PDU session or not.

-	If the target is PCF, it includes the DNN replacement capability to indicate if the NF instance supports DNN replacement or not.

-	If the target NF is NWDAF, it includes the Analytics ID(s) (possibly per service), NF Set ID and NF Type of the NF data sources, if available, NWDAF Serving Area information. In addition, it includes Analytics aggregation capability and/ or Analytics metadata provisioning capability, if such capability is provided by the NWDAF. It may include the Supported Analytics Delay per Analytics ID. If consumer is NWDAF, it may also include the ML model Filter information parameters S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s), if available (see clause 5.2 of TS 23.288 [50]). Details about NWDAF specific information are described in clause 6.3.13 of TS 23.501 [2].

NOTE 13:	The Supported Analytics Delay is provided for an Analytics ID only when the NRF had received Real-Time Communication Indication for this Analytics ID in the NWDAF discovery request.

-	If the target is a trusted AF, it includes one or multiple combination(s) of the S-NSSAI and DNN corresponding to the AF. In addition, it may include supported Application Id(s), Event ID(s) supported by the AF and Internal-Group Identifier.

-	NF Set ID.

-	NF Service Set ID.

-	If the target NF is SMF, it may include the SMF(s) Service Area.

NOTE 14:	If no SMF Service Area is provided, the AMF assumes that a SMF can serve the whole PLMN.

-	If the target NF is P-CSCF, it includes P-CSCF FQDN(s) or IP address(es) and optional Access Type(s) associated with each P-CSCF.

-	If the target NF is NEF, it may include Event ID(s) provided by AF and/or it includes one or multiple combination(s) of the S-NSSAI and DNN corresponding to the untrusted AF served by the NEF.

-	SCP domain the NF belongs to.

NOTE 15:	Only one SCP domain is registered in NF profile for an NF.

-	If the target is SCP:

-	SCP domain(s).

-	Remote PLMNs reachable through SCP.

-	Endpoint addresses or Address Domain(s) (e.g. IP Address or FQDN ranges) accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	If the target NF is 5G DDNMF, it may include IP Address or FQDN of 5G DDNMF.

-	If the target NF is MB-SMF, it may include the MBS Session ID(s), Area Session ID(s), corresponding MBS service area(s) as described in TS 23.247 [78].

-	If the target NF is DCCF, it includes DCCF serving area information, NF type of the NF data sources, NF Set ID of the NF data sources. Details about DCCF specific information are described in clause 6.3.19 of TS 23.501 [2].

See clause 4.17.4 and 4.17.5 for details on the usage of this service operation.

******************* Change 4 *****************

5.2.26.1	General

The following table shows the UPF Services and UPF Service Operations.

Table 5.2.26.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_EventExposure | Notify | Subscribe/Notify | NEF, AF, NWDAF, TSNAF/TSCTSF |
|  | Subscribe | Subscribe/Notify | NWDAF, SMF |
|  | Unsubscribe | Subscribe/Notify | NWDAF, SMF |
| Nupf_GetPrivateUEIPaddr | Get | Request/Response | NEF |

******************* Change 5 *****************

5.2.26.3	Nupf_Nupf_GetPrivateUEIPaddr service

5.2.26.3.1	General

This service provides the UE IP address translation information (UE private IP address and IP domain).

5.2.26.3.2	Nupf_GetPrivateUEIPaddr_Get service operation

Service operation name: Nupf_GetPrivateUEIPaddr_Get.

Description: NF service consumer gets the UE private IP address and IP domain.

Inputs, Required: Public IP address and port.

The Public IP address and port uniquely identifies the UE IP address behind a NAT

Inputs, Optional: None.

Outputs, Required: UE private IP address

Outputs, Optional: IP domain of UE private IP address

******************* Change 6 *****************

5.2.6.27.2	Nnef_UEId_Get operation

Service operation name: Nnef_UEId_Get

Description: Get the UE identifier.

Inputs, Required: UE address (i.e. IPv4/IPv6 address or MAC address), AF Identifier.

Inputs, Optional: DNN, S-NSSAI, Port number (e.g. TCP or UDP port), IP domain, Application port ID, MTC Provider Information.

Outputs, Required: Result, AF specific UE Identifier represented as an External Identifier.

Outputs, Optional: None.

******************* Change 7 *****************

5.2.26.2	Nupf_EventExposure Service

5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There are several operations for this service:

-	Notifying events on the PDU Session to the NFs.

-	Allow consumer NFs to subscribe and unsubscribe for an Event ID on UPF.

-	Allow consumer NFs to acknowledge an event notification.

The following events can be notified to a NF consumer:

-	QoS Monitoring. This event provides QoS Flow level performance information. For example, it can be used for URLLC: the event notification may contain the QoS Monitoring report as described in clause 5.33.3.2 of TS 23.501 [2], or for UPF Data collection by NWDAF for analytics (see TS 23.288 [50]).

The event notification may contain following information:

-	QoS monitoring result e.g. end to end delay for specific QoS flow or for specific PDU session.

-	QoS Flow Bandwidth measurements: It provides bitrate measurements (UL, DL and/or overall) for a PDU Session and QoS Flow.

-	UserDataUsageMeasures. This event provides information of user data usage of the User PDU Session. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]).

The event notification may contain following information:

-	Volume Measurement: measures of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) with or without application granularity. This measurement can also include number of packets transmitted for applications where that is possible to differentiate.

-	Throughput Measurement: measures of data throughput (UL and DL) measures aggregated for the PDU Session or per application.

-	UserDataUsageTrends. This event provides statistical measurements. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]).

The event notification may contain following information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement period for the PDU Session or per application.

-	TSC management information (UMIC, PMIC, NW-TT port number) as defined in clause 5.8.2.11.14 of TS 23.501 [2].

If the consumer of UPF service is NWDAF and the target of UE is any UE, according to the Analytic ID from consumer, the NWDAF can decide which kind of information should be collect from UPF and event ID to use. Subscription may be performed directly towards UPF or via SMF. In the latter case, the SMF determines the UPF(s) towards which the final consumer subscription is to be relayed and sends the subscription request to UPF (as described in clause 4.15.11). The UPF collects the data according to the event ID, and exposes the related information directly regardless of whether the subscription has ben relayed by the SMF.

Editor's note:	The condition for a final consumer of UPF events to directly subscribe to UPF or to request SMF to do third party subscription are FFS.

******************* Change 7 *****************

4.17.6	SMF Provisioning of available UPFs using the NRF

4.17.6.1	General

This clause describes the provisioning of available UPFs in SMF using the NRF as documented in clause 6.3.3 of TS 23.501 [2].

This optional node-level step takes place prior to selecting the UPF for PDU Sessions and may be followed by N4 Node Level procedures defined in clause 4.4.3 where the UPF and the SMF exchange information such as the support of optional functionalities and capabilities.

As an option, UPF(s) may register in the NRF. This registration phase uses the Nnrf_NFManagement_NFRegister operation and hence does not use N4.

For the purpose of SMF provisioning of available UPFs, the SMF uses the Nnrf_NFManagement_NFStatusSubscribe, Nnrf_NFManagement_NFStatusNotify and Nnrf_NFDiscovery services to learn about available UPFs.

NOTE 1:	The protocol used by UPF to interact with NRF is described in TS 29.510 [37]

UPFs may be associated with UPF Provisioning Information in the NRF. The UPF Provisioning Information consists of:

-	a list of (S-NSSAI, DNN);

-	UE IPv4 Address Ranges and/or IPv6 Prefix Range(s) per (S-NSSAI, DNN); and

NOTE 2:	The above information can be used by the SMF for UPF selection when static IP address/prefix allocation is required for a UE.

-	NATed IPv4 Address Ranges and/or NATed IPv6 Prefix Range(s) per (S-NSSAI, DNN, IP domain).

NOTE X:	The NATed IP address/prefix ranges can be used for UPF discovery when the UPF implements NAT functionality.

-	a SMF Area Identity the UPF can serve. The SMF Area Identity allows limiting the SMF provisioning of UPF(s) using NRF to those UPF(s) associated with a certain SMF Area Identity. This can e.g. be used if an SMF is only allowed to control UPF(s) configured in NRF as belonging to a certain SMF Area Identity.

-	the supported ATSSS steering functionality, i.e. whether MPTCP functionality or ATSSS-LL functionality or both are supported.

-	the supported UPF event exposure service and supported Event IDs, e.g. local notification of QoS Monitoring to AF or e.g. events for data collection to NWDAF by Nupf_EventExposure_Notify.

The SMF Area Identity and UE IPv4 Address Ranges and/or IPv6 Prefix Range(s) are optional in the UPF Provisioning Information.

******************* End of Changes ****************