SA WG2 Meeting #158	S2-2309958

# Goteborg, Sweden, August 21 – 25, 2023        	  	  			            (revision of S2-2309660)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0161 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter V-SMF mobility registration update procedure in HR-SBO case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, LG Electronics, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | § 6.7.2.7 contains Editor's note: This procedure needs a figure and a more precise description needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a figure and providing a more precise description in § 6.7.2.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.7.2.7	Inter V-SMF mobility registration update procedure in HR-SBO case

This clause defines the procedure for inter V-SMF intra-VPLMN and inter-PLMN mobility registration update procedure for HR-SBO PDU Sessions. This procedure is based on I-SMF insertion/change/removal defined in clauses 4.23.3 and 4.23.4.3 of TS 23.502 [3].

Figure 6.7.2.7-1: Inter V-SMF mobility registration update in HR-SBO

1.	The UE establishes a HR-SBO PDU Session as described in clause 6.7.2.2.

2.	The UE initiates Mobility Registration Update procedure to the Target AMF as described in clause 4.23.3 and 4.2.2.2.2 of TS 23.502 [3]. During the registration procedure, the target AMF (in roaming case) receives the SMF selection data including HR-SBO allowed indication per DNN/S-NSSAI from the UDM in the step 14b of the procedure in clause 4.2.2.2.2 of TS 23.502 [3].

3.	3a. (V-SMF insertion or V-SMF change) At step 17 of clause 4.2.2.2.2 of TS 23.502 [3], the Target AMF selects and inserts a V-SMF supporting HR-SBO and provides the received HR-SBO allowed indication for the established Session for DNN, S-NSSAI in the PDUSession_CreateSMContext Request sentto the target V-SMF.

3b. (V-SMF removal) in case the Target AMF is an AMF of HPLMN (the UE moves from a VPLMN to HPLMN), in step 3b-1, the Target AMF sends a PDUSession_CreateSMContext Request sent to the H-SMF. The H-SMF may select an EASDF in HPLMN and configure the DNS context in this EASDF (Neasdf_DNSContext_Create) and may select and configure UPF(s) in HPLMN as in step 3b-2. For the V-SMF removal case, the steps 4 to 10 are skipped.

4a.	(V-SMF insertion case) The (target) V-SMF retrieves SM context from H-SMF using Nsmf_PDUSession_Context Request/Response, followed by the Nsmf_PDUSession_Context Request/Response.

4b.	(V-SMF change case) The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response indicating HR-SBO support in the Request in the case of an intra-PLMN V-SMF case.

In step 4b, if source and target V-SMFs belong to same VPLMN, the SM context from the source V-SMF includes Authorization Result for HR-SBO, VPLMN Specific Offloading Information, and (if previously received by source V-SMF from H-SMF) the HPLMN address information, and the DNS Server address provided by the HPLMN.

NOTE X:	step 4b in inter PLMN case (the target V-SMF retrieves SM context from the source V-SMF) assumes inter PLMN agreements on context exchange at both AMF and V-SMF levels.

5.	As in step 4 of Figure 6.7.2.6 -1 the (target) V-SMF may invoke Neasdf_DNSContext_Create.

6.	As in step 5 of Figure 6.7.2.6 -1, the (target) V-SMF may perform UL CL/BP and local PSA insertion/change/removal.

7.	The target V-SMF invokes Nsmf_PDUSession_Update Request to the H-SMF as in step 7 of Figure 6.7.2.6-1.

8.	As in step 8 of Figure 6.7.2.6 -1, the H-SMF invokes Nudm_SDM_Get service, authorizes the request for HR-SBO based on the SM subscription data, and retrieves the VPLMN Specific Offloading Policy from H-PCF in case the HR-SBO is authorized.

9.	As in step 9 of Figure 6.7.2.6 -1, once the HR-SBO is authorized, the H-SMF sends the VPLMN Specific Offloading Information in the Nsmf_PDUSession_Update Response message to the target V-SMF.

NOTE: As the H-SMF sends VPLMN Specific Offloading Information related with the target PLMN for inter-PLMN case, this allows to support different VPLMN Specific Offloading Information between the source and the target VPLMN.

10.	As in step 9 of Figure 6.7.2.6 -1, the (target) V-SMF may configure the V-EASDF based on the received VPLMN Specific Offloading Information.

10a. The (target) V-SMF may also update the configuration of the V-UPF based on the received VPLMN Specific Offloading Information.

11.	As in step 6 of Figure 6.7.2.6-1, the target V-SMF (in the case of V-SMF removal, the H-SMF) answers to the target AMF with PDUSession_CreateSMContext Response.

12.	The registration procedure continues as defined in clause 4.2.2.2.2 of TS 23.502 [3] from step 18 on.

13. In the case of V-SMF removal or change, when, at step 17a of clause 4.23.4.3 of TS 23.502 [3], receiving Nsmf_PDUSession_ReleaseSMContext Request from the source AMF, the source V-SMF initiates a Neasdf_DNSContext_Delete if a V-EASDF was used for the PDU Session.

In the case of V-SMF insertion, when at step 9 the H-SMF sends to the V-SMF VPLMN Specific Offloading Information in Nsmf_PDUSession_Update Response, the DNS context in the old EASDF may be removed by the H-SMF using Neasdf_DNSContext_Delete service.

14.	(V-SMF insertion or V-SMF change): The target V-SMF sends the DNS information for the UE (received at step 9) and EAS rediscovery indication via PCO in a PDU Session Modification Command sent to the UE.

(In the case of V-SMF removal): the H-SMF sends the DNS information (received at step 3) for the UE and EAS rediscovery indication via PCO in a PDU Session Modification Command sent to the UE.

15. As in step 13 of Figure 6.7.2.6 -1, notification from H-SMF to the AF.

16.	As in step 14 of Figure 6.7.2.6 -1: EAS discovery procedure by UE may take place.

END OF CHANGES