3GPP TSG-SA2 Meeting #158	S2-2309737

# 21 - 25 August, 2023, Goteborg, Sweden

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0879 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification for ADRF ML Model storage and retrieval |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Except for Result Indication, the Address of Model file stored in ADRF should not be a mandatory output parameter for Nadrf_MLModelManagement_StorageRequest and Nadrf_MLModelManagement_Retrieval Request; |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Refine output parameters for Nadrf_MLModelManagement_StorageRequest; - Refine output parameters for Nadrf_MLModelManagement_Retrieval Request; - Some other miscellaneous corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether output parapmeter for ADRF services is mandatory or optional. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5B.1, 10.3.2, 10.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

# 5B	Analytics Data Repository Functional Description

## 5B.1	General

The ADRF offers services that enable a consumer to store and retrieve data and analytics. The analytics are produced by the NWDAF as described in clause 6.1 and data are collected as described in clause 6.2.

Data may be stored in the ADRF by:

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageRequest containing the data or analytics to be stored. The ADRF response provides a result indication.

-	a consumer sending the ADRF an Nadrf_DataManagement_StorageSubscriptionRequest requesting that the ADRF subscribes to receive data or analytics for storage. The ADRF then subscribes to the NWDAF or DCCF for data or analytics, providing ADRF Notification Address (+Notification Correlation ID). Analytics or Data are subsequently provided as notifications using DCCF, NWDAF or MFAF services (Ndccf_DataManagement Nnwdaf_DataManagement or Nmfaf_3caDataManagement service).

Data may be retrieved from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_RetrievalRequest request to the ADRF to retrieve data or analytics for a Storage Transaction Identifier or a Fetch Instructions received from the ADRF in an Nadrf_DataManagement_RetrievalNotify. The ADRF determines the availability of the data or analytics in its repository and sends in the response to the consumer either the data or analytics.

-	a consumer sending an Nadrf_DataManagement_RetrievalSubscribe request to the ADRF to retrieve data or analytics for a specified data or analytics collection time window. If the time window includes the future and the ADRF has subscribed to receive the data or analytics, subsequent notifications received by the ADRF are sent by the ADRF to the notification endpoint.

The ADRF determines the availability of the data or analytics and sends a success/failure indication in the response to the consumer. The ADRF then sends one or more notifications using an Nadrf_DataManagement_RetrievalNotify to the Notification Address (+Notification Correlation ID) specified by the consumer. The notification(s) either provide the data or analytics or provide instructions to the endpoint to fetch the data or analytics using an Nadrf_DataManagement_RetrievalRequest.

Data may be deleted from the ADRF by:

-	a consumer sending an Nadrf_DataManagement_Delete request.

An ADRF may be configured to register the data it is collecting with a DCCF. The registration uses the Ndccf_ContextManagement service specified in clause 8.3.2. The registration may subsequently be used by the DCCF to obtain data from the ADRF as described in clause 6.2.6.3.6.

The ADRF offers services that enable a consumer to store, delete and retrieve ML models. The ML models are trained by NWDAF containing MTLF as described in clause 5.1.

ML model(s) may be retrieved from the ADRF by:

-	a consumer sending the ADRF an Nadrf_MLModelManagement_Retrieve Request as described in clause 10.3.4, containing the Analytics ID(s), one or more tuples of unique ML Model identifier stored in ADRF or Storage Transaction Identifier. The ADRF response provides the address (e.g., URL) of ML model file stored in ADRF.

ML models may be stored in the ADRF by:

-	a consumer sending the ADRF an Nadrf_MLModelManagement_StorageRequest as described in clause 10.3.2, containing the ML model or ML model address to be stored. The ADRF response provides a result indication.

ML model may be deleted from the ADRF by:

-	a consumer sending an Nadrf_MLModelManagement_Delete request as described in clause 10.3.3. The ADRF response provides a result indication.

When a consumer requests data or analytics be stored in an ADRF, it may specify Storage Handling information requesting:

-	a lifetime for how long the data or analytics should be stored; and

-	that a notification alerting the consumer be sent prior to data deletion from the ADRF.

The ADRF, DCCF or NWDAF may be configured with default operator storage policies. The policies specify how long data or analytics are to be stored and conditions when the default policy supersedes Storage Handling Information provided in a request from the Data or Analytics Consumer. Based on the default operator policy and the Storage Handling information, the ADRF, DCCF or NWDAF determines the Storage Approach that will be applied for the data or analytics.

The Storage Approach is comprised of the life-time for how long the data or analytics will be stored and the notification requirement to be applied when data or analytics are to be deleted from the ADRF.

When more than one consumer requests a storage life-time for the same data or analytics, the Storage Approach should be based on the longest requested storage life-time.

The response to the consumer request for data or analytics includes the Storage Approach.

NOTE:	The default operator policy for how long data or analytics are to be stored may be longer or shorter than the life-time requested by the consumer. A default operator policy may for example accept only consumer requested life-times that are shorter or longer than the default policy.

The ADRF, DCCF or NWDAF determines when the data or analytics life-time has expired. When data or analytics is to be removed from the ADRF, an alert is sent to the Consumer that the data is about to be deleted. The alert contains a Storage Transaction Identifier that can be used by the consumer to retrieve the data or analytics.

# * * * * Next changes * * * *

### 10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store or update ML model(s). ML model(s) or ML model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: NF instance ID of the NWDAF containing MTLF and set of:

-	the tuple (unique ML Model identifier and address (e.g. URL or FQDN) of Model file and Storage size required for each of the ML model).

NOTE:	The ADRF can download ML model files based on the ML model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML model and locally stores the ML model is left for implementation.

Inputs, Optional: None.

Outputs Required: Result Indication, one of the following:

-	[Conditional] one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Storage Transaction Identifier.

# * * * * Next changes * * * *

### 10.3.4	Nadrf_MLModelManagement_Retrieval Request service operation

Service operation name: Nadrf_MLModelManagement_Retrieval Request

Description: The consumer NF uses this service operation to request the ADRF to get the ML model(s) stored in ADRF.

Inputs, Required:

-	Storage Transaction Identifier; or

-	one or more tuples of unique ML Model identifier(s).

Inputs, Optional: None.

Outputs Required: Result Indication.

-	[Conditional] one or more tuples of unique ML Model identifiers and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: None.

* * * End of Changes * * *