3GPP TSG-WG SA2 Meeting #158	S2-2308907

# Goteborg, Sweden, August 21 – 25, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0898 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove EN in Network Performance analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The analytics subset is provided by NWDAF according to the AOI. How the analysis subset is determined depends on the implementation of the NWDAF. Therefore, editor’s note is not required. Remove the Editor note: Editor's note: Area subsets smaller than cells are FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove Editor note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The network performance analytics in finer granulariy is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Observed statistics during the Analytics target period |
| > Area subset(Note X) | List if TA(s) or Cell ID(s) within the requested area of interest as defined in clause 6.6.1. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| > Analytics target period subset | Time window within the requested Analytics target period as defined in clause 6.6.1. If a Temporal granularity size was provided in the request or subscription, the duration of the Analytics target period subset is greater than or equal to the Temporal granularity size. |
| > gNB status information (NOTE 1) | Average ratio of gNBs that have been up and running during the entire Analytics target period in the area subset |
| > gNB resource usage (NOTE 1) (NOTE 2) | Usage of assigned resources |
| > gNB resource usage for GBR traffic (NOTE 1) (NOTE 2) | Usage of assigned resources for GBR traffic (average, peak) |
| > gNB resource usage for Delay-critical GBR traffic (NOTE 1) (NOTE 2) | Usage of assigned resources usage for Delay-critical GBR traffic (average, peak) |
| > Number of UEs (NOTE 1) | Average number of UEs observed in the area subset |
| > Communication performance (NOTE 1) | Average ratio of successful setup of PDU Sessions |
| > Mobility performance (NOTE 1) | Average ratio of successful handover |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: Average usage of assigned resources for uplink and downlink traffic is provided. The average and peak usage is provided as a percentage. |  |

Editor's note:	The possibility to provide the gNB resource usage for GBR traffic and gNB resource usage for Delay-critical GBR traffic based on existing network performance management parameters defined in TS 28.552 [8] is to be confirmed by SA WG5.

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Predicted analytics during the Analytics target period |
| > Area subset | List if TA(s) or Cell ID(s) within the requested area of interest as defined in clause 6.6.1. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| > Analytics target period subset | Time window within the requested Analytics target period as defined in clause 6.6.1. If a Temporal granularity size was provided in the request or subscription, the duration of the Analytics target period subset is greater than or equal to the Temporal granularity size. |
| > gNB status information (NOTE 1) | Average ratio of gNBs that will be up and running during the entire Analytics target period in the area subset |
| > gNB resource usage (NOTE 1) (NOTE 2) | Usage of assigned resources (average, peak) |
| > gNB resource usage for GBR traffic (NOTE 1) (NOTE 2) | Usage of assigned resources for GBR traffic (average, peak) |
| > gNB resource usage for Delay-critical GBR traffic (NOTE 1) (NOTE 2) | Usage of assigned resources usage for Delay-critical GBR traffic (average, peak) |
| > Number of UEs (NOTE 1) | Average number of UEs predicted in the area subset |
| > Communication performance (NOTE 1) | Average ratio of successful setup of PDU Sessions |
| > Mobility performance (NOTE 1) | Average ratio of successful handover |
| > Confidence | Confidence of this prediction |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: Average usage of assigned resources for uplink and downlink traffic is provided. The average and peak usage is provided as a percentage. |  |

Editor's note:	The possibility to provide the gNB resource usage for GBR traffic and gNB resource usage for Delay-critical GBR traffic based on existing network performance management parameters defined in TS 28.552 [8] is to be confirmed by SA WG5.

NOTE 1:	The predictions are provided with a Validity Period, as defined in clause 6.1.3.

NOTE 2:	The analytics on number of UEs are related to the information retrieved from the AMFs.

The number of network performance information entries is limited by the maximum number of objects provided as part of Analytics Reporting Information.

The NWDAF provides Network Performance Analytics to a consumer at the time requested by the consumer in the Analytics target period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters at the time indicated in the Analytics target period.

# * * * * End of changes * * * *