3GPP TSG-SA3 Meeting #112 	S3-234295

# Gothenburg, Sweden, 14 -18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1744 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security handling in network sharing scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Huawei, HiSilicon, Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A description regarding how security is handled in MOCN network sharing scenario is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A description of security handling in MOCN is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of description for security handling in MOCN newtork sharing scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex W.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF 1st CHANGE*******

## W.4.X	Security handling in network sharing scenario

In Multiple-Operator Core Network (MOCN) network sharing scenarios, if MBS content protection is needed, then the content may be protected at the application layer (c.f., W.4.2).

NOTE x:	The content protection mechanism used at the application layer is outside the scope of 3GPP.

If service-layer security specified in clause W.4.1.3 of the present document is used in MOCN network sharing scenarios, the AF may be required to generate the MSK, MSK ID, MTK and MTK ID common to the participating PLMNs. The AF provides the generated MSK, MSK ID, MTK and MTK ID to the MBSSF(s) of the participating PLMNs, when creating broadcast MBS sessions and also when key update is required. Furthermore, it is up to the AF to select a MCC and MNC among the PLMNs for the Key Domain ID. As mentioned in clause 6.3.2.1 of TS 33.246 [102], the UE should not try to use the MCC and MNC in another context, e.g., the UE should not compare the received MCC || MNC to parameters in radio level.

NOTE z: Security aspects between the AF and the MBSSF(s) is outside the scope of 3GPP.

NOTE y: If the MBSSF uses MSK and MTK generated by itself to protect the traffic, the resource sharing during network sharing will not be used.

************ END OF 1st CHANGE*******