| 3GPP TR 28.925 V1.0.0 (2023-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on enhancement of service based management architecture (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Intpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Issue investigations and potential issue solutions	10

4.1	Issue #1:  Scope of specifications	10

4.1.1	Description	10

4.1.1.1	28.533	10

4.1.1.2	28.621	10

4.1.1.3	28.622	10

4.1.1.4	28.623	11

4.1.1.5	28.662	11

4.1.2	Potential solutions	11

4.1.2.1	Solution #1-1 scope of 28.533 [2]	11

4.1.2.2	Solution #1-2 SBMA for Generic NRM	12

4.1.2.2.1	28.621	12

4.1.2.2.1.1	Solution #1-2.a Update TS28.621 [43]	12

4.1.2.2.1.2	Solution #1-2.b New TS for SBMA	12

4.1.2.2.1.3	Solution #1-2.c New TSs for IRP and SBMA	12

4.1.2.2.1.4	Solution #1-2.d update TS28.621 [43] scope	12

4.1.2.2.1.5	Solution #1-2.e Merge stage 1, 2 and 3.	12

4.1.2.2.2	28.622	12

4.1.2.2.2.1	Solution #1-2.f update TS28.622 [23]	12

4.1.2.2.2.2	Solution #1-2.g New TS for SBMA	12

4.1.2.2.2.3	Solution #1-2.h New TSs for IRP and SBMA	12

4.1.2.2.2.4	Solution #1-2.i update TS28.622 [23] scope	12

4.1.2.2.2.5	Solution #1-2.j Merge stage 1, 2 and 3.	12

4.1.2.2.3	28.623	12

4.1.2.2.3.1	Solution #1-2.k update TS28.623 [44]	12

4.1.2.2.3.2	Solution #1-2.l New TS for SBMA	13

4.1.2.2.3.3	Solution #1-2.m New TSs for IRP and SBMA	13

4.1.2.2.3.4	Solution #1-2.n update TS28.623 [44] scope	13

4.1.2.2.3.5	Solution #1-2.o Merge stage 1, 2 and 3.	13

4.1.2.4	Solution #1-4 new SBMA TS for Generic RAN NRM	13

4.2	Issue #2: Content errors	13

4.2.1	Description	13

4.2.2	Potential solutions	14

4.2.2.1	Solution #2-1 3GPP specific information in TSs classified as generic	14

4.2.2.2	Solution #2-2 for stage 1 content in stage 2 TS 28.533 [2]	14

4.3	Issue #3: Reference errors	14

4.3.1	Description	14

4.3.2	Potential solutions	14

4.3.2.1	Solution #3-1 Make not used references void	14

4.4	Issue #4: SBMA supporting manangement of 5G SA and NSA scenarios	14

4.4.1	Description	14

4.4.1.1	Analysis of the existing specification capabilities	14

4.4.1.2	Management support for NG-RAN Overall Architecture	15

4.4.1.3	Management support for EN-DC Overall Architecture	16

4.4.2	Potential solutions #4-1	17

4.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs	18

4.5.1	Description	18

4.5.2	Potential solutions	18

4.5.2.1	ETSI ISG ZSM	18

4.5.2.1.1	Introduction	18

4.5.2.1.2	Potential solutions #5-1	19

4.6	Issue #6: Software management feature in SBMA for 5G	19

4.6.1	Description	19

4.6.2	Potential solutions	19

4.7	Issue #7: Inventory missing in 5G	19

4.7.1	Description	19

4.7.2	Potential solutions	19

4.8	Issue #8: Use of Models in SBMA	19

4.8.1	Description	19

4.8.2	Potential solutions	20

4.9 	Issue #9: Improvement on the management function description	23

4.9.1	Description	23

4.9.2	Potential solutions	25

4.9.2.1	Management function description	25

4.9.2.2	Usage of CRUD operations and NRM fragment in SBMA	25

4.9.2.3	Notifications supported by management sets in SBMA	26

4.10	Issue #10: Use of 32.401 in SBMA	26

4.10.1	Description	26

4.10.2	Potential solutions	26

4.11	Issue #11: Use of 32.404 in SBMA	27

4.11.1	Description	27

4.11.2	Potential solutions	27

4.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101	27

4.12.1	Description	27

4.12.2	Potential solutions	28

4.13	Issue #13:  Analysis on IRP specification needs to support SBMA	29

4.13.1	Description	29

4.13.2	Potential solutions	30

4.14	Issue #14: Providing a usage guide for SBMA series of specifications	33

4.14.1	Description	33

4.14.2	Potential solutions	33

4.15	Issue# 15: Extend TS 32.300 to support SBMA	33

4.15.1	Description	33

4.15.2	Potential solutions	33

5	Conclusion and Recommendation	34

5.1	Issue #1 Scope of specifications	34

5.1.1	Solution #1-1 scope of 28.533 [2]	34

5.1.2	Solution #1-2 SBMA for Generic NRM	34

5.1.2.1	Issue #1-2.a:  No requirements specification generic NRM for SBMA	34

5.1.2.2	Issue #1-2.b: Scope of TS28.622 [23]	34

5.1.2.3	Issue #1.2.c: No clear solution set definition for generic NRM in SBMA	34

5.1.3	Solution #1-3 scope of referred TSs in 28.533 [2].	34

5.1.4	Solution #1-4 new SBMA TS for Generic RAN NRM	34

5.2	Issue #2 Content errors	34

5.3	Issue #3 Reference errors	35

5.3.1	Solution #3-1 Make not used references void	35

5.4	Issue #4 SBMA supporting management of 5G SA and NSA scenarios	35

5.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs	35

5.6	Issue #6: Software management feature in SBMA for 5G	35

5.7	Issue #7 Inventory missing in 5G	35

5.8	Issue #8: Use of Models in SBMA	35

5.9 	Issue #9: Improvement on the management function description	35

5.10	Issue #10: Use of 32.401 in SBMA	36

5.11	Issue #11: Use of 32.404 in SBMA	36

5.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101	36

5.13	Issue #13: Analysis on management features for SBMA support	36

5.14	Issue #14: Providing overview for SBMA series of specifications	37

Annex A: Change history	38

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document studies on the potential enhancement of service based management architecture based on the existing 5G service based management architecture.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[4]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[5]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[6]	ETSI GS ZSM 002: “Zero-touch network and Service Management (ZSM); Reference Architecture”.

[7]	3GPP TS 28.510: "Configuration Management (CM) for mobile networks that include virtualized network functions; Requirements".

[8]	3GPP TS 28.511: "Configuration Management (CM) for mobile networks that include virtualized network functions; Procedures".

[9]	3GPP TS 28.512: "Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 2".

[10]	3GPP TS 28.513: "Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 3".

[11]	3GPP TS 28.515: "Fault Management (FM) for mobile networks that include virtualized network functions; Requirements".

[12]	3GPP TS 28.516: "Fault Management (FM) for mobile networks that include virtualized network functions; Procedures".

[13]	3GPP TS 28.517: "Fault Management (FM) for mobile networks that include virtualized network functions; Stage 2".

[14]	3GPP TS 28.518: "Fault Management (FM) for mobile networks that include virtualized network functions; Stage 3".

[15]	3GPP TS 28.520: "Performance Management (PM) for mobile networks that include virtualized network functions; Requirements".

[16]	3GPP TS 28.521: "Performance Management (PM) for mobile networks that include virtualized network functions; Procedures".

[17]	3GPP TS 28.522: "Performance Management (PM) for mobile networks that include virtualized network functions; Stage 2".

[18]	3GPP TS 28.523: "Performance Management (PM) for mobile networks that include virtualized network functions; Stage 3".

[19]	3GPP TS 28.525: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[20]	3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[21]	3GPP TS 28.527: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 2".

[22]	3GPP TS 28.528: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 3".

[23]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[24]	3GPP TS 32.103: "Integration Reference Point (IRP) overview and usage guide".

[25]	3GPP TS 28.537: "Management capabilities ".

[26]	3GPP TS 32.425: "Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN".

[27]	3GPP TS 28.552: "5G performance measurements".

[28]	3GPP TS 28.500, "Concepts, use cases and requirements".

[29]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[30]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM Information Service (IS)".

[31]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements".

[32]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[33]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[34]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[35]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[36]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[37]	3GPP TS 32.156 "Telecommunication management; Fixed Mobile Convergence (FMC); Model repertoire".

[38]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[39]	3GPP TS 32.404: "Telecommunication management; Performance Management (PM); Performance measurements; Definitions and template".

[40]	3GPP TS 25.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[41]	3GPP TS 25.433 "UTRAN Iub interface Node B Application Part (NBAP) signalling".

[42]	3GPP TS 32.300: “Telecommunication management; Configuration Management (CM); Name convention for Managed Objects”.

[43]	3GPP TS 28.621: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[44]	3GPP TS 28.623: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[45]	3GPP TS 28.532: "Management and orchestration; Generic management services; "

[46]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[47]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)”

[48]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management"

[49] 	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3"

[50] 	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

[51] 	3GPP TS 28.538: "Management and orchestration; Edge Computing Management"

[52] 	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) "

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSS	Business Support System

MnS	Management Service

PM	Performance Management

SBMA	Service Based Management Architecture

WI	Work Item

# 4	Issue investigations and potential issue solutions

## 4.1	Issue #1:  Scope of specifications

### 4.1.1	Description

#### 4.1.1.1	28.533

TS 28.533 [2] (the TS is only valid for 5G and exists in Rel-15,16 and 17):

Scope:

There is no limitation in the scope that the TS is only valid for 5G (or rather not valid for 2G, 3G and legacy 4G.

There are no borders for where the SBMA is valid, which gives some problems when dealing with external entities governed by other fora, e.g. interfaces with MANO, towards verticals, other operators etc. All that control is left to access control, which is not (yet) specified. There is a link with the studies FS_MNSAC and FS_NSCE.

References:

28.510 [7], 28.511 [8], 28.512 [9], 28.513 [10], 28.515 [11], 28.516 [12], 28.517 [13], 28.518 [14], 28.520 [15], 28.521 [16], 28.522 [17], 28.523 [18], 28.525 [19], 28.526 [20], 28.527 [21] and 28.528 [22] are referred, but those TSs are not valid for 5G, They are used in Annex A.4, which is informative.

#### 4.1.1.2	28.621

TS: 28.621 [43] (The TS is valid for 3G, 4G and 5G and is valid for IRPs only while up to and including Rel-14 but from and including Rel-15 to and including the current release (Rel-18) it is valid for both IRP and SBMA:

The current TS28.621 [43] has a title which is IRP onlywhile the content is valid for IRP architecture as well as SBMA. It is not clear which Stage 1 requirements are applicable for SBMA.

#### 4.1.1.3	28.622

TS: 28.622 [23] (The TS is valid for 3G, 4G and 5G and is valid for IRPs only while up to and including Rel-14 but from and including Rel-15 to and including the current release (Rel-18) it is valid for both IRP and SBMA):

The current 3GPP TS 28.622 [23] has following issues:

- The current 3GPP TS 28.622 [23] has an IRP only title: “3rd Generation Partnership Project Technical Specification Group Services and System Aspects Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP) Information Service (IS) (Release 18)”. However, the specification is applied also for Service Based Management Architecture (SBMA).

- Section 1 Scope: The scope is covering IRP architecture only. No SBMA description. This is issue is covered in section 4.1 issue #1.

- Section 2 References:

  - A few references are not used. This is issue is covered in section 4.3 issue #3.

  - Some referred TSs are IRP only which is not applicable for SBMA architecture.

- Section 3.1 Definitions: There are definitions covering IRP architecture. However, which definitions are applicable for IRP or SBMA or both are unclear.

- Section 4.2 Class diagrams: This section has mixed IRP and SBMA information. Some of the information are only applicable for IRP.  Some are only applicable to SBMA. Some are applicable for both SBMA and IRP. The text in this section is not clear which part is applicable for which architecture.

- Section 4.3 Class definitions:

  - This section has the same issue as “Class diagrams” section. It mixes class definitions for IPR architecture only, SBMA architecture only, or both. It is not clear which part is applicable to which architecture.

  - Some classes applicability is not clear enough. The follows are a few examples only, other classes are not list here:

    - ‘EP_RP’ is applicable for both IRP architecture and SBMA architecture. But it has an SBMA only attribute “supportedPerfMetricGroups”. The attribute list becomes very confused.

    - Another example is that ‘ThresholdMonitor’ is defined for SBMA architecture. But the text does not make it clear if it is (or not) also applicable for IRP architecture.

- Section 4.4 Attribute definitions: This section has the same issue as “Class diagrams” section. It contains mixed information for IRP only, SBMA only, or both. It is not clear which part is applicable to which architecture. One example is in above bullet F.b.

- Section 4.5 Common notifications: The description text is applicable for SBMA only. However, the notifications shall be applicable for both IRP and SBMA architecture.

#### 4.1.1.4	28.623

TS: 28.623 [44] (The TS is valid for 3G, 4G and 5G and is valid for IRPs only while up to and including Rel-14 but from and including Rel-15 to and including the current release (Rel-18) it is valid for both IRP and SBMA):

The title, the scope, the definition, and the Solution Set (SS) definitions of current 3GPP TS 28.623 [44] are all IRP architecture. However, the specification also contains SBMA IOCs descriptions. It is very hard to understand which part is applicable to which architecture.

Different solution sets are not consistent among themselves.

#### 4.1.1.5	28.662

TS: 28.662 [52] (The TS is valid for 3G, 4G and 5G and exists in Rel-15, 16 and 17):

This TS is made for IRP architecture, but is applied also for Service Based Management Architecture, SBMA.

This NRM would be beneficial to be included in SBMA, but an update is needed.

### 4.1.2	Potential solutions

#### 4.1.2.1	Solution #1-1 scope of 28.533 [2]

a.	Introduce the limitation that the TS is not valid for 2G, 3G and 4G in the Scope clause.
b.	Introduce text that 28.533 [2] is valid for SBMA.
c.	Introduce text that access rights for interfaces are outside the scope for this TS (this is be replaced with a TS number when such a TS is published). 
Also to be discussed whether access rights per external (and internal) interfaces are to be documented in some way.

#### 4.1.2.2	Solution #1-2 SBMA for Generic NRM

##### 4.1.2.2.1	28.621

###### 4.1.2.2.1.1	Solution #1-2.a Update TS28.621 [43]

Update the current TS28.621 [43] title to cover SBMA architecture NRM and add any Stage 1 requirements for generic NRM in SBMA.

###### 4.1.2.2.1.2	Solution #1-2.b New TS for SBMA

Keep the current TS28.621 [43] for IRP only. Create a new TS for Stage 1 for generic NRM in SBMA requirements.

###### 4.1.2.2.1.3	Solution #1-2.c New TSs for IRP and SBMA

Create a new TS for generic NRM IRP (with the IRP content from 28.621 [43]) and create a new TS for Stage 1 for generic NRM in SBMA requirements.

###### 4.1.2.2.1.4	Solution #1-2.d update TS28.621 [43] scope

Update the current TS28.621[43] in the scope clause that Rel-14 is the version valid for IRP while the current release is valid for SBMA, because up to and including Rel-14 TS28.621[43] is valid only for IRPs while from Rel-15 up to and including the current release (Rel-18) it is valid for both IRP and SBMA (see issue description for issue#1 28.621).

###### 4.1.2.2.1.5	Solution #1-2.e Merge stage 1, 2 and 3.

New specifications may be merged into one TS for stage 1, 2 and 3.

##### 4.1.2.2.2	28.622

###### 4.1.2.2.2.1	Solution #1-2.f update TS28.622 [23]

Update the current TS28.622 [23] with enough clarifications to cover both IRP and SBMA architecture NRM.

###### 4.1.2.2.2.2	Solution #1-2.g New TS for SBMA

Create a new TS for generic NRM in SBMA and maintain TS 28.622 [23] for IRP information only.

###### 4.1.2.2.2.3	Solution #1-2.h New TSs for IRP and SBMA

Create a new TS for generic NRM information service in SBMA and keep TS28.622 [23] as is (with a note that any SBMA related updates will be continued in the new TS after R18.

###### 4.1.2.2.2.4	Solution #1-2.i update TS28.622 [23] scope

Update the current TS28.622[23] in the scope clause that Rel-14 is the version valid for IRP while the current release is valid for SBMA, because up to and including Rel-14 TS28.622 [23] is valid only for IRPs while from Rel-15 up to and including the current release (Rel-18) it is valid for both IRP and SBMA (see issue description for issue#1 28.622).

###### 4.1.2.2.2.5	Solution #1-2.j Merge stage 1, 2 and 3.

New specifications may be merged into one TS for stage 1, 2 and 3.

##### 4.1.2.2.3	28.623

###### 4.1.2.2.3.1	Solution #1-2.k update TS28.623 [44]

Update the current TS28.623 [44] title, scope, definition, and the Solution Set (SS) definitions to cover both IRP and SBMA architecture NRM.

###### 4.1.2.2.3.2	Solution #1-2.l New TS for SBMA

Create a new TS for Stage 3 Solution Set (SS) definitions for SBMA architecture NRM. This new stage 3 TS can be combined with the proposed new stage 2 TS.

Keep TS28.623 [44] for IRP only.

###### 4.1.2.2.3.3	Solution #1-2.m New TSs for IRP and SBMA

Create a new TS for generic NRM information service in SBMA and keep TS28.623 [44] as is (with a note that any SBMA related updates will be continued in the new TS after R18.

###### 4.1.2.2.3.4	Solution #1-2.n update TS28.623 [44] scope

Update the current TS28.623 [44] in the scope clause that Rel-14 is the version valid for IRP while the current release is valid for SBMA, because up to and including Rel-14 TS28.622 [23] is valid only for IRPs while from Rel-15 up to and including the current release (Rel-18) it is valid for both IRP and SBMA (see issue description for issue#1 28.622.

###### 4.1.2.2.3.5	Solution #1-2.o Merge stage 1, 2 and 3.

New specifications may be merged into one TS for stage 1, 2 and 3.

#### 4.1.2.3	Solution #1-3 scope of referred TSs in 28.533 [2]

The TSs 28.510 [7], 28.511 [8], 28.512 [9], 28.513 [10], 28.515 [11], 28.516 [12], 28.517 [13], 28.518 [14], 28.520 [15], 28.521 [16], 28.522 [17], 28.523 [18], 28.525 [19], 28.526 [20], 28.527 [21] and 28.528 [22] are present in an informative annex so they can be removed.

#### 4.1.2.4	Solution #1-4 new SBMA TS for Generic RAN NRM

Replace 28.662 [52] with a new 5G TS for SBMA with information only applicable to SBMA. New information might be introduced. IRP applicable information is kept in 28.662 [52], which would be valid for older systems using IRP the architecture.

## 4.2	Issue #2: Content errors

### 4.2.1	Description

TS 28.533 [2]:

Content error:

TS 32.103 [24] claims that 28.533 [2] is a "generic" TS (in contrast to "5G specific"), yet clause 4.6 is using 5G entities, clause 5.2 and 5.4 are specifying 3GPP management system interactions with NFV MANO and NWDAF.
Clause 4.2.2: MnS component type A is said to be generic wrt operations and notifications. But is really PM notifications etc. generic?

Clause 6 contains use cases and requirements, which should not be included in a stage 2 TS.

28.533 [2] clams to be the architecture TS for SBMA, but it does not have any boundaries for where it is valid (e.g. is service management and BSS included?).

Implementation errors:

Clause 6: The requirement tag REQ-MnSD-FUN-X has not been given a number.

### 4.2.2	Potential solutions

#### 4.2.2.1	Solution #2-1 3GPP specific information in TSs classified as generic

28.533 [2] clause 4.2.2: Discuss whether operations and notifications used in 5G that are specific for a certain functionality is included in MnS component type A which is stated to be generic. The TS is to be changed accordingly to the agreement.

Atl.1	28.533 [2] should be generic, so the content which are not generic should be deleted, 
Alt.2	28.533 [2] should be 5G specific, so all information about generic and 5G specific TSs should be removed.

#### 4.2.2.2	Solution #2-2 for stage 1 content in stage 2 TS 28.533 [2]

Move clause 6 in 28.533 [2] to 28.537 [25].

Change REQ-MnSD-FUN-X to REQ-MnSD-FUN-1 (but not in 28.533 [2]).

## 4.3	Issue #3: Reference errors

### 4.3.1	Description

TS 28.533 [2]:

32.101 [3] is referenced, but that TS is not valid for 5G. But the reference is only present in clause 2.

32.425 [26] is referenced, but that TS is not valid for 5G (also 28.552 [27] refers to 32.425 [26])

28.500 [28] is referenced, but that TS is not valid for 5G.

28.622 [23] is referenced, but that is an IRP TS (but used for 5G).

TS: 28.622 [23]:

32.111-2 [29] is referenced, but the reference is only present in Clause 2.

32.662 [30] is referenced, but the reference is only present in Clause 2.

### 4.3.2	Potential solutions

#### 4.3.2.1	Solution #3-1 Make not used references void

  - Make reference to 32.101 [3] void in 28.533 [2].

  - Make the references to 32.111-2 [29] and 32.662 [30] void in 28.622 [23].

## 4.4	Issue #4: SBMA supporting manangement of 5G SA and NSA scenarios

### 4.4.1	Description

#### 4.4.1.1	Analysis of the existing specification capabilities

Service based management architecture (SBMA) and corresponding MnS are introduced in 5G management architecture in TS 28.533 [2], and a different management reference model (i.e. IRP) had been used to manage the network before 5G era in 3GPP TS32.101 [3].

The analysis is based on the following understanding of the existing specification capabilities:

- The management mechanism of LTE supports interface IRP and NRM IRP models.

- The management mechanism of 5G supports MnS which includes MnS component type A (Operation/Notification), MnS component type B (NRM models) and MnS component type C (Alarm/Performance information).

- LTE NRM (with enhancement of YAML or YANG solution set) can be used as MnS component type B and work together with MnS component type A.

#### 4.4.1.2	Management support for NG-RAN Overall Architecture

As description in 3GPP TS 38.300 [4], an NG-RAN node is either a gNB or an ng-eNB which are interconnected with each other by means of the Xn interface and connected with 5GC by means of the NG interface, more specifically to the AMF by means of the NG-C interface and to the UPF by means of the NG-U interface. The NG-RAN architecture is introduced in 3GPP TS 38.300 [4], as follows.

Figure 4.4.1.2-1: NG-RAN Overall Architecture

NOTE: The ng-eNB node provides E-UTRA user plane and control plane protocol terminations towards the UE, and connects via the NG interface to the 5GC.

In order to provide management support for NG-RAN, the 3GPP management system needs to support the management for gNB, ng-eNB and 5GC. There are potential 2 management options to support, as follows.

NG-RAN management Option#1

Figure 4.4.1.2-2: NG-RAN management Option#1

In the NG-RAN management Option#1:

- The gNB management domain provides MnS (including MnS component type A, B and C) for the management of gNB.

- The ng-eNB management domain provides IRP (including interface IRP and NRM IRP) for the management of ng-eNB.

- The 5GC management domain provides MnS (including MnS component type A, B and C) for the management of 5GC.

NG-RAN management Option#2

Figure 4.4.1.2-3: NG-RAN management Option#2

In the NG-RAN management Option#2:

- The gNB management domain provides MnS(including MnS component type A, B and C) for the management of gNB

- The ng-eNB management domain provides MnS ((including MnS component type A, B and C) for the management of ng-eNB. In this case, ng-eNB NRM used as MnS component type B, which means the YAML/YANG solution set for ng-eNB needs to be provided.

- The 5GC management domain provides MnS (including MnS component type A, B and C) for the management of 5GC.

#### 4.4.1.3	Management support for EN-DC Overall Architecture

NG-RAN supports Multi-Radio Dual Connectivity (MR-DC) operation whereby a UE in RRC_CONNECTED is connected to two different nodes, one providing NR access and the other one providing either E-UTRA or NR access. One node acts as the MN and the other as the SN. The MN and SN are connected via a network interface and at least the MN is connected to the core network (e.g. EPC).

The following figure illustrates the MR-DC with EPC (i.e. EN-DC) architecture in TS 37.340 [5].

Figure 4.4.1.2-4: EN-DC Overall Architecture

NOTE: the en-gNB node provides NR user plane and control plane protocol terminations towards the UE, and acts as Secondary Node in EN-DC.

In order to provide management support for EN-DC, 3GPP management system needs to provide the management for en-gNB, eNB and EPC. There are also potential 2 management options to support, as follows.

EN-DC management Option#1

Figure 4.4.1.2-5: EN-DC management option#1

In EN-DC management option#1,

- The en-gNB management domain provides MnS (including component type A, B and C) for management of en-gNB.

- The eNB management domain provides IRP (including interface IRP and NRM IRP) for the management of eNB.

- The EPC management domain provides IRP (including interface IRP and NRM IRP) for the management of EPC.

Figure 4.4.1.2-6: EN-DC management Option#2

EN-DC management Option#2

In EN-DC management option#2,

- The en-gNB management domain provides MnS (including component type A, B and C) for management of en-gNB.

- The eNB management domain provides MnS (including component type A, B and C) for management of eNB. In this case, eNB NRM used as MnS component type B, which means the YAML/YANG solution set for eNB NRM needs to be provided.

- The EPC management domain provides MnS (including component type A, B and C) for management of EPC. In this case, EPC NRM used as MnS component type B, which means the YAML/YANG solution set for EPC NRM needs to be provided.

### 4.4.2	Potential solutions #4-1

The above potential 4 management mechanism options described in clause 4.4.1 could be classified as two management options, as follows.

Figure 4.4.2-1: Management Option A and B for 5G SA and NSA

Option#A (including above NG-RAN management Option#1 and EN-DC management Option#1)

- The legacy nodes (e.g. eNB, ng-eNB and EPC) management domain provides IRP (including interface IRP and NRM IRP) for the management of legacy nodes.

- The 5G nodes (e.g. gNB, en-gNB and 5GC) provides MnS (including MnS component type A, B and C) for the management of 5G nodes.

Option#B (including above NG-RAN management Option#2 and EN-DC management Option#2)

- The legacy nodes (e.g. eNB, ng-eNB and EPC) management domain provides MnS (including MnS component type A, B and C) for the management of legacy node. In this case, legacy node NRM used as MnS component type B, which means the YAML/YANG solution set for legacy node NRM needs to be provided.

- The 5G nodes (e.g. gNB, en-gNB and 5GC) provides MnS (including MnS component type A, B and C) for the management of 5G nodes.

## 4.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs

### 4.5.1	Description

There are some relevant works on the architectures and frameworks for automation of management and orchestration in other Standards Developing Organizations (SDOs) in industry. This clause provides a brief overview on the works in the related SDOs.

### 4.5.2	Potential solutions

#### 4.5.2.1	ETSI ISG ZSM

##### 4.5.2.1.1	Introduction

The goal of ETSI ZSM is to enable zero-touch automated network and service management in a multivendor environment.

As documented in ETSI GS ZSM 002 [6], it provides the ZSM framework reference architecture as shown in figure 4.5.2.1.1-1 and distributed management and data services, organized into management domains and integrated via an integration fabric. The integration fabric is used to enable management service consumption, communication, and integration with 3rd party management systems. The cross-domain data service allows data sharing across domains. All management services provide a set of capabilities for their consumption.

Figure 4.5.2.1.1-1: ZSM framework reference architecture

In figure 4.5.2.1.1-1, every management domain, as well as the E2E service management domain, provides a set of ZSM service capabilities by management functions that expose and/or consume a set of service end-points. The cross-domain integration fabric facilitates providing capabilities and accessing end-points cross-domain. Some services are only provided and consumed locally inside the management domain.

##### 4.5.2.1.2	Potential solutions #5-1

3GPP management service deployment framework based on ZSM framework reference architecture is captured in clause 5.3 of TS 28.533 [2]. 3GPP SBMA is aligned with ZSM framework reference architecture.

## 4.6	Issue #6: Software management feature in SBMA for 5G

### 4.6.1	Description

3GPP TS 32.531[31], TS 32.532[32], TS 32.533[33] define the requirements, Information Solution and CORBA solution set for software management which includes Automated software management and non-automated software management. 3GPP TS 28.525 [19] and 28.526[20] describes the management aspects for VNF application software (i.e. 3GPP network function software), which includes reusing the software Management IRP [32] for the interaction between NM and EM. The management of software for VNF itself is not defined in 3GPP, which is in the scope of ETSI NFV.  Software Management IRP [32] is applicable for the deployments using IRP Framework, which is not applicable for the deployments using SBMA.

The 5G management is using the SBMA, so corresponding software management solution is missing.

### 4.6.2	Potential solutions

Introduce a software management feature in SBMA which can be applicable for 5G and future network is needed. The management of software for VNF itself is not included, which is defined by ETSI NFV.

## 4.7	Issue #7: Inventory missing in 5G

### 4.7.1	Description

LTE has specifications about inventory: 3GPP TS 28.631 [34], 3GPP TS 28.632 [35] and 3GPP TS 28.633 [36] that define the requirements, Network Resource Model Information Service (IS) and Solution Set. 5G does not have anything similar, even though 5G also consist of software, hardware, network functions and licences.

The attribute table is not follow the latest template documented in TS 32.156 [37], for example, the legal value for “isReadable”, “isWritable”,”invariant” and “isNotifyable” should be True or False.

### 4.7.2	Potential solutions

Replace 3GPP TS 28.631 [34], 3GPP TS 28.632 [35] and 3GPP TS 28.633 [36] with a new 5G TSs for SBMA.

- Provide the YANG and YAML solution sets for inventory management NRM in SBMA.

## 4.8	Issue #8: Use of Models in SBMA

### 4.8.1	Description

The following existing concepts are related to SBMA:

- TS 28.533 [2] Management Function: A Management Function (MnF) is a logical entity playing the roles of MnS consumer and/or MnS producer. Management Function can be deployed as a separate entity or embedded in Network Function to provide MnS(s).

Figure 4.8-1: Examples of MnS deployment scenario

- TR 21.905 Network Element: A discrete telecommunications entity which can be managed over a specific interface e.g. the RNC.

- TS 28.533 [2] / TS 23.501 Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

The following IOCs are specified to represent Network Element and Network Function:

- TS 28.622 [23] ManagedElement IOC: This IOC represents telecommunications equipment or TMN entities within the telecommunications network providing support and/or service to the subscriber. A ManagedElement IOC is used to represent a Network Element defined in TS 32.101[3] including virtualization or non-virtualization scenario.

- TS 28.622 [23] ManagedFunction IOC: This IOC is provided for sub-classing only. It provides attribute(s) that are common to functional IOCs. Note that a ManagedElement may contain several managed functions, a managed function may contain other managed functions as specified for the specific subclass. This IOC can represent a telecommunication function either realized by software running on dedicated hardware or realized by software running on NFVI.

The following IOCs as specified in TS 28.622 [23] are used to represent Management Function provided by MnS Producer:

- TS 28.622 [23] MnsAgent IOC: The MnsAgent represents the MnS producers, incl. the supporting hardware and software, available for a certain management scope that is related to the object name-containing the MnS Agent.

- TS 28.622 [23] ManagementNode IOC: This IOC represents a telecommunications management system (EM) within the TMN that contains functionality for managing a number of ManagedElements (MEs). (Note: This description is to be updated to replace use of EM)

### 4.8.2	Potential solutions

This clause provides the description of using NRM to represent management of management function, network function, and network element.

Management Function could be deployed in following different deployment scenarios:

- The Management Function can be deployed in management system, including

  - The Management Function deployed in domain management system.

  - The Management Function deployed in cross domain management system.

- The Management Function deployed in Network Element.

From the three deployment scenarios of Management Functions above. Management Functions deployed in domain management system and Management Function deployed in Network Element need to be managed as managing network node in 3GPP management system. The way of managing the Management Function are modelled in corresponding management function IOC.

ManagementNode IOC represents a telecommunications management system within the TMN provided by vendor that contains management functionalities for managing a number of ManagedElements (MEs).

The Management Functions are managed by corresponding management function IOC as defined in TS 28.541 [46], TS 28.104 [47], TS 28.105 [48], TS 28.536 [49] and TS 28.312 [50]:

- IOC for Management of D-SON function:

- D-SON of ANR in TS 28.541 [46] DANRManagementFunction IOC

- D-SON Energy Saving (ES) functions in TS 28.541 [46] DESManagementFunction IOC

- D-SON RACH functions in TS 28.541 [46]  DRACHOptimizationFunction,

- D-SON MRO functions in TS 28.541 [46] DMROFunction,

- D-SON PCI Configuration functions in TS 28.541 [46] DPCIConfigurationFunction,

- D-SON LBO functions in TS 28.541 [46] DLBOFunction,

- IOC for Management of management functions:

- EM centralized SON Energy Saving (ES) functions in TS 28.541 [46] CESManagementFunction IOC

- EM centralized-SON PCI Configuration functions in TS 28.541 [46] CPCIConfigurationFunction,

- EM centralized-SON Energy Saving (ES) functions in TS 28.541 [46] CESManagementFunction,

- EM centralized-SON CCO functions in TS 28.541 [46] CCOFunction

- MDAFunction in TS 28.104 [47]

- AIMLTrainingFunction in TS 28.105 [48]

- AssuranceClosedControlLoop in TS 28.536 [49]

Editor’s note: other management functions to be added with working progress. The criteria for identifying what kind of ManagementFunction should be managed is FFS.

The following table captures the relation between the concepts and related models:

Table 4.8.1-1 Relation between the concepts and related models

| Concepts | Illustration of Related Management Models |
| --- | --- |
| TS 28.622 [23] ManagementNode | A telecommunications management system within the TMN provided by vendor that contains management functionalities for managing a number of ManagedElements (MEs) are represented by ManagementNode IOC defined in TS 28.622 [23], and a number of management functions. |
| TS 28.533 [2] Management Function | The corresponding XXXManagementFunction IOC is used to support the managing of XXXMnF. |
| TR 21.905 Network Element | A network element is represented by ManagedElement IOC defined in TS 28.622 [23]. |
| TS 28.533 [2] / TS 23.501 Network Function | A Network function is represented by ManagedFunction IOC defined in TS 28.622 [23]. |

It’s recommended the following IOCs which could be used to represent MnFs to be managed. The modelling is not changed for existing functions to keep the backward compatibility.

Table 4.8.1-2 IOCs which could be used to represent MnFs to be managed

| TS | IOC | Inheritant from | Contained by |
| --- | --- | --- | --- |
| TS 28.541 [46] | - CPCIConfigurationFunction - CESManagementFunction - CCOFunction | Top | - ManagedEntity(SubNetwork/ManagedElement/NRCellDU/ManagementNode) - ManagedEntity(SubNetwork/ManagedElement/NRCellCU/ManagementNode) - SubNetwork/ManagementNode |
| TS 28.104 [47] | MDAFunction | ManagedFunction | MDAEntity (SubNetwork/ManagedElement/ManagedFunction/ManagementNode) |
| TR 28.105 [48] | AIMLTrainingFunction | ManagedFunction | ManagedEntity(Subnetwork/ManagedElement/ManagementFunction/ManagementNode) |
| TS 28.536 [49] | AssuranceClosedControlLoop | Top | SubNetwork/ManagedElement/ManagementNode |

Editors’ Note: whether the MnFs to be managed IOCs are needed to be inherited from a same root IOC is FFS.

The ManagementFunction IOCs listed in Table 4.8-2 can be used to support different deployment scenarios based on which managed entity contain such Management Function IOCs. For example, following Figure 4.8.2-1 is one example for management function model, in which all ManagementFunction IOCs (i.e. MDAFunction, AIMLtrainingFunction, AssuranceClosedControlLoop, CESManagementFunctuion, CCOFunctuion and CPCIConfigurationFunction) are contained by SubNetwork.

Figure 4.8.2-1 An example of management function model

Above example for management function model can be used to support following deployment scenario for management function, see Figure 4.8.2-2. In this deployment scenario example, RAN network management provides the following management capabilities (including MDA capabilities, AIML Training capabilities, AssuranceClosedControlLoop capabilities, CESManagement capabilities, CCO capabilities and CPCIConfigurationcapabilites).

Figure 4.8.2-2 An example of deployment scenario for management function contained by SubNetwork

The ManagementFunctions in management function model in Figure 4.8.2-1 provides the capabilities to allow cross domain management to control these management capabilities via corresponding MnSs. See below:

  - MDAFunction is used to represent MDA capabilities in RAN network management.

  - AIMLtrainingFunction is used to represent AIMLTraining capabilities in RAN network management.

  - AssuranceClosedControlLoop is used to represent AssuranceClosedControlLoop capabilities in RAN network management.

  - CESManagementFunctuion is used to represent CESManagement capabilities in RAN network management.

  - CCOFunctuion is used to represent CCO capabilities in RAN network management.

  - CPCIConfigurationFunction is used to represent CPCIConfiguration capabilities in RAN network management.

## 4.9 	Issue #9: Improvement on the management function description

### 4.9.1	Description

The following information is documented in TS 32.101 [3] to describe the TMN architecture aspects and the management functions.

Analysis:

- TS 32.101 [3] defines the management principles and high-level requirements for the management of PLMNs. Some descriptions in corresponding area in clause 7 of TS 32.101 [3] can be improved and used for SBMA.

2. The following management features are captured in annex of TS 28.533 [2], as follows.

Figure 4.9.1-1: Overview of 5G management specifications

The related management features description in TS 28.533 [2] are limited compared with the management functions description in TS 32.101 [3].

### 4.9.2	Potential solutions

#### 4.9.2.1	Management function description

Elaborate the management functional overviews in TS 28.533 [2] with taking into account of the management function description information in TS 32.101 [3] clause 7.

#### 4.9.2.2	Usage of CRUD operations and NRM fragment in SBMA

Following is the overview of usage of CRUD operations and NRM fragments in SBMA.

If CRUD operations are supported, it is not needed to support the equivalent non-CRUD operations. (e.g. if CRUD operations for NtfSubscriptionControl IOC are supported, it is not needed to support subscribe/unsubscribe operations).

Note: The usage of non-CRUD operations in the SBMA are not listed in the following table (e.g. Streaming management operations, defined in 28.532 [45] section 11.5.1, by Streaming Data Reporting MnS are not-CRUD operations).

Table 4.9.2-1 Overview of the usage of CRUD operations and NRM fragments in SBMA

| Supported MnS Components |  | Usage |
| --- | --- | --- |
| MnS Component type A | MnS Component type B |  |
| CRUD operations | Generic NRM (TS 28.622 [23]) (e.g. SubNetwork, ManagedElement IOC) | Common Management |
| CRUD operations | NRM fragment for NR (TS 28.541 [46]) | NR Provisioning |
| CRUD operations | NRM fragment for NRM 5GC (TS 28.541 [46]) | 5GC Provisioning |
| CRUD operations | NRM fragment for Network Slice (TS 28.541 [46]) | Network Slice Provisioning |
| CRUD operations | PM control NRM fragment (TS 28.622 [23]) | Performance |
| CRUD operations | Threshold monitoring control NRM fragment (TS 28.622 [23]) | Performance Threshold Monitoring |
| CRUD operations | Trace control NRM fragment (TS 28.622 [23]) | Trace |
| CRUD operations | QoE Measurement Collection control NRM fragment (TS 28.622 [23]) | QoE |
| CRUD operations | FM control NRM fragment (TS 28.622 [23]) | Fault Supervision |
| CRUD operations | File retrieval NRM fragment (TS 28.622 [23]) | File Retrieval |
| CRUD operations | File download NRM fragment (TS 28.622 [23]) | File Download |
| CRUD operations | Notification subscription control NRM fragment (TS 28.622 [23]) | Subscription |
| CRUD operations | Heartbeat notification control NRM fragment (TS 28.622 [23]) | Heartbeat |
| CRUD operations | NRM fragment for ML training (TS 28.105 [48]) | AI/ML Management |
| CRUD operations | NRM fragment for MDA request and MDA report (TS 28.104 [47]) | MDA |
| CRUD operations | NRM fragment for DANR Management (TS 28.541 [46]) NRM fragment for DES Management (TS28.541 [46]) NRM fragment for DRACH Management (TS28.541 [46]) NRM fragment for DMRO Management (TS28.541 [46]) NRM fragment for DPCI Management (TS 28.541 [46]) NRM fragment for CES Management (TS 28.541 [46]) NRM fragment for CPCI Management (TS 28.541 [46]) NRM fragment for DLBO Management (TS 28.541 [46]) NRM fragment for CCO Management (TS 28.541 [46]) | SON Policy |
| CRUD operations | NRM fragment for intent (TS 28.312 [50]) | Intent Driven Management |
| CRUD operations | Edge NRM Fragment (TS 28.538 [51]) | Edge Computing Provisioning |
| CRUD operations | ManagementDataCollection control NRM fragment (TS 28.622 [23]) | ManagementDataCollection |
| CRUD operations | MnS Registry NRM fragment (TS 28.622 [23]) | MnS Registry and Discovery |
| CRUD operations | Assurance management NRM fragment (TS28.536 [49]) | Communication Service Assurance |

Note: the term “Usage” needs to be revisited in the normative phase.

#### 4.9.2.3	Notifications supported by management sets in SBMA

TBD

## 4.10	Issue #10: Use of 32.401 in SBMA

### 4.10.1	Description

Concepts and definitions in 32.401 [38] are used in 28.552 [27], Therefore 32.401 [38] is referred to in 28.552 [27].

32.401 [38] is written for an IRP architecture, for which it is still needed. To include SBMA in 32.401 [38] will be almost duplicating the specification.

### 4.10.2	Potential solutions

It is proposed to make

- a new TS for Performance Management (PM); Concept and requirements or

- to include changes into 28.550 [28].

in a new work item in Rel-18.

## 4.11	Issue #11: Use of 32.404 in SBMA

### 4.11.1	Description

Major issues found:

- Concepts, definitions and the template in 32.404 [39] are used in 28.552, therefore 32.404 [39] is referred to in 28.552 [27].

- Some terms (e.g. IRP and NE) used are not valid in SBMA.

- Some normative references are only to IRP TSs, e.g. references to 25.331 [40] and 25.433 [41].

- The PM LCM in Annex B.5 is different than the CM LCM. It should be investigated whether they should be aligned/changed.

### 4.11.2	Potential solutions

It is proposed to update TS 32.404 [39] in a new work item in Rel-18.

## 4.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101

### 4.12.1	Description

In TS 32.101[3], Clause 5.1 illustrates the management reference model which shows the Operations Systems interfacing with other systems. An Operations System supports management interfaces to other systems.

Figure 1: Management reference model

A number of management interfaces in a PLMN are identified in figure 1, namely:

1)	between the Network Elements (NEs) and the Element Manager (EM) of a single PLMN Organisation;

2)	between the Element Manager (EM) and the Network Manager (NM) of a single PLMN Organisation;

NOTE:	In certain cases, the Element Manager functionality may reside in the NE in which case this interface is directly from NE to Network Manager). These management interfaces are given the reference name Itf-N and are the primary target for standardization.

3)	between the Network Managers and the Enterprise Systems of a single PLMN Organisation;

4)	between the Network Managers (NMs) of a single PLMN Organisation;

4a) between the Domain Managers (DMs) of a single PLMN Organisation.

5)	between Enterprise Systems & Network Managers of different PLMN Organisations;

5a) between the Domain Managers (DMs) of different PLMN Organisations.

6)	between Network Elements (NEs).

7)	between the Network Management Layer Service (NMLS) and the Network Manager (NM).

IRPs may be implemented at interfaces 2, 3, 4, 5 and 7.

TS 28.533[2] introduces the Service Based Management Architecture (SBMA). The fundamental building block of the Service Based Management Architecture (SBMA) is the Management Service (MnS). A MnS is a set of offered capabilities for management and orchestration of network and services. An MnS producer offers its services via a standardized service interface composed of individually specified MnS components (MnS component type A, B, C).

Analysis:

- In TS 32.101 [3], there is clearly showing the entities and the corresponding interfaces in the management reference model.

- In TS 28.533 [2], the interaction of paradigm of MnS producer and MnS Consumer is defined without indicating the entities.

- The entities in TS 32.101 [3] can be illustrated using the MnS consumer and MnS producer according to the way of using interface IRPs.

### 4.12.2	Potential solutions

There are the following aspects to be considered in the solution of using MnS.

- The management services may be used to support the interoperability in different management scenarios.

a)	Interoperability within Operation system of a single PLMN Organisation

b)	Interoperability between BSS and Operation systems within a single PLMN Organization

c)	Interoperability between BSS of different PLMN Organisations.

d)	Interoperability between Operation Systems of different PLMN Organisations.

- Different set of management services may be used for the interoperability between different entities.

- Figure 4.12.2-1 illustrates an example of using management services for supporting interoperability in a multiple players scenario:

Figure 4.12.2-1: Example of possible usage of MnS

## 4.13	Issue #13:  Analysis on IRP specification needs to support SBMA

### 4.13.1	Description

The overview of interface IRP and NRM IRP are illustrated in clause 5 and clause 6 in TS 32.103[24] as follows:

Figure 4.13-1: 3GPP Interface IRPs

Figure 4.13-2: 3GPP NRM IRPs

Some interface IPR specifications (e.g. Basic CM IRP, Alarm IRP) have been developed to corresponding SBMA specifications (e.g. provisiong MnS, fault supervision MnS), some NRM IRP specifications (e.g. Generic NRM IRP) are reused for SBMA, there are still some IRP specifications haven't produced corresponding SBMA specifications.

### 4.13.2	Potential solutions

Following table 4.13.2-1 provides the overview analysis on 3GPP Interface IRPs and the related SBMA specifications.

Table 4.13.2-1: overview analysis on 3GPP Interface IRPs and the related SBMA specifications

| 3GPP Interface IRP specifications |  | Whether correponding SBMA specification is needed | Status |
| --- | --- | --- | --- |
| Generic/Common Interface IRPs | Name convention for Managed Objects (32.300) | Yes | The name convention is used in SBMA. |
|  | Notification IRP (32.301/2/6) | Yes | SBMA specification is already defined (TS 28.532 [45]) |
|  | Generic IRP (32.31x) | Not needed as it’s management of interface IRPs |  |
| FM related interface IRPs | Alarm IRP (32.111-x) | Yes | SBMA specification is already defined (TS 28.532 [45]) |
|  | Advanced Alarm Management IRP (32.12x) | To be further investigated |  |
|  | Test management IRP (32.32x) | discontinue MnS as there is lack of requirements. |  |
| CM related interface IRPs | Basic CM IRP (32.60x) | Yes | SBMA specification is already defined (TS 28.532 [45]) |
|  | Bulk CM IRP (32.61x) | Yes | YANG (RFC 7950) and Netconf (RFC6241) allows updating a single boolean attribute/leaf or the whole configuration. Individual and bulk are not handled separately. Restful SBMA specification is already defined (TS 28.532 [45]) |
|  | Kernel CM IRP (32.66x) | Yes | YANG (RFC 7950) and Netconf (RFC6241) allows updating a single boolean attribute/leaf or the whole configuration. Individual and bulk are not handled separately. Part of operations defined in Kernel CM IRP have already a SBMA specification, e.g. notifyMOICreation, notifyMOIDeletion, etc. (cf. TS 28.532 [45]). |
| PM file format/collection & Trace related Interface IRPs | Performance Management IRP (32.41x) | Yes | SBMA specification is already defined (TS 28.532 [45] and TS 28.550 [28]) |
|  | Performance Measurement collection (32.43x) | Yes | SBMA specification is already defined (TS 28.532 [45] and TS 28.550 [28]) |
|  | Trace Management IRP (32.42x, 32.44x) | Yes | Already reused for SBMA |
|  | Notification Log IRP (32.33x) | Yes | Aready covered by FS_eSBMAe SID (see FS_eSBMAe_WoP#7) |
| Supporting interface IRPs | File Transfer IRP (32.34x) | Yes | SBMA specification is already defined (TS 28.537 [25] and TS 28.622 [23]) |
|  | Communication Surveillance IRP (32.35x) | Yes | SBMA specification is already defined (TS 28.537 [25],TS 28.622 [23], TS 28.532 [45]) |
|  | Entry Point IRP (32.36x) | Yes | SBMA specification is already defined (5GDMS, TS 28.537 [25], TS 28.622 [23]) |
| Other interface IRPs | Partial Suspension of Itf-N IRP (32.38x) | discontinue MnS as there is lack of requirements. |  |
|  | Delta synchronization IRP (32.39x) | discontinue MnS as there is lack of requirements. |  |
| SON related Interface IRPs | Self-Configuration IRP (32.50x) | Yes | SBMA specification for PnC is already defined (TS 28.314/5/6) SBMA specification for SBMA is covered by RANSC WI |
|  | Software Management IRP (32.53x) | Not addressed in the present document. | Analysis is covered by Issue #6 in clause 4.6 |

Following table 4.13.2-2 gives the overview analysis on  3GPP NRM IRPs to derive which IRP spepcifications needs to be convert to SBMA specifications in the future.

Table 4.13.2-2: overview analysis on 3GPP NRM IRPs and the related SBMA specifications

| 3GPP NRM IRP specifications |  | Whether correponding SBMA specification is needed | Status |
| --- | --- | --- | --- |
| FMC model | Umbrella Information Model (28.620) | Yes | Reused for SBMA |
| Common NRM IRP | Generic NRM IRP (28.621/2/3) | Yes | Covered by Issue #4 in clause 4.1. |
| 3GPP Access Network (AN) related NRM IRPs | Generic RAN NRM IRP (28.661/2/3) | Yes | Covered by Issue #4 in clause 4.3 |
|  | GERAN NRM IRP (28.654/5/6) | Not applicable |  |
|  | UTRAN NRM IRP (28.651/2/3) | Not applicable |  |
|  | E-UTRAN NRM IRP (28.657/8/9) | Yes | Covered by Issue #4 in clause 4.3 |
|  | Generic RAN NRM IRP 28.661/2/3) | Yes | There is a dependency from 28.541 [46] to 28.662 [52] |
| 3GPP Core Network (CN)/IMS related NRM IRPs | CN NRM IRP (28.701/2/3) | Not applicable |  |
|  | CN NRM IRP (28.704/5/6) | Not applicable |  |
|  | EPC NRM IRP (28.707/8/9) | Not applicable |  |
| 3GPP H(e)NB related NRM IRPs | HNS NRM IRP (28.671/2/3) | Not applicable |  |
|  | HeNS NRM IRP (28.674/5/6) | Not applicable |  |
| Service management related NRM IRPs | SuM NRM IRP (28.751/2/3) | discontinue MnS as there is lack of operators requirements. |  |
| Supporting NRM IRPs | Inventory Management NRM IRP (28.631/2/3) | Yes | Covered by Issue #7 in clause 4.7 and5.4. They are already included in (informative) annex E in TS 28.533 [2] about 5G specifications though still lacking SBMA-compatible solution set |
|  | Transport Network NRM IRP (28.731/2/3) | Not applicable |  |
|  | Signalling Transport NW IF NRM IRP (28.734/5/6) | Not applicable |  |
|  | State management data definition IRP (28.624/5&) | Yes | Most of the relevant contents from 28.625 have been copied into 28.622 [23] /28.541 [46]. |
| Self-Organizing Networks NRM IRPs | SON Policy NRM IRP (28.627/8/9) | Not applicable |  |
| Non-3GPP access interworking NRM IRPs | EPC and non-3GPP access interworking system Network Resource Model (NRM) IRP (28.611/2/6) | Not applicable |  |
| Wireless Local Area Network NRM IRP | WLAN NRM IRP (28.681/2/3) | Not applicable |  |

## 4.14	Issue #14: Providing a usage guide for SBMA series of specifications

### 4.14.1	Description

TS 32.103 has captured the Telecommunication management; Integration Reference Point (IRP) overview and usage guide for managing of 2G, UMTS and LTE network. The usage guide in TS 32.103 is very useful for external readers to get an overview of how IRP series of specifications.

### 4.14.2	Potential solutions

It is recommended to create a new specification to capture the overview and usage guide information for SBMA series of specifications.

## 4.15	Issue# 15: Extend TS 32.300 to support SBMA

### 4.15.1	Description

TS 32.300 [42] is written for IRP based architecture, the same level information is also needed for 5G/SBMA. To extend the support to SBMA, there are issues which need to be resolved :

- The scope of this specification is set to IRP, e.g., in section 1 scope.

- There are specific references to IRP specifications, e.g. in the definitions, abbreviations, and section 7.4 refers to TS 32.103 [24] for IRP, etc.

- Some contents are limited to IRP scope, e.g. some statements are prefixed with IRP.

- There are references to IRP SS, XML.

### 4.15.2	Potential solutions

The following changes are needed to support SBMA:

- Update the scope to be generic applicable to both IRP and SBMA.

- Complement the existing reference to IRP specifications, the same level information shall be extended for SBMA, e.g., section 7.4 of TS 32.103 [24].

- For the content and statement that are limited to IRP scope, they shall be lifted to support SBMA. e.g., section 4, 5 and 6 of TS 32.300 [42].

- For those specific references to IRP SS, make them to be generic applicable to both IRP and SBMA. e.g., section 6 and 7 of TS 32.300 [42].

# 5	Conclusion and Recommendation

## 5.1	Issue #1 Scope of specifications

## 5.1.1	Solution #1-1 scope of 28.533 [2]

It is recommended to not pursue the solution#1-1.

## 5.1.2	Solution #1-2 SBMA for Generic NRM

### 5.1.2.1	Issue #1-2.a:  No requirements specification generic NRM for SBMA

From Rel-19 and later releases, solution #1.2.c is recommended for normative work. Solution #1.2.e may be applied.

For Rel-15 – 18 solution #1.2.d is recommended for normative work as maintenance work in Rel-15 to Rel-18.

### 5.1.2.2	Issue #1-2.b: Scope of TS28.622 [23]

From Rel-19 and later releases, solution #1-2.h is recommended for normative work. Solution #1-2.j may be applied.

For Rel-15 – 18 solution #1-2.i is recommended for normative work as maintenance work in Rel-18.

Note: There are ongoing working items in SA5, e.g., S5-232083 DP on restructuring TS 28.545, which may have impacts on the detail steps of this this solution.

### 5.1.2.3	Issue #1.2.c: No clear solution set definition for generic NRM in SBMA

From Rel-19 and later releases, solution #1-2.m is recommended for normative work. Solution #1-2.o may be applied.

For Rel-15 – 18 solution #1-2.n is recommended for normative work as maintenance work in Rel-18.

## 5.1.3	Solution #1-3 scope of referred TSs in 28.533 [2].

It is recommended to not pursue the solution#1-3.

## 5.1.4	Solution #1-4 new SBMA TS for Generic RAN NRM

Replace 28.662 [52] with a new 5G TS for SBMA, including stage 1, 2 and 3.

## 5.2	Issue #2 Content errors

It is recommended that the errors are corrected by maintenance CRs.

## 5.3	Issue #3 Reference errors

## 5.3.1	Solution #3-1 Make not used references void

It is recommended to:

- Make the reference to 32.101 [3] void in 28.533 [2].

as a maintenance CR.

## 5.4	Issue #4 SBMA supporting management of 5G SA and NSA scenarios

Two management options are identified for 5G SA and NSA in section 4.4.2.

It is recommended that the NG-RAN management Option#2 is to be supported by specifying YANG and YAML solution sets in a new TS. It may be an own stage 3 TS or included in the stage 2 TS.

## 5.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs

3GPP SBMA is aligned with ZSM framework reference architecture, and related description is captured in TS 28.533[2]. No further modification is needed to existing specifications.

## 5.6	Issue #6: Software management feature in SBMA for 5G

The SBMA software management solution is not addressed in the present document due to lack of requirements.

## 5.7	Issue #7 Inventory missing in 5G

Replace 3GPP TS 28.631 [34], 3GPP TS 28.632 [35] and 3GPP TS 28.633 [36] with one or several new 5G TSs for SBMA for stage 1, 2 and 3.

Propose a Work Item to include inventory NRM in SBMA.

## 5.8	Issue #8: Use of Models in SBMA

It’s recommended to add IOC summary in TS 28.533 [2] Annex E (informative): 5G specifications overview to provide the overview of IOC information which currently captured in multiple specifications. The following grouping could be considered as starting inputs:

- IOCs representing network resources as defined in SA2 and RAN3 (e.g. Network Element, Network Function)

- IOCs representing Management of D-SON functions as shown in section 4.8.2.

- IOCs representing Management of management functions as shown in section 4.8.2.

## 5.9 	Issue #9: Improvement on the management function description

It is recommended to consider the descriptions in TS 32.101 [3] clause 7 where applicable.

## 5.10	Issue #10: Use of 32.401 in SBMA

Replace 32.401 [38] with a new 5G TS for SBMA.

## 5.11	Issue #11: Use of 32.404 in SBMA

Update 32.404 [39] to include SBMA.

## 5.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101

A MnS is a set of offered capabilities for management and orchestration of network and services, SBMA provides the following flexibility:

1.	SBMA does not limit who can play the role of an MnS consumer. Any entities consume SA5 defined MnS are called MnF.

2.	Depending on different scenarios, an entity either plays the role as MnS producers or as MnS consumers.

It is recommended to add examples of using MnS in different scenarios to illustrate the flexibility of Service Based Management Architecture (SBMA).

## 5.13	Issue #13: Analysis on management features for SBMA support

The list below identifies management features in IRP framework that have similar functionality supported in SBMA:

For management operations related:

- Name convention for Managed Objects

- Notification.

- Alarm

- Basic CM

- Bulk CM

- Kernel CM

- Performance Management

- Performance Measurement collection

- Trace Management

- File Transfer

- Communication Surveillance

- Entry Point

For NRM related:

- Umbrella Information Model

- Generic NRM

- Generic RAN NRM

- E-UTRAN NRM

- Generic RAN NRM

- State management data definition

The list below identifies management features in IRP framework that have similar functionality to be supported in SBMA:

For management operations related:

- Notification Log

- Self-Configuration

For NRM related:

- Inventory Management NRM

## 5.14	Issue #14: Providing overview for SBMA series of specifications

SBMA related management features are currently documented in multiple specifications. An overview including the high level summary description and dependency relationship between multiple specifications could help readers to get full picture of series of SBMA specifications.

It is recommended to create a new specification to capture the overview for SBMA series of specifications.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04-2021 | SA5#137e | S5-213042 |  |  |  | Initial skeleton | 0.0.0 |
| 04-2021 | SA5#137e | S5-213043 |  |  |  | Correction on title | 0.0.1 |
| 05-2021 | SA5#137e | S5-213572 S5-213573 S5-213574 |  |  |  | Update approved tdocs in SA5#137e | 0.1.0 |
| 09-2021 | SA5#138e | S5-214451 S5-214649 |  |  |  | First conclusions for 28.533 First conclusion for NSA | 0.2.0 |
| 10-2021 | SA5#139e | S5-215617 S5-215618 |  |  |  | Update approved tdocs in SA5#139e: Add key issue on software management feature in SBMA for 5G Inventory for 5G | 0.3.0 |
| 11-2021 | SA5#140e | S5-216235 S5-216594 |  |  |  | Add potential solution and conclusion for Issue#5 Add key issue on modelling of MnF | 0.4.0 |
| 04-2022 | SA5#142e | S5-222629 S5-222630 |  |  |  | Add issue on the applicable content from TS 32.101 section 7 Proposal to update 32.401 and 32.404 for SBMA | 0.5.0 |
| 05-2022 | SA5#143e | S5-223579 S5-223581 |  |  |  | Add issue on illustration of using MnS in management reference model in TS 32.101. Add issue on analysis on IRP specifications needs to support SBMA. | 0.6.0 |
| 07-2022 | SA5#144e | S5-224160 |  |  |  | Add conclusion for overview and usage guide for SBMA specifications | 0.7.0 |
| 07-2022 | SA5#145e | S5-225799 S5-225800 S5-225670 |  |  |  | Update of the TR 28.925, due to the study being moved to Rel-18 and new proposals pCR TR28.925 Add conclusion for issue #14 pCR TR28.925 Add description on MnFs to be managed | 0.8.0 |
| 11-2022 | SA5#146 | S5-226236 S5-226963 |  |  |  | rapportuer reorganizing modification for section 5 Add conclusion for issue 4.8 | 0.9.0 |
| 03-2023 | SA5#147 | S5‑232317 S5‑232952 S5‑232953 |  |  |  | pCR TR 28.925 Rapporteur cleanup pCR 28.925 Add an example for management function deployment scenarios pCR TR 28.925 Add issue about extending TS 32.300 to support SBMA editorial update. | 0.a.0 |
| 05-2023 | SA5#149 | S5‑234843 S5‑234509 S5‑234510 S5‑234277 S5‑234512 S5‑234513 S5‑234514 S5‑234515 S5‑234516 S5‑234517 S5‑234518 S5‑234519 S5‑234520 S5‑234274 S5‑234521 S5‑234511 |  |  |  | S5‑234843 pCR TR 28.925 Add conclusion and recommendation for issue#9 Improvement on the management function description S5‑234509 pCR TR 28.925 Update solution and add conclusion and recommendation for issue#12 illustration of using MnS in management reference model in TS 32.101 S5‑234510 pCR TR 28.925 Update solution and add conclusion and recommendation for issue#13 Analysis on IRP specification needs to support SBMA S5‑234277 Conclusion for issue 2 S5‑234512 pCR 28.925 Issue description for 28.621 S5‑234513 pCR 28.925 Solution description for 28.621 S5‑234514 pCR 28.925 Conclusion for 28.621 S5‑234515 pCR 28.925 Issue description for 28.622 S5‑234516 pCR 28.925 Solution description for 28.622 S5‑234517 pCR 28.925 Conclusion for 28.622 S5‑234518 pCR 28.925 Issues for 28.623 S5‑234519 pCR 28.925 Solution description for 28.623 S5‑234520 pCR 28.925 Conclusion for 28.623 S5‑234274 Removal of Editor’s note in 28.925-0a0 S5‑234521 pCR TR 28.925 Add overview for management service features supported by CRUD operations in SBMA S5‑234511 pCR TR 28.925 Add conclusion and recommendation for issue#6 software management feature in SBMA for 5G Add references information for 28.621/28.623/28.532/28.541/28.104/28.105/28.536/28.312/28.538/28.662 | 0.b.0 |
| 2023-09 | SA#101 | SP-230930 |  |  |  | Presented for information and approval | 1.0.0 |