TSG SA Meeting #SP-101	SP-231108

11 - 15 September, 2023, Bangalore, India

# Title:	SERP feature status and proposal

# Source:	Apple, Ericsson

# Document for:	Discussion

Agenda Item:	3.4

Abstract:	This contribution provides background information for CR1791 to TS 33.501 in SP-231111 on Protection of the RRC Resume Request message

# 1	Background

WID scope:

The SERP WID was approved in S3-221185 (SP-220538) in SA3#107-e in May 2022, which is the security enhancement for protecting RRCResumeRequest messages, following the conclusion on the solution#17 in TR 33.809. The objectives include:

- Specify the enhancement on the source and target gNB/ng-eNB

- Specify the enhancement on the UE to support the new RRCResumeRequest MAC-I

- Specify the enhancement on the Xn interfaces between gNBs/ng-eNBs.

The conclusion on solution#17 (S3-221072) is quoted below:

“It is concluded that solution #17 is taken as the basis of normative work for the protection of RRCResumeRequest message.”

Discussion history in SA3:

After 8 meeting cycles discussion (from SA3#107adhoc-e in June 2022 to SA3#111 in Berlin in May 2023), one consolidated CR (S3-234032) was generated and submitted to SA3#112 meeting. This CR was revised to S3-234314 during the SA3#112 meeting in order to add more cosigners. The CR was cosigned by 14 companies and only objected by one company during the discussion in SA3#112 meeting in August 2023. The SA3 chair decided that the best course of action was to have the discussion of the CR in the SA plenary.

The concerns raised questioned the benefit of the feature and the corresponding impact to the system. However, this is an optional feature, which is not mandatory to be implemented. Furthermore, the introduction of any new feature will have a degree of impact on the system.

The proponents of this solution believe that this proposal brings important enhancements and provides a tool to future proof the security of the system.

Technical details and mitigation methods to cover other concerns raised are described in clause 3.

# 2	Proposal

It is proposed to approve the corresponding CR1791 to TS 33.501 in SP-231111 on the SERP feature for Rel-18.

# 3	Annex: Feature description

#### 3.1 Benefit of the feature

1. With this enhancement, Every IE included in RRCResumeRequest message can be protected.

RRCResumeRequest include IEs:

The resumeCause in RRCResumeRequest message was defined as 12 different enumerated values:

“emergency, highPriorityAccess, mt-Access, mo-Signalling,
mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess, mcs-PriorityAccess, spare1, spare2, spare3, spare4, spare5”

Tampering on the resumeCause could de-prioritize the UE’s resume procedure and lead to downgrade the service that the UE is supposed to get. For example, when gNB/ng-eNB is in congestion status, UE sending RRCResumeRequest with resumeCause originally set to “highPriorityAccess” can only wait if the resumeCause is modified to, e.g “mo-SMS”. NOTE: which resumeCause could get the prioritized resource depends on gNB/ng-eNB implementation.

2. With this enhancement, the network can detect any tampering on the RRCResumeRequest message by verifying the new RRCResumeMAC-I. The security principle is that, any modified messages should not be allowed to get passed. With the failure verification, network can collect sufficient statistics on the amount of failure cases in different network areas, which will be helpful for the operator network enhancement.

#### 3.2 ResumeMAC-I enhancement and impact on the RRC Resume procedure

Solution#17 is to enhance the RRCResumeRequest message by adding the whole RRCResumeRequest message into the input of the calculation of ResumeMAC-I.

According to the baseline, the MESSAGE inputs for calculating the ResumeMAC-I are: source PCI, target Cell-ID, source C-RNTI.

With the enhancement of the feature, the MESSAGE input for calculating RRCResumeMAC-I will be: source PCI, target Cell-ID, source C-RNTI, all IEs from RRCResumeRequest message. According to the RRCResumeRequest structure, since the spare bit is always set to 0, the new added input is actually the resumeCause and resumeIdentity for the time being. However, specifying that the whole message is to be used as input for the integrity protection of the message allows a future-proof implementation as the whole message will be, by default, integrity protected if and when new IEs are added in the RRC Resume Request message in the future.

With this SERP feature, the RRCResumeRequest procedure will be enhanced as below (Figure-1):

Figure-1: Enhanced RRCResume procedure using new ResumeMAC-I

#### 3.3 Capability requirement on UE and NW

To support the enhanced ResumeMAC-I, both the UE and the NW (including the target gNB/ng-eNB and the source gNB/ng-eNB) are required to be enhanced as follows:

Table-1. Enhanced Operation for UE and NW

| Entity | Functionality | Legacy Operation | Enhanced Operation |
| --- | --- | --- | --- |
| UE | ResumeMAC-I calculation | Based on legacy input | ResumeMAC-I generation based on enhanced input |
| Target gNB/ng-eNB | Forward MAC-I and input to source gNB/ng-eNB | Forward info includes MAC-I, target cell ID, UE I-RNTI | Forward info includes MAC-I, target cell ID, UE I-RNTI, new MAC-I and the new added input |
| Source gNB/ng-eNB | MAC-I checking | Based on legacy input | Based on enhanced input |

#### 3.4 Capability negotiation on UE and NW

The precondition is that only when all of the UE, the target gNB/ng-eNB and source gNB/ng-eNB support this SERP feature, this feature can be enabled. The capability negotiation among these 3 entities is necessary.

The CR1791 to TS 33.501 showed the formal text on the negotiation procedure, below in the description on how each entity indicate its capability:

- The UE reports the support of the ResumeMAC-I enhancement via AS capability report to source gNB/ng-eNB;

- If the source gNB/ng-eNB supports this feature, it can enable the new RRCResumeMAC-I feature via the RRCRelease with SuspendConfig message when setting the UE into INACTIVE state;

- Each gNB/ng-eNB, indicates its support of the new MAC-I feature as the target gNB/ng-eNB role in the SIB1,  and. forwarding the new MAC-I associated input info (e.g. resumeCause, whole RRC Resume Request message) to source gNB/ng-eNB;

- When the INACTIVE UE moves to the target gNB/ng-eNB, UE first checks the indication in SIB1 of current serving cell:

  - If the new MAC-I feature is supported, UE can generate the new MAC-I for RRCResumeReq;

  - Otherwise, UE generates the legacy ResumeMAC-I;

- The target gNB/ng-eNB does not need to know the UE or the source gNB/ng-eNB’s capability. If the target gNB/ng-eNB supports the new MAC-I feature, and UE sends the new MAC-I, the target gNB/ng-eNB will always include new MAC-I and the new added input for the MAC-I calculation in the RETRIEVE UE CONTEXT REQUEST X2 message;

- When the source gNB/ng-eNB receives the RETRIEVE UE CONTEXT REQUEST message from the target gNB/ng-eNB,

- If the source gNB/ng-eNB is able to identify which version of ResumeMAC-I is used,  the gNB/ng-eNB will identify the UE based on the I-RNTI to check whether the UE is configured with the ResumeMAC-I enhancement.

  - If the UE is identified to enable the enhancement, the last target gNB/ng-eNB will perform the enhanced ResumeMAC-I verification;

  - Otherwise, the source gNB/ng-eNB may send the RETRIEVE UE CONTEXT failure message or perform legacy MAC-I verification depends on the local configuration. (refer to Clause 3.5 in this discussion paper)

If the source gNB/ng-eNB is NOT able to identify which version of ResumeMAC-I is used, the gNB/ng-eNB will perform the legacy ResumeMAC-I verification by default.

Table-2. The capability and the feature enable/disable operation

| Entity | Operation if supported | Operation if not supported |
| --- | --- | --- |
| UE | - Report its support of the capability to NW via AS capability signaling; - NW configures the new MAC-I enhancement via RRCRelease with SuspendConfig message; - Check the new MAC-I indication in SIB1 of the camping cell - Perform the new MAC-I calculation if current cell broadcasts the new MAC-I indication in SIB1. | - No indication in AS capability signaling; - No need to check the new MAC-I indication in the SIB1 of the camping cell; - Generate the MAC-I as legacy for the RRCResumeRequest message. |
| Source gNB/ng-eNB | - Enable the new MAC-I enhancement via RRCRelease message if the UE supports the feature; - Check whether the new MAC-I is carried in X2 message from the target gNB/ng-eNB; - Perform the new MAC-I verification if both the following conditions are fulfilled: - The UE is identified via I-RNTI to enable the feature; - The new MAC-I is carried in X2 message. | - Not configure the new MAC-I enhancement in the RRCRelease message; - Perform the ResumeMAC-I verification in legacy way. - No need to check the new MAC-I input in the X2 message |
| Target gNB/ng-eNB | - Broadcast its support of the MAC-I enhancement in SIB1; - Always include the new MAC-I and the new added input for MAC-I verification in the X2 message. | - No indication of the new MAC-I in SIB1; - No need to include the new MAC-I and new added input for MAC-I in X2 message. |

#### 3.5 Verification failure cases handling

During the discussion in SA3, there are some comments on the consideration of the error case handling, e.g. when there is some abnormal cases take place (e.g. attacking scenario changing the capability indication in SIB), how should the error cases be handled.

When the SIB message is modified, target gNB/ng-eNB capability may be not a correct one for the UE, in such cases, when the SIB message can be protected in the future, this issue will probably be mitigated. But before we adopt the SIB protection mechanism, some error handling mechanisms are still needed. Below the table-3 give a summary on how should the source gNB/ng-eNB handle the error cases.

Table-3. Verification failure cases handling operations

| Entity | Operation if supported |
| --- | --- |
| Message #1 UE -> NW(Source gNB/ng-eNB) | Case#1 - UE Support: send indication in AS capability Case#2 - UE not support: legacy AS capability |
| Message#2 NW(Source gNB/ng-eNB) -> UE | Case#1 - Source support: configure UE in RRCRelease message Case#2 - Source node not support: use legacy RRCRelease message |
| Message#3: NW (Target gNB/ng-eNB)->UE | Case#1 - Target node support: new feature indication in SIB1 Case#2 - Target node not support: use old MAC-I in RRCResumeRequest |
| Message#4: UE->NW (Target gNB/ng-eNB) | Case#1 - All Source/UE/target node support: use new MAC-I in RRCResumeRequest Case#2 - Not all the Source/UE/target node support: use old MAC-I in RRCResumeRequest |
| Message#4: X2 message (Target->Source gNB/ng-eNB) | Case#1 - UE sends the new MAC-I and target node support: include new MAC-I and the new added input for the MAC-I calculation in the RETRIEVE UE CONTEXT REQUEST(Clause3.7) X2 message. Case#2 - Otherwise: use legacy RETRIEVE UE CONTEXT REQUEST(Clause 3.7) X2 message. |
| MAC-I verification at Source gNB/ng-eNB | Normal: Case#1 - new MAC-I is used and UE and source node supports: verify in new method Case#2 - No new MAC-I and not all Source/UE/target node support: verify in old method. Abnormal: Case#3 - New MAC-I is used and not all source and UE support: Reject the verification and send notification to target gNB Case#4 - No new MAC-I in X2 however all Source/UE/target node support the new feature: - Case#4.1- verify in old method and if succeed, send RETRIEVE UE CONTEXT RESPONSE - Case#4.2- send the RETRIEVE UE CONTEXT FAILURE (Clause 3.7) with a cause. |

#### 3.6 Impact on RAN2

RAN2 sent an LS (R2-2109121/S3-213168) in SA3#104e in Sep 2021, confirming the solution is technical feasible.

“

Q3: The possibility of specifying the solution in RAN2 specification in Rel-17 timeframe, if the solution is concluded by SA3.

[RAN2 response]

The solution is technically feasible from RAN2 perspective.

”

On the capability negotiation between UE and gNB/ng-eNB, RAN2 made initial conclusion as below:

“

Q1: For the capability negotiation method between UE and gNB/ng-eNB as mentioned above, if there are other preferable alternatives from RAN2 perspective?

Q2: Is there any mechanism for the source gNB/ng-eNB to know the target gNB/ng-eNB capabilities?

[RAN2 response]

RAN2 discussed the capability negotiation methods and made the initial conclusion as follows:

- The feature requires the support of the UE, the source gNB/ng-eNB and the target gNB/ng-eNB;

- From UE perspective

  - The UE indicates its capability in the AS capability and reports to network via RRC signalling;

  - The UE enables the feature only when it knows both source node and target node support it;

- From the source node perspective

  - The source node indicates its capability via the RRC dedicated configuration (i.e. RRCRelease with SuspendConfig) or the SIB (depending on the method);

  - The source node only performs the new ResumeMAC-I verification when the UE is configured with the new feature and the target node supports the new ResumeMAC-I;

- From the target node perspective

  - The node as the target node role indicates its capability via SIB or binds its capability together with the source node’s capability (depending on method);

  - How to indicate the target node’s capability to the source node should be discussed in RAN3.

”

Observation 1: Based on RAN2 reply LS R2-2109121, this solution is technically feasible from RAN2 perspective.

#### 3.7 Potential impacted messages in TS 38.423

Below is the excerpt of the existing messages in TS 38.423, which might be impacted by this feature. However, these are only examples to show the potential impact, and there could be other ways to achieve this feature. The final decision on what changes in X2 messages is in the remit of RAN3.

#### 9.1.1.8	RETRIEVE UE CONTEXT REQUEST

This message is sent by the new NG-RAN node to request the old NG-RAN node to transfer the UE Context to the new NG-RAN.

Direction: new NG-RAN node  old NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | reject |
| UE Context ID | M |  | 9.2.3.40 |  | YES | reject |
| Integrity protection | M |  | BIT STRING (SIZE (16)) | RRC Resume: ResumeMAC-I either contained in the RRC ResumeRequest or the RRCResumeRequest1 message as defined in TS 38.331 [10]) or the ShortResumeMAC-I in the RRCConnection ResumeRequest message as defined in TS 36.331 [14]) RRC Reestablishment: ShortMAC-I contained in the RRCReestablishmentRequest as defined in TS 38.331 [10]) or the ShortMAC-I in the RRCConnection ReestablishmentRequest message as defined in TS 36.331 [14]). | YES | reject |
| New Cell Identifier | M |  | NG-RAN Cell Identity 9.2.2.9 | RRC Resume: Corresponds to the targetCellIdentity within the VarResumeMAC-Input as specified in TS 38.331 [10] or the cellIdentity within the VarShortINACTIVE-MAC-Input as specified in TS 36.331 [14]. RRC Reestablishment: Corresponds to the targetCellIdentity within the VarShortMAC-Input as specified in TS 38.331 [10] or the cellIdentity within the VarShortMAC-Input as specified in TS 36.331 [14]. | YES | reject |
| RRC Resume Cause | O |  | 9.2.3.61 | In case of RNA Update, contains the cause value provided by the UE in the RRCResumeRequest or the RRCResumeRequest1 message, as defined in TS 38.331 [10], or in the RRCConnection ResumeRequest message, as defined in TS 36.331 [14]. | YES | ignore |

#### 9.1.1.10	RETRIEVE UE CONTEXT FAILURE

This message is sent by the old NG-RAN node to inform the new NG-RAN node that the Retrieve UE Context procedure has failed.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node To New NG-RAN node Resume Container | O |  | OCTET STRING | Includes either the RRCRelease message as defined in TS 38.331 [10], or the RRCConnectionRelease message as defined in TS 36.331 [14], encapsulated in a PDCP-C PDU. | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.2.3.40	UE Context ID

This IE is used to address a UE Context within an NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE Context ID | M |  |  |  |
| >RRC Resume |  |  |  |  |
| >>I-RNTI | M |  | 9.2.3.46 | NOTE: How the new NG-RAN node is able to resolve the old NG-RAN ID from the I-RNTI is a matter of proper configuration in the old and new NG-RAN node. |
| >>Allocated C-RNTI | M |  | BIT STRING (SIZE (16)) | Temporary C-RNTI allocated to the UE by the cell where the RRC connection has been requested to be resumed, contained in the MAC RAR as defined in TS 38.321 [35] or in TS 36.321 [36]. |
| >>Access PCI | M |  | NG-RAN Cell PCI 9.2.2.10 | The cell PCI where the RRC connection has been requested to be resumed. |
| >RRC Reestablishment |  |  |  |  |
| >>C-RNTI | M |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRCReestablishmentRequest message (TS 38.331 [10]) or RRCConnectionReestablishmentRequest message (TS 36.331 [14]). |
| >> Failure Cell PCI | M |  | NG-RAN Cell PCI 9.2.2.10 |  |

# References

[1]	TR 33.809 - Study on 5G Security Enhancement against False Base Stations (FBS)

[2] S3-221185 - New WID on Security Enhancement on RRCResumeRequest Message Protection

[3] S3-221072 - Conclusion for key issue#1 on solution#17

[4] R2-2109121/S3-213168 - LS Reply to SA3 on security protection on RRCResumeRequest message

[5] R2-2107574 - Discussion paper on the security protection on RRCResumeRequest

[6] S3-234314 - SERP CR to TS 33.501 on the Protection of the RRC ResumeRequest message

[7] S3-233819 - Discussion paper on SERP summary