3GPP TSG-SA1 Meeting #91e	S1-203407rev

Electronic Meeting, 24 August - 2 September 2020	(revision of S1-203301-203105r2)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.889 | CR | 0169 | rev | 3 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete analysis for Rel-17 On-Network FRMCS requirements. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, Ericsson, Nokia, Nokia Shanghai Bell, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS3 |  |  |  |  |  | Date: |  |  | 2020-09-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 3GPP SA plenary agreed in June 2020 to change the TR “Study on Future Railway Mobile Communication System (FRMCS)” from an 3GPP internal TR (TR 22.889) to an 3GPP external TR 22.989 (see SP-200578), but this change has not yet been implemented. The reason to make the FRMCS study an external TR is so the Railway community can use the TR to understand how well the 3GPP On-Network communications in Rel-17 fulfills the original UIC requirements. This CR completes the analysis for Rel-17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete analysis for On-Network FRMCS requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Important information would be missing in the final analysis based on the outcomes of the normative phases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.16.2.5, 12.17.2.5, 12.23.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************************Change**************************************************

#### 12.16.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-12.16.2-001] | The FRMCS System shall provide means of regulated interworking (s) for telephony services to/from external public networks e.g. system administrator has the ability to restrict access telephony services to/from external networks. | T | Covered by TS 22.179 TS 22.280 | For GSM-R, covered in 22.179 [R-6.18.4.2-001-5]. For other MCX systems, covered in 22.280 [R-6.17.2-001-7] For LMR/PMR (e.g. TETRA, P25 and TIA-603-D refer to section 12.6. For PLMN and PSTN, covered by 22.280 [R.6.17.3.2-002] |
| [R-12.16.2-002] | The FRMCS System shall provide Packet switched (PS) interworking (s) to/from external networks. | T | Partly covered by TS 22.280 | For GSM-R and according to latest decision from UIC (see FRMCS project activities status report in ERIG#70 (July 2018), the requirement is not required. No use case has been identified by UIC ((E )GPRS EVC could not be connected to any FRMCS RBC). For other MCX systems, covered in 22.280 [R-6.17.2-001-7]. For LMR/PMR (e.g. TETRA, P25 and TIA-603-D refer to section 12.6. For PLMN and PSTN, interworking is not covered. |
| [R-12.16.2-003] | The FRMCS System shall provide the necessary signalling and voice codec adaptation in the context of interworking with external networks. | T | Partly covered by TS 22.179 | For GSM-R covered in 22.179 [R-6.18.4.2-005] For other MCX systems, not required. For LMR/PMR (e.g. TETRA, P25 and TIA-603-D refer to section 12.6. For PLMN and PSTN, interworking is not covered |
| [R-12.16.2-004] | The FRMCS System shall provide interworking for supplementary services related to the external network. | A | Not covered. | For other MCX systems, not applicable (other mechanisms are provided). For LMR/PMR (e.g. TETRA, P25 and TIA-603-D refer to section 12.6. For other external systems, not covered. |
| [R-12.16.2-005] | The FRMCS System shall provide interworking communication with external networks in a secured way. | A/T | Partly covered by TS 22.280 | For other MCX systems covered in 22.280 [R-6.17.2-001], [R-6.17.2-002], [R-6.17.2-007]. For LMR/PMR (e.g. TETRA, P25 and TIA-603-D refer to section 12.6. For PLMN and PSTN, interworking is not covered. For other external systems, not covered. |
| [R-12.16.2-006] | The FRMCS System shall provide interworking communication with external networks with the required Quality of Service. | T | Partly covered by TS 22.280 | For other MCX systems, not covered. For GSM-R, covered by [R-6.17.3.1-002]. For LMR/PMR (e.g. TETRA, P25 and TIA-603-D refer to section 12.6. For PLMN and PSTN, interworking is not covered. |
| [R-12.16.2-007] | The FRMCS System shall be able to select interconnection based on multiple interconnections of the same type (i.e. redundancy of physical/logical paths and transmission links for both control and user planes including load aggregation and/or warm stand-by techniques). | A/T | N/A (physical paths) TS 22.280 | Implementation requirement for redundant physical paths. Partly covered by clause 6.17.3.1 for GSM-R. Load aggregation and/or warm stand-by techniques to be supported by the Interworking Function (IWF). |
| [R-12.16.2-008] | The FRMCS System shall select the bearer characteristics based on exchanged signalling information (i.e. dynamic and automatic configuration of interconnections parameters). | T | TS 22.280 | Partly covered by clause 6.17.3.1 for GSM-R. Dynamic and automatic configuration of interconnections parameters to be supported by the Interworking Function (IWF). |
| [R-12.16.2-009] | The FRMCS System shall select the appropriate interconnection type, e.g. CS or PS based on the destination address of the target user. | T | N/A | For GSM-R, PS interworking is not required (see above). |

*************************************End of Change***********************************************

****************************************Change**************************************************

#### 12.17.2.5	Potential requirements and gap analysis

| Reference Number |  | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-12.17.2-001] | FRMCS security requirements according to chapter 12.15 shall apply to On-network and Off-network communication. | A/T | TS 22.278 TS 22.261 TS 22.228 TS 22.280 | Covered for on-network (refer to section 12.15). |
| [R-12.17.2-002] | FRMCS QoS requirements (reference QoS section 12.10) shall apply to On-network and Off-network communication. | A/T | TS 22.261 TS 22.280 TS 22.289 | Covered for on-network (refer to section 12.10). |
| [R-12.17.2-003] | FRMCS positioning accuracy requirements according to chapter 12.14 shall apply to On-network and Off-network communication. | A/T | TS 22.261 | Partly covered for on-network (refer to section 12.14). |
| [R-12.17.2-004] | The FRMCS System requirements applicable for communication recording as specified in the recording and access application use cases shall apply to On-network and Off-network communication. | A/T | TS 22.280 | See gap analysis of sub-clause 6.10. |
| [R-12.17.2-005] | FRMCS On-network communication shall support the flexible use of different radio bearers (see Bearer Flexibility). | A/T | TS 22.278 TS 22.261 TS 22.280 | TS 22.280 See sub-clauses in chapter 5 and 6 |
| [R-12.17.2-006] | FRMCS Equipment shall support On-network only as well as On-and Off-network relay communication. | T | TS 22.278 TS 22.261 TS 22.280 | 22.278 – sub-clause 7A: Mode 1 covered by ProSe UE-to-Network relay (single-hop relay with UE-to-Network relay UE between remote UE and gNb) Mode 2 not covered by ProSe UE-to-UE Relay (multi-hop relays not covered) Mode 3 not covered (multi-hop relays not covered). 22.261: UE-to-Network Relay not covered in 5GS. 22.280 [R-7.11-001]: use of off-net and on-net MCX Services simultaneously |
| [R-12.17.2-007] | FRMCS shall be able to support Off-network communication that tide over a distance between FRMCS UE’s of up to 3km. | T | TS 22.289 | Covered by table 5.2.2-2 (relevant for Off-Network MCData Service only) |
| [R-12.17.2-008] | FRMCS Equipment shall support Off-network only as well as Off-network relay communication. | T | TS 22.278 TS 22.261 TS 22.280 | 22.278 – sub-clause 7A: Mode 1 covered by ProSe UE-to-Network relay (single-hop relay with UE-to-Network relay UE between remote UE and gNb) Mode 2 not covered by ProSe UE-to-UE Relay (multi-hop relays not covered) Mode 3 not covered (multi-hop relays not covered). 22.261: UE-to-Network Relay not covered in 5GS. 22.280 [R-7.11-001]: use of off-net and on-net MCX Services simultaneously |

*************************************End of Change***********************************************

****************************************Change**************************************************

### 12.23.6	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-12.23-001] | The FRMCS System shall be able to support for individual contiguous spectrum blocks, bandwidths less than 5 MHz (e.g. 1.4 MHz or 3 MHz). | T | N/A | Covered by RAN specifications: - Covered by E-UTRA, - Not covered by NR. |

*************************************End of Change***********************************************