TSG-SA Working Group 1 (Services) meeting #4	TSG S1#4 (99)510

Quebec City, Quebec, Canada 6th-9th, July 1999	 	 Agenda Item:6.1.7

3GPP TSG CN3	TSG N3-99150

14th-18th June, 1999

Abiko, Japan

## Liaison Statement

To:		TSG S1, N1, T2

Source:		TSG N3

Topic:		The feasibility of the provision of PIAFS in UMTS

# Abstract

This document describes briefly feasibility study on PIAFS in UMTS which is requested in LS from S1: ”Liaison Statement regarding CS bearer services”(N3-99083). Discussion for clarification to PIAFS has taken place based on ”Presentation of PIAFS”(N3-99129). N3 concludes that PIAFS in UMTS would be possible and consequently makes WI description: ” Work Item description for the support of PIAFS in UMTS”(N3-99149) and will discuss details of IWF for UMTS R’99.

In relation to the above result, N3 requests N1 to study interworking on C-Plane such as UMTS TS 24.008 and also requests T2 to study impact on recognition of PIAFS service.

# Abbreviations

PIAFS	PHS Internet Access Forum Standard

PHS	Personal Handy phone System

PDC	Personal Digital Cellular (Japanese 2nd generation cellular system)

NT	Non Transparent

FTM	Frame Tunneling Mode

L1	Layer 1

# Contents of discussion

## Background in Japan

- PIAFS is already available and used by a lot of user.

- Number of PIAFS user reaches about 500000. Demand for high speed CS data communication is now increasing. Because PDC only provides low speed CS data communication (Maximum 9.6[kbps]), growing ratio of PIAFS user is higher than that of PDC CS data user.

- PDC is mostly used for speech and ratio of CS data user is few. PDC only provides low speed analog modem for CS data connection. Possibility of future extension of PDC’s CS data connection such as interworking with N-ISDN is little considering other solution (PIAFS or PDC-Packet( (Maximum 28.8[kbps]) or UMTS).

## Service aspect

- PIAFS does not compete with other NT service such as FTM and V.110. Major reason for supporting PIAFS is connection to existing PHS-MS or TA for PIAFS.

## Technical aspect

- Non Transparent mode should only be applied to PIAFS in UMTS. There is a possibility of application of Transparent mode, but NT mode has following advantages that are also applied to other NT type of IWF such as FTM and V.110.

- DTX or low power transmission is easily available in NT mode and makes battery saving and high radio capacity.

- PIAFS has 58.4[kbps] mode as well as 29.2[kbps] mode.

- Layer 2 of PIAFS is based not on HDLC but on ”WORM-ARQ” which is used for PDC CS data communication.

- User rate is symmetric, but specification of PIAFS has future possibility of application of asymmetric rate.

- In slide 3 of ”Presentation of PIAFS”(N3-99129), L1d means UDI, L1b means specific L1 between adapter and MS, and L1e means RS-232C.

- PIAFS is specified considering coordination with N-ISDN. There is no difference between interworking with PHS-MS and that with TA for PIAFS.

- Information of English document on PIAFS can be required in 3GPP mailing list.

# References

Following files are attached for references.

- Presentation of PIAFS(N3-99129)

- Work Item description for the support of PIAFS in UMTS(N3-99149)