TSG-SA Working Group 1 (Services) meeting #4	TSGS1#4(99)533

Quebec City, Canada, 5-9 July 1999		Agenda 6.6.8

| CHANGE REQUEST No : |  |  |  |  | HOAD01 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.129 |  | Version: |  |  | 3.0.0 |  |  |  |
| Submitted to TSG_SA |  |  | for approval |  |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GSM UMTS Handover |
| --- | --- |

| Source: | S1 ad hoc on handover 8.7.99 | Date: | 8.7.99 |
| --- | --- | --- | --- |

| Subject: | Editorial improvements of definitions and alignment of terminology. |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification | X |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | To carry out CR agreed at “ad hoc on handover during S1#5” ie: Consistent use of the term ‘UTRA radio access mode’ to distinguish FDD/TDD, ‘radio system’ for UMTS-other Improve the definition of handover, clarifying that the point of handover is that defined bearer QoS is maintained during a process in which the radio transmitter changes. Clarify ‘Same network’ means same MCC+MNC |
| --- | --- |

| Clauses affected: | 7.4 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 1st Editorial Change

# 1	Scope

The scope of this document includes service requirements for handover (terms are defined below) within UMTS systems and between UMTS, other IMT-2000 family members and 2nd generation systems. Particular emphasis has been placed on the description of requirements for handover between UMTS and GSM but requirements specific to other systems are incorporated as required.

UMTS is a "third generation" mobile cellular radio telecommunications system which provides telecommunication and information services to wireless terminals. Mobile cellular systems have the defining characteristic that they are capable of maintaining continuity of service to a wireless terminal as it moves between the radio coverage area, or "cells", associated with different base station sites . This functionality is called "handover". Handover can also occur due to change of radio resource providing a service without necessarily any change of the base stations involved. In particular, when the radio resources providing a service change from one of the UTRA radio access modes to the other (UTRA-FDD and UTRA-TDD), this is regarded as handover.

It is a key requirement of UMTS that it allows for dual or multi-mode (eg UMTS/GSM) terminals to handover traffic from UMTS to other radio systems such as GSM and visa versa. This document describes the service requirements for intra- and inter- system handover that shall be used by other SMG STCs to guide the implementation of UMTS-to-(eg) GSM handover specifications. It defines requirements for the enhancement of the GSM specifications to allow GSM-to-UMTS handover.

The following subject areas are within the scope of these service requirements:

-	user perceived performance that may be influenced by handover;

-	operational requirements relating to handover;

-	security requirements.

The requirements set forth in this document are service requirements, in that they fulfil the following:

-	the requirements are independent of the implementation of the URAN;

-	the extent to which the requirements are met are in principle verifiable using observables that are not internal to the URAN.

## 1.1	Situations in which Service Requirements apply

The service requirements in this document are as far as possible independent of the implementation of the UTRA. They therefore apply to situations where handover would occur regardless of how the UTRA is implemented. Situations envisaged are:

-	handover within UMTS due to change of radio resource caused by UE movement between areas covered by different transmitters;

-	handover within UMTS due to change of UTRA radio access mode;

-	handover due to change of radio system (eg UMTS to GSM).

It is possible that handover (ie change of radio resource) will occur in other situations, for example the technical implementation of the URAN may necessitate it or O&M procedures initiated by the operator may force it. Requirements for these situations are not within the scope of this document, with the exception of two remarks:

-	where the technical implementation of the URAN necessitates handover as a matter of normal operation (i.e. not related to the above situations), then services shall in no way be degraded or adversely affected;

-	the service requirements for handover occurring in situations such as O&M activity are outside the scope of this document.

### 2nd Editorial Change

## 3.1	Definitions

For the purposes of the present document, the following definitions apply:

Connection mode (for a bearer service): characterizes the type of association between two endpoints as required by the bearer service for the transfer of information. A bearer service is either connection-oriented or connectionless. In a connection oriented mode, a logical association called connection needs to be established between the source and the destination entities before information can be exchanged between them. Connection oriented bearer services lifetime is the period of time between the establishment and the release of the connection.

Connectionless (for a bearer service): In a connectionless bearer, no connection is established beforehand between the source and the destination entities ; the source and destination network addresses need to be specified in each message. Transferred information cannot be guaranteed of ordered delivery. Connectionless bearer services lifetime is reduced to the transport of one message.

GSM coverage: an area where  mobile cellular services are provided in accordance with GSM standards

UMTS coverage: an area where mobile cellular services are provided in accordance with UMTS standards.

Multi mode terminal: UE that can obtain service from at least one UTRA radio access mode, and one or more different systems such as GSM bands  or possibly other radio systems such IMT-2000 family members.

Handover: The process in which the radio access network changes the radio transmitters or radio access mode or radio system used to provide the bearer services, while maintaining a defined bearer service QoS.

Intra PLMNhandover: Handover within the same network, ie having the same MCC-MNC regardless of radio access system. Note: this includes the case of UMTS <>GSM handover where MCC-MNC are the same in both cases.

Inter PLMN handover: Handover between different PLMNs, ie having different MCC-MNC.

Inter system handover: Handover between networks using different radiosystems , e.g. UMTS – GSM.

UTRA Radio access mode: the selected UTRA radio access mode ie UTRA-FDD;UTRA-TDD.

Radio system: the selected 2nd or 3rd generation radio access technology, eg UMTS or GSM.

### 3rd Editorial Change

## 4.2	General Operational Considerations

### 4.2.1	Coverage environment

Mechanisms defined to support handover between UMTS and other radio systems (such as other IMT 2000 family members, or GSM) or UTRA radio access modes should effectively cope with a number of coverage scenarios:

-	limited UMTS coverage in a 'sea' of  coverage provided by another radio system or UTRA radio access mode,  or vice-versa;

-	selective operation at a geographical boundary, with extensive UMTS coverage on one side and extensive coverage from another radio system on the other side;

-	geographically co-located areas of UMTS coverage and another radio system.

However the standards should impose no restrictions or assumptions on how an operator might deploy or operate the network in both GSM and UMTS.

### 4.2.2	Inter PLMN Handover Issues

Handovers between PLMNs should remain an optional feature to implement. It is envisaged that handover would take place due to changing radio conditions caused eg by movement of the terminal causing it toleave the coverage area of aPLMN.

The following networks may be involved with an inter-PLMN handover procedure. These concepts are illustrated in Annex A:

-	the user’s home network, i.e. the operator where the user’s subscription may be found;

-	the user’s visited network  where the subscriber user is currently registered, i.e. the network  where the subscriber user has performed the last successful update location procedure. As long as the subscriber user  is roaming roams within the home network, home and visited network are identical.

-	the user's serving network covering the cell that serves the subscriber. After successful completion of the update location update procedure, the serving network is identical with the visited network. After an inter-PLMN handover, the visited network is different from the serving network until a location update procedure has been successfully completed (excepted the case that the subscriber returns into the visited network).

-	the target network covering candidate target cell(s) for inter-PLMNhandover. The target network has overlapping radio coverage with the serving network but not necessarily with the visited network.

The minimum requirements for inter PLMNHO are:

-	continuity of an active call across the handover procedure, where this would be possible for intra-PLMN handover;

-	charging, billing and accounting for inter-PLMN handover should be according to the principles defined in UMTS 22.15. For R'99 the mechanisms currently used in GSM should be provided as a minimum (charging for handover leg is based on vistited network tariff, etc., settlement between operators is based on bulk metering, etc.);

-	the ability to check with the home network whether the user is permitted to handover  from the visited network to a target network;

-	the decision whether the handover request is accepted must be taken by the target network;

-	invocation of the handover procedure only occurs if the target network provides the radio channel type required for the respective call;

-	the avoidance of "network hopping", i.e. successive handover procedures between neighbouring networks for the same call;

-	the possibility of user notification of inter PLMN HO (eg possible tariff change) when it occurs.

### 4.2.3	Charging and Network Management

Means shall be standardised which allow charging records to record the time of handover in the case of inter PLMNoperator handover. Charging records must be able to reflect the level of service , UTRA radio access mode and network type after handover.

A capability to provide network management information relating to frequency of occurrence and type of handover should be defined.

### 4th Editorial Change

## 5.1	Handover due to UE Movement

It should be possible to provide a technical implementation of handover such that there is no measurable impact on the quality of any service when handover due to UE movement occurs. This does not imply that all UMTS handovers will achieve this ideal. However, the standards shall define at least one UTRA radio access mode in which this is possible given the following:

-	UE speed stays within limits for given service;

-	UE stays constantly within UMTS coverage of a single UTRAN.

## 5.2	Handover Between UTRA Radio Access Modes

The standards shall permit a technical implementation of handover between radio access modes, although there may be a temporary degradation of QoS on bearer services at the time of handover.

### 5th Editorial Change

Annex A (informative):
Illustration of elements in inter-PLMN handover

Figure 1 illustrates the above definitions taking an example of European GSM networks. The subscriber's home network is France. The visited network where the subscriber is registered in a VLR is Germany. The signalling connection between HLR and VLR is indicated by dotted lines. The calls for the subscriber are controlled by the MSC collocated to the VLR where the subscriber is registered. This MSC is called "anchor MSC".

Handover to a different MSC may occur if the cell serving the subscriber after handover is not controlled by the anchor MSC. This MSC is called the "serving MSC".Even after the call has been handed over to a different MSC, the call control function remains in the anchor MSC. The signalling connection and circuit switched connection established between anchor MSC and serving MSC are indicated by a solid line.

When the French subscriber registered in a German network roams near the border to the Netherlands, inter-PLMN  handover may occur. In this case a Dutch network is the target network. After handover, the anchor MSC located in a German network continues to control the call. The German network remains the visited network where the subscriber is registered. The subscriber's location information stored in the HLR remains unchanged. The signalling and circuit switched connections between the anchor MSC and the previously serving MSC in theGerman network will be released when the Mobile Station (MS) is served by a cell within a Dutch network. The Dutch network becomes the serving network. From the Dutch network the subscriber may be handed over to a Belgian network.