TSG-SA Working Group 1 (Services) meeting #5	TSG S1 (99) 1046

San Diego, USA 29th Nov – 3rd Dec 1999	Agenda Item:

To:	3GPP CN WG1, CN WG3, SA WG2

Cc:

Source: 	S1

Title: 	LS on addition of general bearer services

S1 would like to inform to N1, N3 and S2 regarding a requirement of additional general bearer services.

This requirement has been already taken into account in S1 and also the relevant WGs.  This is for the adjustment of the general bearer services in the specification TS22.002.  See attached.

| 3GPP SA1 #6/ SMG1 #65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | (99) 1008 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| San Diego, USA, 29 Nov – 3 Dec |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.002 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.1.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  |  | X | Core Network |  |  |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Ericsson, NTT DoCoMo |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 1999-11-30 |  |  |  |  |  |
| Subject: |  |  | Addition of new general bearer service user data characteristics for 33.6kbit/s modem, FTM and multimedia calls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Addition of 33.6kbit/s to Fixed Network User Rate. General bearer service user data characteristics shall be enhanced to provide the Frame Tunnelling Mode (FTM) and Multimedia Calls to reflect N1 and N3 result of study. - 3G-H.324/M is used as the multimedia system for the CS domain. - “H.223 & H.245” which is a new indication is added to indicate 3G-H.324/M calls. - 3G-H.324/M calls are supported over 64kbit/s(UDI), 56kbit/s(RDI), 33.6kbit/s(3.1kHz audio), 28.8kbit/s(3.1kHz audio) and 32kbit/s(UDI) - 3G-H.324/M calls are provided by the use of synchronous general bearer service(BS 30). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

## 3.1	General bearer service user data characteristics

The tables below describe the characteristics of the General Bearer Services. The indicated fixed network user rates are possible, but support of General Bearer Service does not imply support of all rates.

### 3.1.1	3,1 kHz Audio

| Fixed Network User Rate | Access Structure | Information Transfer Capability | QoS attributes | Note |
| --- | --- | --- | --- | --- |
| 0.3 kbit/s | Asynch | 3,1 kHz | NT or T | note 2 |
| 1.2 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | notes 1 and 2 |
| 2.4 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | note 2 |
| 4.8 kbit/s | Asynch, Synch | 3,1 kHz | NT or T | note 2 |
| 9.6 kbit/s | Asynch, Synch | 3,1 kHz | NT or T |  |
| 14.4 kbit/s | Asynch, Synch | 3,1 kHz | NT or T |  |
| 19.2 kbit/s | Asynch, Synch | 3,1 kHz | NT or T |  |
| 28.8 kbit/s | Asynch, Synch | 3,1 kHz | NT or T |  |
| 33.6 kbit/s | Asynch | 3.1 kHz | NT |  |
|  | Asynch | 3,1 kHz | NT | Note 3 |

NOTE 1:	Not applicable to synchronous NT service.

NOTE 2:	These services are also supported by the GSM Phase 2 Specifications.

- NOTE 3:	This is used with high speed modems such as V.90 (56kbit/s). Modem type = 'Autobauding Type 1' is selected. FNUR has no meaning in this case.

### 3.1.2	V.110 UDI

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 0.3 kbit/s | Asynch | V.110 | NT or T | note 2 |
| 1.2 kbit/s | Asynch, Synch | V.110 | NT or T | note 1 note 2 |
| 2.4 kbit/s | Asynch, Synch | V.110 | NT or T | note 2 |
| 4.8 kbit/s | Asynch, Synch | V.110 | NT or T | note 2 |
| 9.6 kbit/s | Asynch, Synch | V.110 | NT or T | note 2 |
| 14.4 kbit/s | Asynch, Synch | V.110 | NT or T |  |
| 19.2 kbit/s | Asynch, Synch | V.110 | NT or T |  |
| 28.8 kbit/s | Asynch, Synch | V.110 | NT or T |  |
| 38.4 kbit/s | Asynch, Synch | V.110 | NT or T |  |
| 48 kbit/s | Synch | V.110 | T |  |
| 56 kbit/s | Synch | V.110 | T (in a 64 kbit/s environment) |  |

NOTE 1:	Not applicable to synchronous NT service.

NOTE 2:	These services are also supported by the GSM Phase 2 Specifications.

### 3.1.3	X.31 Flag Stuffing UDI

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 2.4 kbit/s | Synch | X.31 Flag Stuffing | NT | note 1 |
| 4.8 kbit/s | Synch | X.31 Flag Stuffing | NT | note 1 |
| 9.6 kbit/s | Synch | X.31 Flag Stuffing | NT | note 1 |
| 14.4 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 19.2 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 28.8 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 38.4 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 48 kbit/s | Synch | X.31 Flag Stuffing | NT |  |
| 56 kbit/s | Synch | X.31 Flag Stuffing | NT |  |

NOTE 1:	These services are also supported by the GSM Phase 2 Specifications.

### 3.1.4	V.120

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 1.2 kbit/s | Asynch | V.120 | NT |  |
| 2.4 kbit/s | Asynch, Synch | V.120 | NT |  |
| 4.8 kbit/s | Asynch, Synch | V.120 | NT |  |
| 9.6 kbit/s | Asynch, Synch | V.120 | NT |  |
| 14.4 kbit/s | Asynch, Synch | V.120 | NT |  |
| 19.2 kbit/s | Asynch, Synch | V.120 | NT |  |
| 28.8 kbit/s | Asynch, Synch | V.120 | NT | note 1 |
| 38.4 kbit/s | Asynch, Synch | V.120 | NT |  |
| 48 kbit/s | Asynch, Synch | V.120 | NT |  |
| 56 kbit/s | Asynch, Synch | V.120 | NT | note 2 |

NOTE 1:	Requires a new code point in V.120 specification to be defined.

NOTE 2:	Not applicable in a 56 kbit/s environment.

### 3.1.5	Bit Transparent Mode

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 56 kbit/s | Synch | Bit transparent | T (RDI) (in a 56 kbit/s environment) |  |
| 64 kbit/s | Synch | Bit transparent | T (UDI) (in a 64 kbit/s environment) |  |

### 3.1.6		PIAFS

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 32 kbit/s | Asynch | PIAFS | NT |  |
| 64 kbit/s | Asynch | PIAFS | NT |  |

### 3.1.X		Frame Tunnelling Mode

| Fixed Network User Rate | Access Structure | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- |
| 56kbit/s | Asynch | X.31 flag stuffing | NT |  |
| 64 kbit/s | Asynch | X.31 flag stuffing | NT | Note1 |

Note1: Not applicable in a 56kbit/s environment.

### 3.1.X		Multimedia Call

| Fixed Network User Rate | Access Structure | Information Transfer Capability | User Information Layer 1 protocol | QoS Attribute | Notes |
| --- | --- | --- | --- | --- | --- |
| 28.8 kbit/s | Synch | 3.1kHz Audio | H.223 & H.245 | T |  |
| 32.0 kbit/s | Synch | UDI | H.223 & H.245 | T |  |
| 33.6 kbit/s | Synch | 3.1kHz Audio | H.223 & H.245 | T |  |
| 56 kbit/s | Synch | RDI | H.223 & H.245 | T |  |
| 64 kbit/s | Synch | UDI | H.223 & H.245 | T |  |