3GPP TSG-SA1 Meeting #102	S1-231267

Berlin, Germany, 22 – 26 May 2023	(revision of S1-23XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.153 | CR | 0062 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dual Connectivity and Multiple Access Technologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, Verizon, T-Mobile US |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Dummy |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a lack of explicit MPS stage 1 requirments for: - When dual connectivity is being used in a single or pair of 3GPP RATs for a UE and MPS is authorized, and - When MPS is authorized and the UE is using 3GPP RAT(s) (e.g., NR, E-UTRA) and non-3GPP WLAN access technology and attached/registered to the same EPC or 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new clause and requirements on dual connectivity and a new clause and requirements on multiple access technologies |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of explicit normative requirements for MPS treatment when (a) dual connectivity is used in a single or pair of 3GPP RATs for a UE, and (b) the UE is using 3GPP RAT(s) and non-3GPP WLAN access technology and attached/registered to the same EPC or 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 5.18 and 5.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START of CHANGE***

## 5.18	Dual connectivity

Dual connectivity can be used in a single or pair of 3GPP RATs (e.g., E-UTRA or NR) for improved UE throughput.

When dual connectivity is being used for a UE and MPS is authorized, MPS priority shall be provided for the MPS calls/sessions using the dual connectivity based on operator policy.

## 5.19	Multiple access technologies

When one or more 3GPP access technologies as well as a non-3GPP WLAN access technology are applicable, the 3GPP system will select the most appropriate 3GPP or non-3GPP access technology for a service, potentially allowing multiple access technologies to be used for one or more services active on a UE. This subclause covers MPS handling when one or more 3GPP RATs and a non-3GPP WLAN access technology are being used for one or more services active on a UE.

The system shall support at least the same 3GPP access selection policy controls for MPS handling as those defined for normal (non-MPS) services.

The system shall support MPS when the UE is using E-UTRA and non-3GPP WLAN and attached to the same EPC.

The system shall support MPS when the UE is using 3GPP RAT(s) and non-3GPP WLAN and registered to the same 5GC.

According to the general requirements on multiple access technologies specified in TS 22.261 clause 6.3.2.1, the 5G system can:

-	steer a UE to select certain 3GPP access network(s) based on operator policy,

-	dynamically offload part of the traffic (e.g., from 3GPP RAT to non-3GPP access technology), taking into account traffic load and traffic type based on operator policy, and

-	optimally distribute user traffic over access technologies in use, taking into account e.g., service, traffic characteristics, radio characteristics, and UE's moving speed, when a UE is using two or more access technologies simultaneously.

The following are corresponding MPS specific requirements for the above features:

-	when steering a UE with MPS subscription to select certain 3GPP access network(s), based on on operator policy, the 5G system shall support the option to steer the MPS subscribed UE to 3GPP access network(s) supporting MPS,

-	when dynamically offloading part of the traffic (e.g., from 3GPP RAT to non-3GPP access technology), based on operator policy, the 5G system shall take into account if the network towards which the traffic is offloaded supports MPS, and

-	when a UE is using two or more access technologies simultaneously, and optimally distributes user traffic over access technologies in use, based on operater policy, the 5G system shall take into account whether the access technologies support MPS.

*** END of CHANGE***