3GPP TSG-SA WG1 Meeting # 104 	<S1-233125>

# Chicago, US, Nov 13, 2023 - Nov 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.843 | CR | <0007> | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CPR alignment with agreed CR for 22125 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_UAV_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current some CPRs in TR22.843 is different with the agreed CRs for TS22.125 in SA1 #103 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To align CPRs with agreed CRs for TS 22.125 in SA1 #103 meeting, and wording change for CPR 6.1-009. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some CPRs in TR22.843 will not align with the content in TS 22.125, which may lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1	Network support and exposure for UAV usage

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1-001 | Based on operator’s policy, the 5G system shall be able to support a method to predict, monitor network conditions and QoS (e.g. bitrate, latency, reliability) and report to 3rd party along a continuous geographic planned flight path of a UAV at specific times of its expected flight duration. | P.R 5.2.6-001 P.R 5.6.6-001 |  |
| CPR 6.1-002 | Based on operator’s policy, the 5G system shall be able to provide UTM with the information about geographic areas where UAV service requirements could or could not be met based on predicted network conditions and QoS (e.g. bitrate, latency, reliability). | P.R 5.2.6-002 |  |
| CPR 6.1-003 | Based on a 3rd party request, the 5G system shall be able to assist the UTM with mechanism to provide to the 3rd party alternative UAV flight paths, e.g., based on required waypoints, QoS, and exclusion zones. | P.R 5.2.6-003 |  |
| CPR 6.1-004 | Based on a 3rd party request and operator’s policy, the 5G system shall be able to reconfigure network resources to provide the required QoS along a UAV planned flight path, e.g. at particular geographical area(s) and time(s). | P.R 5.2.6-004 |  |
| CPR 6.1-005 | The 5G system shall be able to support a mechanism to enable a network operator to track an airborne connected UE which doesn’t have an aerial subscription . | P.R 5.7.6-001 |  |
| CPR 6.1-006 | The 5G system shall be able to support service enablement layer exposure mechanisms for the UTM or other authorized 3rd party to provide the UAV application with configuration information to route and switch traffic between one active and one standby PLMN connection, e.g. for C2 communication reliability and redundancy purpose, or to route different traffic across different PLMN connections simultaneously, e.g. C2 traffic via one PLMN and other data via the second PLMN. NOTE 1: The above requirement can be extended to scenarios where one network is a PLMN and one is an NPN. NOTE 2: There is no impact on legacy network selection. NOTE 3: It is assumed that UAV traffic handling, over each PLMN, is subject to NW control mechanisms (e.g. in accordance with MNO routing priorities, available QoS/NW resources, etc.). | P.R 5.8.6-001 P.R 5.9.6-001 |  |
| CPR 6.1-007 | The 5G system shall be able to support mechanisms for the UTM to configure different aerial flight zones where UAV application settings and communication QoS may be different, and provide network and UAV with means to identify those flight zones. | P.R 5.10.6-001 |  |
| CPR 6.1-008 | The 5G system shall be able to support mechanisms for the UTM to provide network and UAV with policy information including UAV application settings and communication QoS to be applied in specific aerial flight zones, e.g. based on flight zone type indicated or available to the UAV in a certain geographical location. | P.R 5.10.6-002 |  |
| CPR 6.1-009 | Based on operator policy and UTM’s request, the 5G system shall be able to provide UTM with UEs’ location and 3GPP identity. | P.R 5.5.6-001 |  |
| CPR 6.1-010 | Subject to user consent and national or regional regulation, based on operator policy, and the UTM’s request, the 5G system may be able to provide aerial object location(s) derived by 5G Wireless Sensing to the UTM. | P.R 5.5.6-002 |  |

6.2	Safety and Security

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2-001 | The 5G system shall be able to detect that a connected UE is airborne, when the UE doesn’t have an aerial subscription.. | PR 5.1.6-001 |  |
| CPR 6.2-002 | The 5G system shall support means to determine whether UTM supports regulatory requirements e.g., maximal VLOS distance between the UAV and the UAV controller. Note: The requirement applies to the case of both UAV and UAV controller connected to the 3GPP network. | PR 5.3.6-001 |  |
| CPR 6.2-003 | Based on operator’s policy, the 5G system shall be able to support a method to monitor and provide a 3rd party with information about deviations and violations along a UAV flight path and time. Note: Deviations can be e.g. in location and/or time with respect to the original flight plan. Violations can be e.g. with respect to exclusion zones provided together with the flight plan or known via other means. | P.R 5.6.6-002 |  |
| CPR 6.2-004 | Based on operator’s policy, the 5G system shall be able to support a method to provide UTM and UAVs with the information collected or generated by the 5G system (e.g., based on sensing results), including e.g. the location or relative distance between 3GPP UAVs and other flying objects (can be drones not using 3GPP connectivity). | P.R 5.4.6-001 |  |
| CPR 6.2-005 | The 5G system shall be able to track the UAV-Controller, regardless of the type of connection. | PR 5.3.6-002 |  |
| CPR 6.2-006 | Based on MNO policies and/or regulatory requirements, the 5G system shall be able to inform the UTM, when the 5G system detects a specific UAV condition or event, in order to enable UTM control of the UAV communication (e.g., when detecting violation of exclusion zones or maximal distance between UAVs). | PR 5.3.6-003 PR 5.3.6-004 |  |

* * * End of Changes * * * *