3GPP TSG-SA WG1 Meeting #104	S1-233043

# Chicago, USA, 13- 17 November 2023

Source:	vivo

Title:	Discussion of PS Data Off exemption for services over IMS DC

Agenda item:	3

Document for:	Discussion

- Introduction

In the SA2 meeting in Xiamen, SA2 has discussed the PS data off scenarios related to IMS data channel. TS 22.011 has specified the requirement of supporting PS data off and PS data off exempt for services over IMS Data Channel. During the discussion there was no common understanding in SA2 how to interpret the term “Services over IMS Data Channel” in TS 22.011. While some companies believe that the term “Services” can refer to single application using IMS Data Channel as transport connection, others believe that “Services” refers to the operator provided IMS Data Channel service.

SA2 seeks to clarify whether applications using IMS Data Channel should be configurable separately for PS data off exempt or all applications over IMS Data Channel should be treated in the same way.

This discussion paper provides some considerations and proposals on the PS Data Off exemption for services over IMS DC to SA2.

- Discussion

As described in clause 10.2 of TS 22.011, The UE and network will cease the sending of uplink or downlink IP packets of all services that are not 3GPP PS Data Off Exempt Services when 3GPP PS Data Off is activated in the UE. Services over IMS Data Channel are 3GPP PS Data Off Exempt Service. The corresponding descriptions are as below:

When 3GPP PS Data Off is activated in the UE, in order to preserve charging consistency:

-	the UE shall inform the network that 3GPP PS Data Off is activated,

-	the UE shall cease the sending of uplink IP Packets of all services that are not 3GPP PS Data Off Exempt Services via PDN connections in 3GPP access networks,

-	the network shall cease the sending of downlink IP Packets to the UE for all services that are not 3GPP PS Data Off Exempt Services via PDN connections in 3GPP access networks,

Each of the following operator services shall be configurable by the HPLMN operator to be part of the 3GPP PS Data Off Exempt Services:

-	MMTel Voice;

-	SMS over IMS;

-	USSD over IMS (USSI);

-	MMTel Video;

-	Services over IMS Data Channel;

As described in clause AC.1 of TS23.228, in the scope of Rel 18, the data channels are always established in the context of an IMS MMTel session, so it is right to treat all of the services over IMS Data Channel as 3GPP PS Data Off Exempt Service. The corresponed descriptions are as below:

Data channels are always established in the context of an IMS MMTel session.

Observation 1：If IMS Data Channel is always established in the context of IMS MMTel session. Considering that MMTel services are PS Data Off exempt services, it is right to treat all the services over IMS Data Channel as 3GPP PS Data Off Exempt Service.

But in Release 19, SA2 NG-RTC Ph2 aims to enhance IMS DC services and operational aspects including handling of PS data off exemption, support of IMS data channel without accompanying audio/video media ("standalone" IMS DC). As stated in in WT 4-1, the IMS Data Channel can be established without accompanying IMS session. The corresponding descriptions are as below:

-	WT-4.1: how to support standalone IMS data channel without accompanying audio/video/messaging media in an IMS session.

So, it is common consensus that IMS Data Channel can be existed standalone without accompanying any media in IMS session. Back to the use case discussion, in the description of 3GPP TS 26.114,  media components listed above are considered as core components transmitted in IMS Data Channel：

At least one of the first three of these components is present in all conversational multimedia telephony sessions.

-	Speech: The sound that is picked up by a microphone and transferred from terminal A to terminal B and played out in an earphone/loudspeaker. Speech includes detection, transport and generation of DTMF events.

-	Video: The moving image that is, for example, captured by a camera of terminal A, transmitted to terminal B and, for example, rendered on the display of terminal B.

-	Text: The characters typed on a keyboard or drawn on a screen on terminal A and rendered in real time on the display of terminal B. The flow is time-sampled so that no specific action is needed from the user to request transmission.

-	Data: Any other data for real-time interaction, closely related to the multimedia telephony session that may be generated or consumed by either one of terminal A or terminal B, possibly via terminal external connections and/or physical connectors, optionally processed by application-specific logic at one or both terminals, and optionally presented on and controlled by the user interface at one or both terminals.

Suggest a standalone IMS Data Channel application is used to transmit files with large data volumes, operators may implement a variety of charging policies for large data transmission IMS DC services, such as charging by traffic volume or charging by duration. Under this circumstance, it is more appropriate not to only treat this IMS Data Channel application as PS Data Off exemption.

Furthermore, SA5 has passed a new WID “Charging Aspects of IMS Data Channel” (SP-230621), one of the objectives is:

- Potential enhancements to 5GC charging to support volume based charging for IMS data channel, considering QoS handling of IMS data channel.

So, volume-based charging on some of the services over IMS data channel has already been studied in SA5, laterally proved that not all the IMS Data Channel services should be considered 3GPP PS Data Off Exempt Service.

Observation 2：If IMS Data Channel is established standalone without accompanying audio/video/messaging media in an IMS session, some of the applications using IMS Data Channel may not be 3GPP PS Data Off Exempt Service, some of the applications using over IMS Data Channel may not be 3GPP PS Data Off Exempt Service.

- Conclusions

SA1 may provide guideline according the observations in clause2.

- If IMS Data Channel is always established in the context of IMS MMTel session. Considering that MMTel services are PS Data Off exempt services, it is right to treat all the services over IMS Data Channel as 3GPP PS Data Off Exempt Service.

- If IMS Data Channel is established standalone without accompanying audio/video/messaging media in an IMS session, some of the applications using IMS Data Channel may not be 3GPP PS Data Off Exempt Service, some of the applications using IMS Data Channel may be 3GPP PS Data Off Exempt Service.