3GPP TSG SA WG 1 Meeting #104 	S1-233053

Chicago, US, 13 - 17 November 2023	(revision of S1-23XXXX)

Source:	Xiaomi

pCR Title:	Removing Editor’s Note on performance requirements in clause 6.2

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:                     Chen Dong chendong7@xiaomi.com

Abstract: This pCR proposes to remove the Editor’s note on performance requirements.

1. Introduction

Clause 6.2 contains performance requirements with TBD:

Editor’s note: TBD if sensing service categories 5 and 8 are included in Rel-19.

Sensing service categories 5 and 8 are as important as other categories in 5G Wireless sensing.

2. Reason for Change

The Editor’s Note should be removed.

3. Conclusions

With clarification of the importance of the sensing service categories 5 and 8, the Editors Note can be removed.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137 v 1.0.0.

* * * First Change * * * *

6.2	Requirements

The 5G system shall be able to provide sensing results with the performance requirements in Table 6.2-1.

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 | 10 | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 5 | 1 | 1 | 1 | 1 | 1 | 1000 | 1 | 5 | 5 | Outdoor (e.g., detection of human, UAV) requiring higher performance than category 1 |
|  | 3 | 95 | 1 | N/A | 1 | N/A | 1 | 1 x 1 | 100 (NOTE 2), or 1000 (NOTE 3) | 0.1 | 2 | 2 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian, otherwise, N/A | 0.5 | 5 x 5 for factories 0.5 may be needed | 250 | 0.25 | 1 | 5 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV, AGV, vehicle) requiring higher performance than category 3 |
|  | 5 | 95 | short range radar; 0.02, otherwise; 0.1 | 0.5 | 0.03 | N/A | 0.4 | 0.1 x 0.6 | 50 | 0.05 | 1 | 1 | Indoor/outdoor (e.g., detection tracking of human, animal, UAV, AGV, vehicle) requiring higher performance than category 4 |
| Environment monitoring | 6 | 95 | 10 | 0.2 | N/A | N/A | N/A | N/A | 6000 | 60 | 10 | 3 | Nature of environments monitored by sensing. |
| Motion monitoring | 7 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 4). |
|  | 8 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing |
| NOTE 1: The additional information on some of the performance requirements can be found in [2]. NOTE 2: The value 100 ms is sourced from [3] and is valid for sensing at crossroads. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: Additional KPI on human motion rate accuracy is defined for contactless sleep/sports monitoring [2]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *