3GPP TSG-SA WG1 Meeting #108	S1-244393

Orlando, Florida, USA, 18-22 November2024								(revision of S1-244387)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0036 | rev | 2 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New use case: Train driver takes-over an Ad hoc Group Call from another FRMCS device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC, Ericsson, Kontron |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Allow train driver to transfer an Ad hoc Group to a back-up device (e.g. handheld) outside of the cabin, e.g., in case of an accident. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Take-over of a functional alias by an MC User |  |  |  |  |  |  |  |  |
| Take |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature is already existing in GSM-R and for that reason is mandatory for migration period. Moreover, it is an operational need for resilience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.1, 6.22.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 6.22	Voice communication transfer related use cases

### 6.22.1	Introduction

In this chapter the use cases related to the transfer of incoming or ongoing voice communication, as well as the forwarding of incoming voice communication, using Ad hoc Group Calls and also the take-over of an ongoing Ad hoc Group Call from another FRMCS device, are described. The following use cases are defined:

-	Transfer of an incoming voice communication

-	Transfer of an ongoing voice communication

-	Service interworking and service continuation with GSM-R

-	Call forwarding for Ad hoc Group Call

-	Take-over by the train driver of an ongoing Ad hoc Group Call from another FRMCS device

The use cases are applicable for user-to-user and multi-user voice communications.

NOTE 1:	The assumption for all use cases is that Ad hoc Group Calls will be used for all operational railway communications.

Next change

### 6.22.6	Use case: Train driver takes-over an ongoing Ad hoc Group Call from another FRMCS device

#### 6.22.6.1	Description

The use case describes the usage from a train driver of another device (e.g., handheld), as a back-up FRMCS device in case the cabin device dedicated to voice calls and RECs, cannot be used. This can happen either because the cabin device is out of service or any external train equipment (e.g., antenna, ...) is defected or in case of accident (e.g. collisions with obstacles or animals, etc.) where the train driver is sometimes obliged to leave the train cabin. Consequently, the train driver is obliged to take-over ongoing emergency calls, multi-train calls, driver to controller calls, etc., from this alternative FRMCS equipment.

#### 6.22.6.2	Pre-conditions

The train driver is successfully logged into the FRMCS system using this cabin device and can be addressed by the FRMCS system through his/her FRMCS User Identity_A on the cabin device.

Moreover, the train driver is also successfully logged in the FRMCS system using a handheld. He/she can also be addressed by the FRMCS system on this handheld using his/her FRMCS User Identity_B.

To summarise, the train driver has successfully registered his/her FRMCS User Identity_A to the functional identity_A and FRMCS User Identity_B to the functional identity_B and can be addressed using both functional identities, depending on the device.

The functional identity_A of the train driver cannot be shared between multiple FRMCS Users.

#### 6.22.6.3	Service flows

1) The train driver has initiated a REC, using functional identity_A. The REC is in progress. The train is stopped due to a collision with an obstacle. The cabin device is not working properly and the train driver decides to leave the train cabin after having informed the controller responsible for the area.

2) The train driver wants to use the handheld to continue the REC call outside the cabin. As he/she is already logged to the handheld, FRMCS User Identity_B request FRMCS system to take-over the functional identity_A.

3) The FRMCS System checks if there is any conflict with the other functional identities of FRMCS User Identity_B before allowing functional identity_A to be taken over.

4) The FRMCS User Identity_B takes over functional identity_A through the handheld and gets connected to the ongoing REC.

5) The FRMCS system informs FRMCS User Identity_A that functional identity_A has been deactivated for him/her.

6) The train driver performs remote log off from the cabin device.

#### 6.22.6.4	Post-conditions

1) The train driver continues the REC call from the handheld, using the functional identity_A.

2) Functional identity_A has been deregistered for FRMCS User Identity_A of the cabin device.

#### 6.22.6.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.22.6-001] | The FRMCS system shall allow a ‘human’ FRMCS user to login into multiple FRMCS devices, separately, by using different FRMCS User Identities (e.g., FRMCS User Identity_A, such as cabine device and FRMCS User Identity_B, such as handheld). | A | 22.280 | [R-5.10-001] |
| [R-6.22.6-002] | The FRMCS system shall allow the ‘human’ FRMCS user to take-over a functional alias, when logged into multiple FRMCS devices, using separate FRMCS User Identities for each device. | A/T | 22.280 | [R-5.9a-009] Partially covered. Only authorised user is allowed to take-over a functional alias. Update is needed to include other MC users. CR is needed |

End of changes