3GPP TSG-SA WG1 Meeting #108 	S1-244407

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-244218,4295,4234)

Title:	Use case on active WPT for IoT devices

Agenda Item:	8.1.5

Source:	ZTE Corporation, Huawei, Hisilicon

Contact: 		xu.ling@zte.com.cn,  edward.hall@huawei.com,  si.yuan@zte.com.cn, 	dai.bo@zte.com.cn

Abstract: <provide a short description of the content>

This contribution proposes a new use case for TR22.870, which is about 6G active wireless power transfer (A-WPT) for IoT devices.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- N. Ayir, T. Riihonen, M. Allén and M. F. Trujillo Fierro, "Waveforms and End-to-End Efficiency in RF Wireless Power Transfer Using Digital Radio Transmitter," in IEEE Transactions on Microwave Theory and Techniques, vol. 69, no. 3, pp. 1917-1931, March 2021, doi: 10.1109/TMTT.2020.3047654s

- 3GPP TR 22.840, Study on Ambient power-enabled Internet of Things, V19.0.0

* * * Next Change * * * *

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3GPP Wireless power transfer (WPT) service: The 6G network transmit energy through wireless signals based on UE request, and UE obtains energy from the wireless signals sent by the 6Gnetwork.

Active wireless power transfer (A-WPT): 3GPP IoT device or UE actively request energy, then 3GPP network delivery wireless power through wireless signals to response this request.

* * * Next Change * * * *

Followings are all new texts

9	Massive Communication

9.x	Use case on Active WPT for IoT device

9.x.1	Description

Power consumption has always been the key problem for the IoT devices. Considering the rise of the factory management due to labour and device cost for battery change, the deployed base station could act as steady source of energy via electromagnetic radiation of wireless signals to support operation of those devices, i.e. base station provides wireless power transfer service.

One typical example is the instruments/sensors in the chemical factory to monitor the processing procedure. Currently, these instruments/sensors are wired deployed or equipped with a battery. For the wired deployment, it usually involves the labour cost, plastic pipe cost, construction of shallow trench, etc., which leads to huge deployment cost. On the other hand, for the case of battery equipment, periodic or temporary battery replacement (every two/three years) involves high management cost. Under some hazardous areas/core production areas, wired deployment or battery replacement are even impossible.

Ambient energy harvesting is considered to address the issue, e.g., solar-powered, wind-powered and powered by P-WPT (Passive wireless power transfer, UE passively harvests energy based on existing wireless signals in the environment and does not interact with the network.).  But, powered by ambient energy harvesting requires the IoT device located near an energy source, such as heat, wind and base station. In some cases, there may be no energy source in the environment and IoT devices cannot harvest energy (e.g., solar-powered devices cannot work at night). In other cases, IoT devices may not be able to harvest enough energy from the environment. For example, when using P-WPT, IoT devices passively receive wireless signals (which may be a communication signal sent to other users) for charging. The power of passively received signals is often low and may not meet the charging power required by IoT devices, and the communication waveform is designed for data transmission rather than only energy delivery. Hence, the energy harvesting efficiency from the ambient environment cannot be guaranteed. Wireless Power Transfer can be used alongside ambient energy harvesting, or other power source techniques to ensure a resilient power supply to remote devices.

In a word, ambient energy harvesting is dependent on the external environment which cannot provide a steady supply of energy.

In order to deal with the situation described above, more active power delivery for low-power IoT devices is being developed which is using radio frequency wireless power transfer (RF-WPT) technology [2]. The charging station transmit RF energy signals to low-power devices, which receive them using a rectifying antenna and recharge the battery. When it comes to 3GPP, base stations can actively transmit energy signals to the low-power devices to response its request, i.e. A-WPT. The 6G network can provide 3GPP WPT service (e.g., Inform base station/charging station to transmit energy signals) after receiving the charging request from the low-power IoT device. The IoT device can also exchange related charging status information to network to improve charging efficiency (e.g. report received signal strength indicator).

With the provided A-WPT service, factory IoT devices can request and obtain a stable supply of energy and can keep working continuously for a long time, which can reduce factory maintenance costs and save labor expenses. For 3GPP operators, providing A-WPT services can make full use of spectrum resources and improve network utilization. In addition, A-WPT can reduce battery consumption and minimize environmental pollution caused by waste batteries, improve the flexibility of energy use, and thus contribute to sustainable social development and green low-carbon initiatives.

In addition, high intensity electromagnetic radiation may cause harm to human body. Therefore, the base station need to have the ability to sense people and stop sending energy signals if human body blocks them.

Following give an introduction of basic A-WPT service flow for low power IoT device in factory.

9.x.2	Pre-Conditions

Various sensors are widely deployed within industrial buildings/factories to monitor the environment information or machine status, such as temperature/humidity/air/water quality or equipment vibration. These sensors are registered to the network to report real-time information. Instead of using a wired connection, these sensors are equipped with energy storage components, such as capacitance and battery, and these components could be recharged. When the battery level is below the threshold, these sensors could send a request to the network for wireless energy service. According to the instructions of the 6G network, the base stations emit wireless energy signals.

9.x.3	Service Flows

- Wireless IoT devices S1 to Sn are deployed within a warehouse/factory to obtain and report environment information, such as smoke/temperature, etc.

- 6G network broadcast system message and indicates that 3GPP WPT service are available.

- After a period of time continuous working, devices S1 to Sn are almost out of the stored energy and request energy service from the network. The charge request may include identification information, message identifiers, Charge duration, etc.

- Upon the requests, the network (core network, such as the energy management function) would authenticate the UE and indicate the base stations (including charging station, serving base station and/or neighbour base station) to provide energy service to the devices S1 to Sn.

- Upon the received energy signals, the devices S1 to Sn store the energy for future usage.

- When people is detected in the energy delivery area, the base station stops sending energy signals.

- The devices S1 to Sn send charge termination request to 6G network when the stored energy reach certain threshold.

- The 6G network stops 3GPP WPT service, e.g. the network notifies the base stations (or charging stations) to stop sending energy signals.

- IoT devices S1 to Sn remain normal work during wireless charging and continues to monitor environmental data.

Figure 9.x.3-1: Example Flow for Wireless Power Transfer

With regarding to the wireless power time (how long the network emit wireless power signals each time) and frequency (how often the network emits wireless power signals), one of the main factors is the device’s capability. If a device has greater energy storage capacity, it would require longer service time and lower service frequency. On the contrary, if the device has smaller energy storage capacity, it generally would require less service time and higher service frequency. Another factor is the operation flow of the device, which means how often the UE would use the stored energy to do transmission operation or sensing operation. More intensive operation flow would lead to higher wireless power transfer frequency. Hence, the wireless power transfer time would generally span from seconds to hours and wireless power transfer service frequency would generally span from several times per minutes to once per day.

9.x.4	Post-Conditions

Thanks to the active wireless power transfer services provided by the network, the sensor based IoT devices could be recharged without involvement of labour effort. The lifespan of the IoT devices could be greatly increased, and hence the factory management cost due to battery replacement is significantly decreased. The network-based wireless power transfer can provide timely, continuous, accurate, and adequate energy, which is a reliable source for IoT devices.

9.x.5	Existing features partly or fully covering the use case functionality

In TS22.261, Table 7.5.2-1, Performance requirements for highly reliable machine type communication can be reused.

SA1 has studied environment monitoring based on ambient IoT sensors in TR 22.840 [3], especially, use case on ambient IoT sensors in smart homes, descriptions of the scenarios are:

For UEs only supporting any of the following, or a combination of, NB-IoT, GERAN EC-GSM-IoT [18], and  Category M1[13] of E-UTRAN enhanced-MTC, the UE shall interpret the interval value to be between 2 and 240 hours, with a step size of 2 hours between 2 and 80 hours and a step size of 4 hours between 80 and 240 hours.

Monitoring tasks in smart home scenarios can be roughly divided into two types:

- Monitoring of room environment. By deploying specific sensors that enable communication services, people can acquire real-time monitoring data (e.g. temperature, humidity) of their room environment no matter where they are.

- Monitoring of emergency situations. By deploying specific sensors that enable communication services, risk factors such as gas and smoke can be detected in the air and alarmed timely. When the sensor detects that the gas concentration in the home exceeds the threshold, it usually activates a very strong audio signal to alarm people at home. However, in case of people are out of home (e.g. while at work, or on shopping), it cannot reach people on this situation. So, it should be necessary to notify the family members through their phones as well.

In contrast with conventional battery-based sensors, Ambient IoT sensors can obtain and/or store energy from the environment, such as light, heat, wind, and radio waves, which can be converted to useable electrical energy [12]. Different ambient energy harvesting technologies have their own advantages and disadvantages, suitable for use in different environments. Energy harvesting technologies are not in the scope of the study.

The service requirements for the ambient IoT sensors are defined as

The 5G system shall be able to support communication services for Ambient IoT devices.

The 5G system shall support suitable security mechanisms for Ambient IoT devices, including authentication, encryption and data integrity.

The 5G system shall be able to provide the required communication service according to KPI given in table 5.6.6-1.

In TS22.369, the service requirements are defined from the communication aspects. However, one of the most important aspects of the sensor based IoT devices without battery/with limit battery is energy supply. The ambient IoT sensors only consider energy harvesting from the ambient environment. The ambient energy poses the characteristics of random, opportunistic and unsteady. The power supply of the IoT-based sensors cannot be guaranteed. For example, there may be no light at night for indoor scenario, such indoor factory or warehouses. To ensure continuous working of the sensors, proactively wireless power transfer services from the base station should be supported, i.e, the IoT devices could initiate energy service to the network for wireless power transfer. New challenges to the wireless cellular systems are foreseen and need to be addressed.

9.x.6	Potential New Requirements needed to support the use case

[PR 9.x.6 -1] The 6G network shall be able to provide wireless power service for sensor based IoT device which is battery-less or with energy storage capability

[PR 9.x.6 -2]Subject to operator’s policy, the 6G network shall be able to broadcast that 3GPP WPT service is available.

[PR 9.x.6 -3] Subject to operator’s policy, the 6G system shall be able to support 3GPP WPT service control and management, e.g., collect energy service request, activate and/or deactivation 3GPP WPT service.

[PR 9.x.6 -4] Subject to operator’s policy, the 6G network shall be able to support to transmit RF energy signals to provide 3GPP WPT service.

[PR 9.x.6 -5] The 6G IoT devices shall be able to request 3GPP WPT service from 6G network.

[PR 9.x.6 -6] The 6G IoT devices shall be able to receive 3GPP RF energy signals for energy harvesting.

[PR 9.x.6 -7] Subject to operator’s policy, the 6G network shall support to authenticate an/multiple IoT device(s) to request 3GPP WPT service.

[PR 9.x.6 -8] Subject to operator’s policy, the 6G network shall support to authorize an/multiple IoT device(s) for using 3GPP WPT service.

[PR 9.x.6 -9] The 6G network shall be able to detect the human body and temporarily stop transmission of 3GPP RF energy signals when the human body blocks the transmission path.

[PR 9.x.6 -10] Subject to operator’s policy, the 6G network shall be able to provide 3GPP WPT service for 6G IoT devices with performance specified in Table 9.x.6-1.

Table 9.x.6-1: KPIs for 3GPP WPT for IoT device

| Scenario | Charging Duration (note 1) | Charging Power Level (note 2) | Charging Service Availability (note 3) | Charging service Reliability (note 4) | Charging Range | Device density | IoT Devices moving speed |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WPT for IoT devices in factory | TBD | uW~mW level | 99% | TBD | ≤100m indoor | <5000/km2 (note 5) | Stationary |
| NOTE 1: Charging Duration means the amount of time an IoT device takes from the time it initiates a charge request to the time it completes the charge. NOTE 2: Charing Power Level means the power level of the received signals at the RF front end. NOTE 3: Charging Service Availability means the proportion of successfully completed charging service times to the total number of services. NOTE 4: Charging Service Reliability means the proportion of the time for receiving signal power to reach the charging threshold to the charging duration NOTE 5: It is obtained from the survey of several chemical factories, which could be typical values. |  |  |  |  |  |  |  |